# SANITATION SAFETY PLANNING                            IN KURUNEGALA AREA

National Water Supply and Drainage Board

December 2024

Table of Contents

INTRODUCTION	7

Kurunegala Water Reclamation System	7

1	PREPARATION FOR SANITATION SAFETY PLAN (SSP)	11

Lead organization	11

1.2	The SSP Team	11

1.2.1	SSP Team Leader	11

Table 02: The SSP Operation Team	16

Table 03: The SSP Stakeholder Team	20

Stakeholder analysis	24

Table 04: Stakeholder Analysis	24

1.3	Establishment of SSP Priorities	31

Sewered areas	33

Unsewered areas	33

2	SYSTEM DESCRIPTION	35

2.1	System Map	35

2.2	Characterize System Flows	47

Table 2.1 – Characterization of system flow	47

2.3	Identify Exposure Groups	51

2.4	Supporting Information	59

3	HAZARDOUS EVENTS, ASSESSMENT OF EXISTING CONTROL MEASURES AND RISK EXPOSURE	61

Table3.1.2: Semi-quantitative risk assessment matrix	63

3.1	Assessment and Prioritization of Risk Exposure	87

4	DEVELOPMENT AND IMPLEMENTATION OF INCREMENTAL IMPROVEMENT PLAN	90

4.1	Consideration of Options to Control Identified Risks	90

4.2	Development of an Incremental Improvement Plan	95

4.3	Implementation of the Improvement Plan	96

5	MONITORING OF CONTROL MEASURES AND VERIFICATION OF PERFORMANCE	97

5.1	Definition and Implementation of Operational Monitoring	97

6	DEVELOPMENT OF SUPPORTING PROGRAMMES AND REVIEW PLANS	108

Identification and Implementation of Supporting Programmes	108

Programme to enhance the protection during cleaning of Main collecting well. The ssp was target to improve the safety of workers during the cleaning of main well at Kurunegala wastewater treatment plant. There are is no perfect closing system for stop the entering of waste water to the main collecting well during the preventive maintenance work. Therefore, it was decided to install a gate at manhole prior to the main collecting well. Currently, the preparation of suppliers list is being progressed.	108

	Programme to enhance the safety on the treatment Plant	108

At present, there is no perfect security fence on the treatment plant. Hence, it was decided to install a perfect fence on the plant in order to secure the safety of worker and visitors during walking on the treatment plant floor. Currently, procurement of material for the above is being processed.	108

●	Programme to reduce the storm water collecting to the sewer conveyance system.	109

●	Programme to ensure the quality of the treatment process and the effluent by improving the onsite testing facility in the Kurunegala net work area.	109

●	Map the location of the leak point/overflow, on the network map.	109

6.1	Periodic Reviewing and Updating the SSP Outputs	109

Introduction	115

Section 01. System Information	116

Section 02. Chain of Command – Lines of Authority	118

Section 03. Events that Cause Emergencies	120

Section 04. Emergency Notification	121

Media Notification	123

Section 05. Effective Communication	125

Designated public spokesperson	125

Designate a spokesperson and alternates	125

Section 06. Response Actions for Specific Events	126

​

| UDA | Urban Development Authority |
| --- | --- |
| VT | Vacuum Truck |
| VTU | Vacuum Truck Unit |
| WR WWTP WWTS | Water Reclamation Wastewater Treatment Plant Wastewater Treatment System |

Abbreviation

| BOD | Biochemical Oxygen Demand |
| --- | --- |
| CEA | Central Environmental Authority |
| CMR | Compliance Monitoring Report |
| COD | Chemical Oxygen Demand |
| DA | Department of Agriculture |
| FOG | Fat, Oil, and Grease |
| LA | Local Authority |
| MOH | Ministry of Health |
| NWSDB/ NWS&DB | National Water Supply and Drainage Board |
| PHI | Public Health Instructor |
| PPE | Personal Protection Equipment |
| PS | Pump Station |
| RSC | Regional Support Center |
| SSP | Sanitation Safety Plan |
| SSC | Sewer Service Connection |
| SEWAGE TREATMENT PLANT | Sewage Treatment Plant |
| TP | Treatment Plant |
| TSS | Total Suspended Solids |

Definition of Terms

Biosolids – Organic materials resulting from the treatment of sewage and septage sludge; residue generated during the treatment of domestic sewage and septage in a treatment facility

NWS&DB – National Water Supply and Drainage Board

Desludging — The process of cleaning or removing the accumulated domestic sludge or septage.

Dewatering — This is the process of reducing the moisture content of sludge to lessen the volume and odor, e.g. vacuum, filter, centrifuge, belt filter press, filter press, sludge drying beds, and lagoons.

Disinfection — This is the process of destroying pathogenic organisms either by physical (e.g. application of heat) or chemical (e.g. chlorine application) means.

Effluent – A general term denoting any wastewater, partially or completely treated, or in its natural state, flowing out of a manufacturing plant, industrial plant, or treatment plant.

Exposure— Contact of a chemical, physical or biological agent with the outer boundary of an organism (e.g. through inhalation, ingestion, or dermal [skin] contact).

Hazard—A biological, chemical, or physical constituent that can cause harm to human health

Helminth— Helminths are a broad range of organisms that include intestinal parasitic worms: trematodes (flatworms, also commonly known as flukes, e.g. Schistosoma), nematodes (roundworms, e.g. Ascaris, Trichuris and the human hookworms) or cestodes (tapeworms, e.g. Taenia solium, the "pork tapeworm")

Influent – Waste water flowing through the sewer lines/conveyance into a treatment plant

Pathogens – Disease-causing organisms (e.g. bacteria, helminths, protozoa, or viruses).

Risk – The likelihood and consequences that something with a negative impact will occur.

Septage - Also known as Domestic Sludge or the solid particle of domestic sewage, which settles at the bottom of the sedimentation tank and is digested by anaerobic bacteria, purely from domestic sources, exclusive of industrial and hazardous wastes.

Sewage - or also known as Domestic Sewage — wastewater composed of raw liquid and solid waste coming from residential and commercial uses, exclusive of industrial and hazardous waste.

Wastewater – Sewage and faecal sludge

## INTRODUCTION

The water Reclamation Section of  North Western (WRS-NW) has developed a Sanitation Safety Plan (SSP) to prioritize public health during the implementation of its Sewerage Management Program (SMP). This SSP is designed to protect individuals involved in various stages of the sewerage management process, including septic tank desludging, sewage transportation, treatment processes, and waste disposal from Sewerage Treatment Plants. Furthermore, the plan safeguards downstream farmers who utilize treated effluent for irrigation and dried sludge cake as fertilizer, as well as the consumers of agricultural products grown with these materials.

The coverage area includes mainly the Kurunegala Municipal limits. In addition, feacal sludge treatment covers the Kurunegala Municipal area, Kurunegala Polgahawela Mawathagama Narammala Pradeshiya Sabha areas.

### Kurunegala Water Reclamation System

The Water Reclamation Center in Kurunegala (Figure 1) is a combined sewage and septage treatment plant occupying a 2.5-acre site at Ayurvedha Junction, Kurunegala. This facility is designed to process a total flow of 5,000 m3/day. Furthermore, it is capable of treating septage from the Kurunegala municipal area and its surrounding suburbs..

A total of 3800 utilities are already connected to the sewer lines of the existing sewerage system, benefiting more than 40,000 residential and 200,000 floating populations. These sewer lines are connected to the water Reclamation plant through a system comprising 7 lifting stations and approximately 2000 manholes, spanning 136 kilometers of the sewer line. Figure 2 illustrates the sewered areas served by the treatment plant.

## PREPARATION FOR SANITATION SAFETY PLAN (SSP)

  - SSP area and lead organization

    - SSP area

The Water Reclamation RSC(NW) of NWSDB has initiated this SSP process, focusing primarily on the territorial division covered by the Kurunegala municipality. Additionally, as the Water Reclamation Center accepts vacuum trucks from the Kurunegala area and its suburbs, the SSP encompasses all sanitation steps within the sanitation service chain, including toilet, containment-storage/treatment, conveyance, treatment, and disposal. However, the current scope of the SSP excludes the end-use of Sewage Treatment Plant by-products.

### Lead organization

National Water Supply and Drainage Board (NWSDB) is the SSP lead organization.

### The SSP Team

### SSP Team Leader

The Manager of Water reclamation section is the SSP team leader. Manager has many years of experience the sanitation sector and has resources, knowledge and management skills to lead project implementation.

SSP Core Team

The SSP Core Team is composed of WRS (NW) employees from NWSDB Water Reclamation Section. The operation and management of the Sewage Treatment Plant, Management and operations of the Sewerage Network, House connection and future extensions are totally handled by this team.

The SSP Core Team shall undertake the development of the SSP and the initial implementation and monitoring. The SSP Core Team will continue to expand the SSP Team composition by inviting identified primary stakeholders in the sanitation chain

Multi-Stakeholders

Equally important members of the SSP Team are the multi-stakeholders, such as individuals or groups that could affect or be affected by the activities carried out in relation to WRS (NW) SMP, including individuals and groups from local government units, national agencies and users of the Sewage Treatment Plant end-products. Specific details about multi-stakeholders are shown in Table 02.

The multi-stakeholders provide pertinent data and coordination works that are key in policy implementation. Their role and early involvement in sanitation safety planning will be important in the health risk reduction measures listed in the SSP.

4

Table 01: SSP CoreTeam

|  | Role | Job Title | Name | Responsibilities | Contact Details |
| --- | --- | --- | --- | --- | --- |
| 01 | Team Leader, SSP | Manager (WR) | Eng. W.M.A.B. Thilakarathna | - Lead the team - Call regular meetings - Responsible for SSP development, implementation, and sustainability - Initiate revisions/improvements of SSP | anu457@gmail.com 0706606461 |
| 02 | Assistant Team Leader, SSP | Senior Engineer (WR) | Mr. D.M.B.T.B. Dissanayake | - Oversee major SSP activities - Monitor disposal of dried sludge, solid waste, and other residual wastes | gkss.thilina@gmail.com 071 4461807 |
| 03 | Document Specialist/Secretariat | Engineer (WR) | Mr. R.D.R. Wickramarathne | - Liaison with stakeholders - Document SSP development process - Prepare meeting minutes | randika.gkss@gmail.com 071 9861020 |
| 04 | Chemist | Senior Chemist | Mrs. U.S.D. Chamika | - Monitor effluent quality for reuse - Monitor sludge quality - Ensure compliance with government standards | Sewage Treatment Plantlabkuru@gmail.com 070 3164291 |
| 06 | Mechanical/Electrical | Engineer (Mechanical) | Mr. M.A.S.E. Weerakoon | - Provide assistance on Electro – Mechanical management of Water Reclamation Section | weerakoon.gkss@gmail.com 077 3648356 |
| 05 | Sewer network management And On-site desludging personnel | Engineering assistant within records civil special | Mr Premarathna | -Monitor the septage collection from septic tanks of households -Monitor the handling and transport of the collected septage to the SEWAGE TREATMENT PLANT Overall management of Sewerage network and new connections | … 0777754322 |
| 07 | Plant Supervisor | Engineering Assistant (Civil) | Mr. A.S.K. Athukorala | - Supervise the day-to-day operation of the SEWAGE TREATMENT PLANT -Provide needed data on operations (i.e., volume treated, chemicals used, hours of operations, etc.) | saman.gkss@gmail.com 071 4436356 |
| 08 | Electrical Management of Water Reclamation section | Engineering Assistant (Electrical) | Mr. S.B.N.S.C. Ananda | - Provide feedback on the quality of effluent of the SPTP - Report on effluent quality - Update SSP Team on standards/regulations | ananda.gkss@gmail.com 070 3029583 |

### Table 02: The SSP Operation Team

| NAME/JOB | Section | TITLE | Expertice | ROLE IN SSP TEAM | CONTACT INFORMATION |
| --- | --- | --- | --- | --- | --- |
| C.J. Ranasinghe | WR ( Ja Ela/Ekala Region) | Regional Manager | Charted Engineer (Civil) | Team Leader | 777633321 |
| D.M.W.J.B.Dissanayake | WR ( Ja Ela/Ekala Region) | Engineer (Civil) | Engineer (Civil) | Facilitator | 764583668 |
| Eshani Jayathilake | Laboratory (Soysapura WRC) | Chemist (Moratuwa/R athmalana) | Water Quality | Water quality testing | 0771453791 |
| W. Athulathmudali | GCWR | Accountant (GCWR) | Financial | Financial Facilitator | 0718066289 |
| Amali Jayakodi | WR Section | Sociologist | SSP Moderator | Conducting meetings and awareness programmes | 0715220242 |
| A.D.J.R.Abeysekara | Raddoluga ma WRC | OIC (Kurunegala ) | Civil Engineering | Deputy Team Leader. O&M of WRC & coordination. | 777764226 |
| J.P.Koralage | O&M | EA (Civil) | Civil Engineering | O&M of WRC & coordination. | 714898653 |
| G.R.D.M.Iddamalgoda | WR ( Ja Ela/Ekala Region) | EA(Mech.) | Mechanicall Engineering | O&M of mechanical section | 716407880 |
| H.L.P.K.Gunawardana | O&M | EA (Electrical) | Electrical Engineering | O&M of electrical section | 773900557 |
| H.D.M.P ERERA | O&M | EA (Civil) | Civil Engineering | O&M of documentation & procumentation. | 0763570891 |
| H.K.S Randila | WR ( Ja Ela/Ekala Region) | Lab Assistant | Sample Testing | Sample testing and quality contral | 0776799245 |
| T.G.C.M. Perera | O&M | Electrician | Electrical O&M | O&M work (Elactrical) | 0751517412 |
| A.A. Ranil Jayantha | O&M | Pump Operator | Pump Operation | Plant operation | 0769039817 |
| K.D.R. Perera | O&M | Pump Operator (Covering) | Pump Operation | Plant operation | 0770535619 |
| W.M.C. Jayashantha | O&M | Pump Operator (Covering) | Pump Operation | Plant operation | 0754785193 |
| W.H.P.M. Jayasekara | O&M | Labour | O&M work | Assisting for Maintainance Work | 0723233017 |
| M.D.A. Fernando | O&M | Labour | O&M work | Assisting for Maintainance Work | 0761100270 |

| S.T.W. Samarathunga | O&M | Driver | Driving | Driving vehicles | 0718001828 |
| --- | --- | --- | --- | --- | --- |
| A.N.Vipun | O&M | Labour | Documentat ion | Data entry | 0701687834 |
| A.D.S. Fernando | O&M | Labour | O&M work | Assisting for Maintainance Work | 0722056668 |
| J.A.C.S. Jayakodi | O&M | Labour | O&M work | Assisting for Maintainance Work | 0771555803 |
| A.D.M. Dharmathilake | O&M | Labour | O&M work | Assisting for Maintainance Work | 0781046816 |
| M.S. Chandrarathna | O&M | Labour | O&M work | Assisting for Maintainance Work | 0767519027 |

6

### Table 03: The SSP Stakeholder Team

| SANITATION STEP | SSP TEAM MEMBERS AND REPRESENTATION |
| --- | --- |
| Toilet and Containment – (Storage/treatment) | PHI – Kurunegala Municipal Council Technical Assistant – Kurunegala Pradeshiya Sabha Technical Assistant – Polgahawela Pradeshiya Sabha Technical Assistant - Mawathagama Pradeshiya sabha Technical Assistant – Narammala Pradeshiya Sabha |
|  | Environmental officer - Central Environmental Authority |
|  | Environmental officer – North Western Province Environmental Authority |
|  | Kurunegala Divisional Secretory – Kurunegala Divisional Secretariat |
|  | Kurunegala Medical officer of health – Kurunegala Municipal Council |
| Conveyance (Emptying and transport of sewage and/or faecal sludge) | SSP team leader – NWSDB O&M Engineer – NWSDB OIC (WR-NW) – NWSDB EA (Civil) (WR-NW) – |
|  | Fecal Sludge suplier - AB Hydrotech - Dinuth cey food (pvt) Ltd - Yehara Enterprises |

|  | Authority for traffic law enforcement and licenses |
| --- | --- |
| Treatment and disposal | SSP team leader – NWSDB O&M Engineer – NWSDB OIC (WR-NW)) – NWSDB EA (Civil) (WR-NW) – NWSDB EA (M&E) (WR-NW) – NWSDB EA (Electrical) |
|  | Central Environmental Authority (CEA) North Western Province Environmental authority |
| Reuse | Water reclamation center WR-NW) |
|  | Irrigation department |
|  | Department of agriculture |

### Stakeholder analysis

Because of the size and complexity of the sanitation systems in Kurunegala, the SSP core team decided to conduct a stakeholder analysis to ensure that all relevant stakeholders were engaged. After a brainstorming session, a list of stakeholders representing the entire sanitation service chain identified and analysed.

### Table 04: Stakeholder Analysis

| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influence/Power | Partici tion Requir |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Entire sanitation | NWSDB | Direct control and oversight of | SSP will help the mto identify the right sanitation | Too much effort and insufficient resources | High | High | Empo ment |
| chain |  | implementation of | interventions. |  |  |  |  |
|  |  | national guidelines and |  |  |  |  |  |
|  |  | standard sateach step of |  |  |  |  |  |
|  |  | the chain. |  |  |  |  |  |
| Entire sanitation chain | District/ Divisional Secretariat | Influence–coordinate with other government organization to ensure | Providing safe sanitation services is a requirement for healthy communities. | Low knowledge about their role in the sanitation sector. | Low | High | Collab tion |
|  |  | that sanitation systems |  |  |  |  |  |
|  |  | do not pose public |  |  |  |  |  |
|  |  | healthrisks.` |  |  |  |  |  |
| Entires anitation chain | Public health official MOH, PHI | Ensuring the public health and providing awareness about the sanitation. | They have been developing studies on water borne diseases. | The scope of their job role is wider. As a result, they have limited time to involve in sanitation sector. | High | Low | Consu ion |

| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influence/Po wer | Partici tion Requir |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Entire sanitation chain | Secretary of Local authority (PS, UC,MC) | Direct control–Support from the legislative branch of the local | They have the legislative power to address the sanitation safety within their | Lack of experience and interest in sanitation. | High | High | Collab tion |
|  |  | government. | boundaries. |  |  |  |  |
| Entire sanitation chain | Chairman of PS | Allocation of funds from local government. | Commitment to the government to improve their cities in every aspect. | Allocation of lesser funds for sanitation improvements and preferring government funds to be allocated to other sectors. | Low | High | Collab tion |
| Entire sanitation chain | Waste and wastewater service workers | Influence – representatives of formal and informal sanitation | Workers along the chain will be responsible for implementing proposed | Limited representation of workers | High | Medium | Collab tion |
|  |  | workers advocating for | system improvements and |  |  |  |  |
|  |  | fair working conditions | monitoring, and can evaluate |  |  |  |  |
|  |  | and pay. | the practicality and safety for |  |  |  |  |
|  |  |  | workers in the system. |  |  |  |  |

| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influence/Power | Partici tion Requir |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sewered system (flush toilet with sewerage and off-site wastewater treatment) |  |  |  |  |  |  |  |
| Toilet | Residents of Kurunegala | Functioning the waste water system in their premises correctly. |  |  | High | Low | Collaborati |
| Conveyance and treatment and disposal | O&M Team Kurunegala Water reclamation Center | Operate and maintain the wastewater conveyance, treatment and disposal system. | Lead organization in sanitation safety plan implementation. | Rising the cost of spare parts. | High | High | Informatio |
| Conveyance | RDA/PRDA | Approving and providing estimates for road damages due to excavations for leak repires and new connections. Reinstatement of roads. |  | Resistance to approve the road damagers. | High | High | Collaborati |
| Enduse/ disposal | CEA | Enforcing laws and regulations related to treated effluent discharge and sludge disposal | Possible improvements to sludge management process. | Resources and capacities to monitor dried sludge quality are lacking. | High | High | Collaborati |
| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influenc e/ Power | Participati Required |

| Enduse | Department of Agriculture | Develop official guidelines and quality standards for agricultural products,and conduct training and projects with farmers. | Use of treated wastewater and dried sludge for, agricultural purposes. | Resources and staff are lacking. | High | Low | Collaborati |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Enduse | Farmers | Use treated wastewater which is discharged to the Dandugam Oya. | Using dried sludge for cultivations as a fertilizer. | Less of knowladge on sanitation. | High | Low | Consultatio |
| On-site system (flush toilets with septic tanks and effluent infiltration, and off-site faecal sludge disposal) |  |  |  |  |  |  |  |
| Toilet, containment –storage/tre atment | House Owners | In charge of maintaining their own on-site systems and should cover the costs of refurbishment and upgrading. | They would be interested in increasing property values. | They are concerned about the costs of repairing and refurbishing their on-site system. | Low | Low |  |
| Conveyance | Vacuum tanker operators –private and public providers | Emptying cesspools and septic tanks. | They are motivated by legalization and recognition. | Lack of competent workers. Some discharging points are faraway from their collection sites. Increace in the tranpotation cost. | High | High | Consultatio |
| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influenc e/ Power | Participati Required |

| Disposal | CEA | have a surveillance team that monitors illegal waste discharges to the environment. However, they have not given enough focus on vacuum tankers. | SSP supports their overall aim of providing leadership in environmental protection. | This could represent new tasks that have not been included instrategic planning. | High | High | Empowerm nt |
| --- | --- | --- | --- | --- | --- | --- | --- |

Establishment of steering committee

Following the results of the stakeholder analysis, the SSP core team decided to invite the following people to form the SSP steering committee:

- District Secretariat –Kurunegala

- Commissioner Kurunegala Municipal council

- Chief Engineer Kurunegala Municipal Council

- Members	of	Local	Authorities	(MCs/UCs/PSs)	Chairman/Secretary (Kurunegala/Polgahawela/Mawathagama/narammala)

- Head of Central Environment Authoruty Kurunegala

- Head of North Western Province Environmental Authority

- Resident Engineer – Road Development Authority Kurunegala

- Resident Engineer – Provincial Road Development Authority Kurunegala

- Head of Water Reclamation Section of NWSDB

- Head of UDA – Kurunegala

- OIC - Environmental Police Kurunegala

- Medical officer Kurunegala Municipal Council

- Head of Agricultural Department - Kurunegala

- SSP Team Leader in NWSDB

### Establishment of SSP Priorities

The SSP core team hosted a kick-off meeting with members of the SSP steering committee. The main objective of the meeting was to decide which sanitation systems were to be prioritized by SSP and which target systems are likely to pose the greatest health risks.

Excreta flow diagrams (SFDs) are a simple and effective way of visualizing the service types in a city and the fate of different excreta streams. Green arrows represent the proportions of excreta that are “safely managed” along the sanitation chain. Red arrows show where the excreta flows are not safely managed.

Members of the steering committee decided to consider following priority areas in SSP;

- Sewered Areas in Kurunegala WRS

- Unsewered Areas – Serviced by Faecal Sludge Management (Those who are having only onsite sanitation facilities)

### Sewered areas

A total of 3,500 utilities are connected to the existing sewerage system, benefiting over 40,000 residents and 200,000 in the floating population. The sewer network spans 136 kilometers, featuring 7 lifting stations and 2000 manholes, and is connected to the Water Reclamation Plant in Kurunegala. Figure 2 illustrates the sewered areas served by the plant.

Despite the extensive network, some households remain unconnected as they were built after the sewer lines were laid and some are located in pockets which are located l. These households typically rely on septic tanks serviced by third-party vacuum trucks, especially when direct connection to the sewer lines is not feasible due to location. Households without septic tanks discharge sewage into drains or nearby creeks, while business areas with retention tanks receive desludging services directly from the reclamation center.

Overflow issues occasionally occur at household interception chambers or manholes due to blockages. Proper maintenance and operation of this infrastructure are critical for capturing domestic wastewater and directing it to the treatment plant, ensuring effective wastewater management before discharge into water bodies.

### Unsewered areas

In unsewered areas within and beyond the plant's coverage, sewage is managed using septic tanks. Households with septic tanks are regularly serviced by third-party vacuum truck management companies under the water reclamation centre’s oversight. Overflow from septic tanks, similar to sewered households, often ends up in drainage systems or nearby creeks. Households or shops without septic tanks discharge sewage directly into these waterways. Figure 3 shows the septage collection areas served by the water reclamation centre through these third-party companies.

Vacuum truck units (VTUs) are deployed to collect septage from unsewered areas, with a schedule ensuring all customers are served once every five years. Customers are notified in advance to prepare their septic tanks for desludging by loosening or removing the cover, as the water reclamation centre and its personnel do not handle this task. Emergency desludging requests, such as for overflowing tanks, are handled using either the centre’s own VTUs or third-party services, but customers are still responsible for preparing the tank.

During desludging, personnel connect hoses, open the septic tank, and pump out the waste. Once complete, they cover the septic tank, though sealing it remains the customer’s responsibility. VTUs continue servicing until reaching capacity, then transport the collected septage to the treatment plant for proper disposal.

## SYSTEM DESCRIPTION

The Water Reclamation Center (Northwestern) in Kurunegala is responsible for a range of sanitation and sewerage services. These include:

- Sewerage Services:

  - Installation of new sewer connections.

  - Maintenance and repair of the sewer network.

  - Operation and maintenance of wastewater treatment plants.

  - Handling customer complaints related to sewerage services.

  - Planning for the expansion of sewerage coverage.

- Sanitation Services:

  - Provision of regular cleaning services for septic tanks and solid retention tanks.

  - Operation and maintenance of sewage treatment plants.

  - Handling customer complaints related to sanitation services.

  - Fleet maintenance of vehicals.

  - Planning for the expansion of sanitation coverage.

Service Delivery:

- Sewered Areas: Receive a full range of sewerage services, including access to the sewer network, treatment facilities, and new connections.

- Non-Sewered Areas: Rely on septic tank management services, including regular desludging and coordination with property owners for proper maintenance.

### System Map

Sewerage Network

The Kurunegala sewerage system begins at the household level, collecting wastewater generated by approximately 3900 residences. This network spans 136 kilometers within the municipal limits, transporting wastewater to the treatment plant via seven strategically placed sewer lifting stations. The system also accommodates wastewater from 800 commercial establishments, provided their waste is of a domestic nature. The Water Reclamation Center (Northwestern) in Kurunegala verifies this through field inspections of each commercial establishment.

The sewerage network currently covers approximately 7 square kilometers, serving a residential population of 40,000 with a daily influx of 200,000 people. Notably, around 90% of the Kurunegala Municipal limits are now covered by the sewerage network. On-site sanitation units and gulley bowsers are utilized to manage wastewater in the remaining unsewered areas.

Septage Facilities

Currently, the Water Reclamation Center accepts gully bowser loads from within the Kurunegala Municipal limits and surrounding areas. Septage from households and commercial establishments is transported to the center using a combination of private gully bowsers and those operated by the National Water Supply and Drainage Board (NWSDB) and other local authorities.

Pump house

#### Water reclamation center

The North Western Water Reclamation System features a sewage treatment facility that combines preliminary treatment, secondary treatment, and disinfection. The WRC-NW processes wastewater from the sewered areas of the Kurunegala Municipality, ensuring safe disposal in compliance with National Environmental Standards. The treatment plant utilizes physical, chemical, and biological methods to effectively remove pollutants from the incoming wastewater.

#### P1: Sewage from households, and commercial establishments

The system starts with households where wastewater is generated. At present around 2500 households are connected to the sewer network. In addition, wastewater from 1400 commercial establishments is accommodated as long as their wastewater meets the discharge standards to public sewer. This is verified by the WRC-NW through field investigations.

#### T1: Sewer network and pumping station

Wastewater generated by consumers flows through a gravity-driven sewer network within the Kurunegala municipality. This network comprises approximately 129 kilometers of gravity sewers and 7 kilometers of pumping sewers. A total of 2000 manholes are integrated into this system. To transport sewage to the treatment plant, seven lifting (pumping) stations are strategically located. Each pumping station consists of an underground sump, a pump house, and a generator room.

Manual cleaning is performed on coarse screens located at the pump houses. These screens remove floating particles larger than 10 millimeters to prevent pump blockages. The removed screenings are collected in a bucket-screen and then lifted for further treatment at the Water Reclamation Center (WRC). The influent wastewater is collected in an underground sump

The sewage flows through sewers to pumping stations (PS-1) under gravity and Pump stations are located at Wilgoda , Yanthampalawa, Sarwodaya, Welangolla, Gettuwana, Bauddhaloka, Theliyagonna

#### P2: Toilet and containment – storage/treatment with septic tanks and soakage pits

At present Water Reclamation Centre accepts gully bowers from Kurunegala Municipal limits and beyond. The Septage from households and commercial utilities are transported via using private gully bowsers and gully bowsers belongs to NWSDB and other local Authorities.

#### T2: Conveyance of fecal sludge by vacuum trucks

The fecal sludge from household and commercial utilities is transported to WRC through VTU owned by NWSDB or 3rd party (Private or local

authority own) VTUs upon request by the households.

#### P3: Pre-treatment of wastewater

The wastewater collected from the network is subjected to pretreatment. The pre-treatment system consists of a mechanical screening unit and a grit removal facility.

Fine Screen

Two mechanized fine screens are installed at the beginning of the wastewater treatment process. These screens remove particles larger than 3 millimeters. The screenings, along with other solid waste, are collected in the grit separator room. The collected material is then dried on a drying bed before being transported to the Sundara Pola solid waste dumping yard for disposal.

Grit chamber

The grit chamber is designed to remove sand, gravel, and other dense mineral particles from the wastewater. As wastewater flows through the 5.8m long, 1m-wide grit channel, these heavier particles settle to the bottom due to gravity. This process effectively separates the denser grit from the lighter organic matter in the wastewater.

Balancing Tank

The wastewater is then stored in the balancing tank. The balancing tank is used to equalize the varying flow and to achieve homogeneousness in the wastewater quality.

#### P4: Secondary treatment: Biological treatment

Secondary treatment includes biological treatment and clarification of sludge. The biological treatment is carried out according to the activated sludge method. The concept is that the living conditions for the microorganisms normally present as decayors in the wastewater are optimized. The biological degradation that would takeplace in the recipient and consume oxygen in the water is now completed in the treatment plant, thus preserving the environment of the recipient. At the wastewater treatment plant the biological treatment consists of anoxic zone followed by the aeration zone.

Aerobic zone

In aerobic conditions, water contains dissolved oxygen, which is introduced at wastewater treatment plants by blowing air into the water. Microorganisms utilize this oxygen, releasing carbon dioxide and water in the process. They rely on organic matter as an energy source for growth while also requiring nutrients. At the plant, air is distributed into the wastewater through two surface aerators positioned beneath a beam above the aeration basin. As aeration is the most energy-intensive process in the plant, optimizing its efficiency is crucial for reducing energy consumption. The aeration tank has a volume of approximately 3612 m³. Sludge volume index (SV30) shall be between 40%-90%

Anoxic tank.

The main purpose of the anoxic  tank is to achieve denitrification in the system to remove nitrogen   from the system. To achieve this a 712 m3 of anoxic tanks are used in the WRC.

Settling tank (Clarifier)

In the sedimentation basins, the activated sludge flocs are separated from the treated wastewater by gravity forces. The sludge is lifted by siphon action to a longitudinal sludge channel. In the settling tank  sludge is controlled by using four sludge  pumps.

Sludge storage tank

Excess sludge is drawn out from the settling tank and conveyed to the  tank. and the volume of the tank is 296m3.

Sludge Dewatering System

#### T5: Conveyance of partially treated water

The surfaced water in the settling tank separated and sent for chlorination by gravity.

#### P5: Disinfection: Chlorination

Clarified effluent flows through a chlorine contact tank with baffle walls, to achieve the effluent requirements regarding fecal bacteria. The biologically treated effluent is disinfected by dosing Chlorine at the inlet of this tank. The chlorination dosage is controlled manually. Around 65 kg of gas Chlorine cylinders are used and one cylinder could be used for about 18 days. Chlorine contact time is around 15 minutes.

#### T6: Conveyance of effluent up to the urban canal

Treated wastewater is conveyed through a storm water drainage system to discharge to the  outfall under gravity according to the central environmental authority regulations.

P6: Discharge of effluent to stream

The outfall of the facility is located in the bank of an urban canal named as Boo Ela. About 78% of the town area of Kurunegala is drained by the Boo Ela and another stream, the Wan Ela. These streams flow some 6 km via residential. commercial and cultivated areas through the Kurunegala city. The construction of the Kurunegala Water Reclamation Facility is to improve the water quality in the Boo Ela canal.

#### P6’: Reuse of effluent

Some part of the treated effluent is used to water the gardening area of water reclamation premises.

#### T7: Conveyance of sludge

When the sludge concentration in the stabilization tank gets higher than the 90%, the sludge in the stabilization tank is directed to sludge drying beds.

#### P7: Sludge treatment: Sludge drying beds

Sludge is further dried by using sludge drying bed. This is build like a green house with a concrete floor. In the Sludge drying bed sludge is further dried.

#### T8: Collecting and storing dried sludge

When sludge is dried well, small part of it is collected to the wheel borrows manually and dumped to plants as fertilizers within the treatment plant premises. The dried biosolids (up to 85% solids by weight) are then transported to the composting facility in a separate land owned by the water reclamation center.

#### P8: Usage of dried sludge (bio-solids) as a soil conditioner

Some of the dried sludge (bio-solids) is taken by the 3rdparty, whom used dried sludge as soil conditioner in coconut cultivated land or used to produce compost.

#### P9: Grit collection

Deposited grit in the grit channel is collected to a wheelbarrow and transported to Sludge storage facility. washed and screened grit will be used for landfilling and backfilling work in Sundarapola solid waste dumping yard.

#### P10: Grit and dried sludge storage area

Grit, and dried sludge (biosolids) from the solar drying beds are transported to the land behind the solar drying beds (Within the water reclamation center premises).

s.

SSSS1=Receipt of sewage from households, commercial and industrial establishments

SS2= Conveyance of WW to WWTP via sewer network and pumping stations

SFS1=Faecal sludge collected in septic tanks and soak pits

SLF1=Liquid fraction that percolates from the pits

SSWF1=Solid waste fraction that screened out from faecal sludge, and then keep to dry.

SSWF2=Solid waste fraction that screened out while going through the fine screen.

SSF1=Sludge fraction separated from treated wastewater and collected in bin .

SSF2=Sludge fraction collected into dump truck from collection bin and transported to solar drying beds.

SWW1= Secondary treated wastewater.

SWW2= Disinfected wastewater send to the boo ela.

SWW3= Disinfected wastewater used for watering for gardening and flushing purposes.

SGF1=Collected grit fraction transported, from grit container to grit storage area.

SSWF3=Dried screening fraction from solar drying beds, transported to Sundarapola solid waste dumping yard.

SSF3=Sludge fraction (sludge cake) dumped in and spread for treatment in solar drying beds.

SSF4=Sludge treated in solar drying beds transported to sludge storage area using dump trucks

SSFGF=Sludge fraction and grit fraction dumped and handled by spreading in storage area behind solar drying beds

SSF5=Dried Sludge fraction taken out by 3rd party for usage in coconut cultivated lands as a soil conditioner

### Characterize System Flows

Based on the information available, the SSP team used tool 2.1 to characterize the system flows and to collect key quantitative information, and information on the microbiological, physical and chemical hazards.

### Table 2.1 – Characterization of system flow

| Sanitation Step | Description of the System Flow (extra-related flow such as ww or sludge, and other waste streams when relevant to sanitation system) | Key Information of the System Flow (Volume, flow, concentration, etc.) | Expected Variations (Seasonal variations or unusual events, such as accidentally mixed components or climate events) | Type of Potential Hazard (Biological, chemical, or physical) |
| --- | --- | --- | --- | --- |
| P1: Toilet and IC of houses, | SS1=Receipt of sewage from households, commercial | About 3500 m3 per day WW is collected. | The sewage could contain faeces and urine (Black water), bathing | Biological Chemical physical |
| commercial | establishments | BOD could reach 350 | water (Grey water), kitchen WW |  |
| establishments |  | mg/l | through grease traps. Storm water during raining seasons |  |
| T1: Sewer network and pumping | SS2=Transfer (Conveyance) of WW from WW generation | About 3500 m3 per day WW is collected. | Strength of WW is reduced due to dilution specially during rainy season | Biological Physical |
| stations | stage to WWTP via sewer | BOD could reach 200 |  | Chemical |
|  | network and pumping stations | mg/l |  |  |
| P2: Toilet and containment-stora | SFS1=Faecal sludge collected in septic tanks and soak pits | About 30 m3 per day WW is collected. BOD | The sludge could contain anal cleansing materials, menstrual | Biological |
| ge/treatment with | [Faecal sludge-solids and water | could reach 1000 mg/l | hygiene products, sharp objects | Physical |
| septic tanks and | that are collected in |  | and other foreign material. It may | Chemical |
| soak pits | underground tanks |  | also contain chemicals present in |  |
|  |  |  | grey water.Depends on the soil |  |

| T2: Conveyance FS by vacuum trucks | SFS2=Fecal sludge emptied into vacuum trucks and transported to the WWTP | About 75 m3 of fecal sludge is received every day. | expected variations : High ph, visual observation grease and oil contamination | Chemical physical Biologica |
| --- | --- | --- | --- | --- |
| P3’ : Pre-treatment of faecal sludge at carse screen | SSWF1=Solid waste fraction is screened out by the bar screen while pumping out the fecal sludge. The screenings are kept to dry beside the pumping stations. | About 1 kg of solid waste is removed each time | The sludge could contain anal cleansing materials, menstrual hygiene products, sharp objects and other foreign material. Disposal of these materials is a big problem to the WRC. | Biological Chemical Physical |
| P3 and T1: Pre-treatment of wastewater | SSWF2=Solid waste fraction is screened out while going through the bucket screen. The screenings are kept to dry beside the pumping stations. | About 100g of solid waste is removed each time | Reduction of water content in washed screening waste | Biological Chemical |
| P4: Secondary treatment (Biological treatment and clarification) | SSF1=Sludge fraction separated from treated wastewater in the stabilization tanks. | No evidence on quantity | No expected variations. | Physical |
| T4: Conveyance of sludge fraction into solar drying beds. | SSF2=Sludge fraction is sent to the solar drying beds by gravity. | About 10m3 twice a week | During the southeast monsoon, drying of sludge fraction takes more time. | Biological Physical |
| P5: Disinfection | SWW1= Chemicals are added to Surface water extracted from settling tanks after the secondary treatment. | Around 3 kg of Bleaching powder is needed for a day | No expected variations. | Chemical |

| P6: Discharge of effluent (treated wastewater) to stream | SWW2= Disinfected wastewater (Disinfected using Bleaching Powder) pumped into Dandugama Oya and dilute with river water | About 1500 m3 per day WW is pumped. BOD of pumped ww is below 30 mg/l | Heavy rainfall leads to heavy dilution of wide range of constituents | Biological |
| --- | --- | --- | --- | --- |
| P6’: Reuse of effluent | SWW3= Disinfected wastewater used for watering for gardening and flushing purposes | About 0.5 m3 per day WW is used. | With heavy rainfall, there is no need to water the gardens. In dry seasons, more WW is required for watering of garden areas. | Biological |
| T3: Conveyance of grit by dump truck | SGF1=Collected grit fraction transported, from grit container to grit storage area | About 5 kg of grit is removed each time | No expected variations. | Biological |
| T4’: Conveyance of washed screenings | SSWF3=Dried screening fraction are collected and eventually incinerated. | About 1 kg of solid (Dried screenings) waste is collected and handed over to the local authority weekly. | With heavy rainfall, solid waste/incinerated residues end up in open drains/mix with surface or groundwater. | Biological Chemical |
| P7 : Sludge treatment | SSF3=Sludge fraction (sludge cake) pumped in and spread for treatment in solar drying beds. Water evaporates and the rest of the water is drained. | About 20m3 of sludge is dumped and spread each time | No expected variations. equipments | Biological |
| T5: Conveyance of dried sludge by dump truck | SSF4=Sludge fraction treated in solar drying beds transported to sludge storage area using dump trucks | About 850kg of sludge is transported each time | No expected variations. | Biological Physical |
| P8: Grit and dried sludge storage | SSFGF=Sludge fraction and grit fraction dumped and handled by spreading in storage area behind solar drying beds | About 5 kg of grit and 850kg of sludge is dumped and spread each time | With heavy rainfall, fecal matter could end up in open drains/mix with surface or groundwater. | Biological |

| P10: Reuse of biosolids | SSF5=Dried Sludge fraction taken out by 3rd party for usage in coconut cultivated lands as a soil conditioner | About 850kg removed from one drying bed. | With heavy rainfall, fecal matter could contaminate surface or groundwater. | Biological |
| --- | --- | --- | --- | --- |

### Identify Exposure Groups

As a next step, the SSP team identified the exposure groups and used tool 2.2 to identify who they are, how many there are, where they are and how exposure occurs. According to the 2018 WHO Guidelines on sanitation and health (WHO, 2018), the people most likely to be exposed to hazards during hazardous events at different steps of the sanitation service chain are as follows.

| Sanitation Step | Exposure Group | Who are the Exposure Groups? (Description of these people) | How many are there? (Actual numbers, if known; otherwise estimate) | What are they doing? (Circumstances under which they might be exposed to hazards in the system flow) | What are they exposed to? (Which system flows and which types of hazards they have contact with) | How often are they exposed to this? (Exposure frequency: daily, weekly, once a year, etc.) |
| --- | --- | --- | --- | --- | --- | --- |
| P1: Toilet and IC of houses, commercial, and industrial establishments | U1 | Users of flush toilets connected to sewer network through ICs | Around 3900 utilities. | ICs are outside the house. Children play and adults perform different activities in the vicinity. | They could have contact with wastewater during overflows. | Overflow and contamination could happen every 2 years, but is more frequent during heavy rains. |
| P2: Toilet and containment-stora ge/treatment with septic tanks and soak pits | U2 | Users of flush toilets connected to septic tanks and | 20,000 households (Around 60,000 people). About 35% are children | Septic tanks and soak pits are usually outside the house. Children play and adults perform different | They could have contact with wastewater during overflows from ICs or toilets. | Overflow and contamination could happen every 2 years, but is more frequent |
|  | L1 | Soak pits in their properties Families living in areas where septic tank effluent and soak pits infiltrate to the ground water | 5,000 households (Around 15,000 people). About 25% are children | activities in the vicinity of the tank. They usually have shallow dug wells, rather than deep tube wells | Wells might contaminate with ww especially where seasonal high ground water table is presence, and distance to the well from unlined septic tank or soakage pits is small. They are exposed to microbial pathogens (Ex: E. Coli). | during heavy rains. Seasonal, could be both during dry and rainy periods depending on situations |
| T2: Conveyance FS by vacuum trucks | W1 | NWSDB or 3rd party (PS, UC,MC, or private party etc.) vacuum truck operators | NWSDB – 4 operators 2 trucks PS – 4 operators 2 trucks UC – 4 operators 2 trucks MC – 4 operators 2 trucks Private – 4 operators 2 trucks | They open the underground tanks (Manhole covers or cover slabs), insert the hose of vacuum truck and empty the lined or unlined septic tanks and the soak pits. | They are in direct contact with faecal sludge, which contains pathogens. Possible bursts due to bio-gas formation in sealed septic tanks, especially where vent pipes have not been provided. | Every day Rare |
| T1: Sewer network and pumping stations | W2 | WWTP Operators Local Community | Operators/laborer’s - 25 40,000 | Cleaning and carrying out O&M and PPM activities They could be inspect blockages or unintentionally touch wastewater | They are is a possibility of direct contacts contact with faecal sludge, which contains pathogens. They are is a possibility of direct contacts contact with faecal sludge, which contains pathogens | Every day Rare |
| P2’: Pre-treatment of faecal sludge at WRC | W2 | WWTP Operators | Operators/laborer’s - 19 | Cleaning and carrying out O&M and PPM activities | They are is a possibility of direct contacts contact with faecal sludge, which contains pathogens | daily |
| P3: Pre-treatment of wastewater | W2 | WWTP Operators | Operators/laborer’s - 19 | Cleaning and carrying out O&M and PPM activities | They might possible to contact with ww, sludge, screening waste, or grit | Daily |
| P4: Secondary treatment (Biological treatment and clarification) | W2 | WWTP Operators | Cleaning and carrying out O&M and PPM activities | Cleaning of walls and channels | He might possible to contact with ww, sludge | Daily |
| T4: Conveyance of stabilized sludge by gravity | W2 | WWTP Operators | Cleaning and carrying out O&M and PPM activities | Opening and closing the the sludgr valve | He might not possible to contact with stabilized sludge | Weekly |
| P5: Disinfection | W2 | WWTP Operators | Cleaning and carrying out O&M and PPM activities | Usually do not come into contact with treated eflurnt | might not possible to contact with treated effluent | Weekly |
| P6: Discharge of effluent (treated wastewater) to stream | L2 | People who consume fish (Fish caught from downstream of the treated effluent discharge location | 500 people | They eat fish without much care | Pathogens | Rare and Only in Rainy season |
| P6’: Reuse of effluent | W2 | Operators who use the treated effluents (ww) to gardening the WRC premises | WWTP Staff - 35 | Patrolling within the WRC premise, to carry out daily routine works | They can be exposed to aerosol which can contain pathogenic microorganisms | Daily |
| T3: Conveyance of grit by wheel barrow | W2 | WWTP Operators | Operators/laborer’s - 19 | Cleaning and carrying out O&M and PPM activities | They can be exposed to direct contact or to aerosol which can contain pathogenic microorganisms | Daily |
| T4’: Conveyance of stabilized sludge by geavity | W2 | WWTP Operators | Operators/laborer’s - 19 | Observation and operation of valves | They are not direct contact with ww, | two or three days per week |
| P7: De watering and driying of Sludge | W2 | WWTP Operators | Operators/laborer’s - 19 | Cleaning and carrying out O&M and PPM activities | They can be exposed to direct contact or to aerosol which can contain pathogenic microorganisms | Daily |
| T5: Conveyance of dried sludge | W2 | WWTP Operators | Treatmnt plant cleaning labours | Collecting dried sludge by shavels | sludge is not fully treated and can contain pathogenic microorganisms. They can expose to microorganisms. | one a week |
| P8: Grit and dried sludge storage | L3 | Families living in the surrounding area of the KurunegalaWRC premises | 200 households ( | They live surrounding areas of WRC and some houses are close by the surface water canal | Mosquito breeding in grit and sludge storage areas. Pathogens from partially treated sludge, grit, and toxic substances from the incinerated screenings might contaminate surface water canal and ultimately ground water (Probable contamination of shallow dug wells) | Often During rainy season |
| P9: Reuse of bio-solids | F C | Farmers/coconut cultivators Consumers of farm products | 10 No information | Usage of partially treated dried sludge as a bio solid/soil conditioner/or use to produce compost They eat the products without much care (Probably the leaves) | Occasional bad smell Pathogenic microorganisms Sharp objects in dried sludge Pathogens | Specially during the dry season Specially during the dry season |

U Sanitation system users: all people who use a toilet.

L Local community: people who live and/or work nearby (who are not necessarily users of the sanitation system) and may be exposed.

W Sanitation workers: all people – formally employed or informally engaged – responsible for maintaining, cleaning or operating (e.g. emptying) a toilet or equipment (e.g. pumps, vehicles) at any step of the sanitation service chain.

WC Wider community: the wider population (e.g. farmers, communities in lower-lying areas) who are exposed to sanitation end-use products (e.g. through recreation or flooding), use sanitation end-use products, or consume products (e.g. fish, crops) that are produced using sanitation end-use products, intentionally or unintentionally. Sanitation end-use products include compost, faecal sludge and wastewater.

Depending on the sanitation service chain to which SSP applies, it might be necessary to treat the following exposure groups separately because they are exposed to very specific hazardous events during the end-use step (e.g. reuse in agriculture or aquaculture, consumption of products).

F Farmers: people who use sanitation end-use products (e.g. untreated, partially treated or fully treated wastewater, biosolids, faecal sludge).

C Consumers: anyone who consumes or uses products (e.g. crops, fish, compost) that are produced using sanitation product

L44

The letters U, L, W, WC, F and C are used as symbols to identify the exposure groups in maps and tables, facilitating the health risk assessment in the subsequent modules.

The broad exposure groups (U, F, C, etc.) can be refined and defined into subgroups to aid the detailed hazard risk assessment, as shown in tool 2.2. For instance, the exposure group “U: sanitation system users” can be divided into U1: users of pit latrines, U2: users of flush toilets with a septic tank, and U3: users of toilets connected to the sewer system. It is important to estimate the number of individuals in each subgroup, how they come into contact with system flows (e.g. wastewater, excreta) and the frequency of exposure.

Although some exposure groups, such as formal workers, are relatively easy to identify, others will be more difficult – for example, communities accessing nearby groundwater sources, seasonal and informal workers, and people living in informal settlements or immigrant populations. Demographics of the exposure groups, such as gender, age and potential social exclusion, should be noted. Climate change or climate variability may increase or decrease the frequency of exposure.

| Symbol | Description |
| --- | --- |
| U1 | All people who use a toilet in sewer system in Kurunegala |
| U2 | All people who use a toilet having onsite sanitation facilities. |
| U3 | All people who affected by flooding the onsite sanitation facility (septic tank and soackage pit). |
| W1 | All people who formally employed or informally engaged with cleaning or operating (e.g. emptying) a toilet or equipment (e.g. pumps, vehicles) at unsewered onsite toilets. |
| W2 | All people who employed in NWSDB and engaged in operation and maintainance work or equipment (e.g. pumps, vehicles) at any step of the sanitation service chain. |
| L1 | All the Local community who affected by contaminated groundwater due to infiltration from septic tanks and soak pits into shallow groundwater |
| L2 | All the Local community who affected by flooding the surrounding septic tanks and sockage pits. |
| L3 | All the Local community who affected by contaminated surface water or shallow ground water due to discharge of faecal sludge without treatment. |
| L4 | All the Local community who affected by ingesting faecal sludge due to spillage during transport. |
| L5 | All the local community who use the road where manhole over flowing occure. |
| L6 | All the local community who affected Contaminated the ground water withwaste water due to the leaks in the pipes of collecting network |
| L7 | All the Local community who use the water in the Dandugam Oya and lives down of the place where treated waste water and storm water are discharged. |

### Supporting Information

    - Regulatory Requirements Water quality standards

The National Environmental (Protection and Quality) regulations, No. 1 of 2008 published in the Gazette No. 1534/18 of February 1, 2022 gives the standards for discharge of wastewater or effluent into inland surface waters.

Table 2.3 – Supporting Information

| Information Sources | Summary of Key Observations |
| --- | --- |
| Standards and regulations |  |
| Central environmental authority (CEA) effluent dischange standards 2022 | BOD, COD, TSS, pH, TDS, E. coli limit, Faecal Coliform (FC), Helminth eggs, |
| Information Related to system Management and Performance |  |
| Demographics and land-use patterns | Population near Ekala industrial area, Population near low-lying flood-prone areas where sanitation conditions are poor, Population where there are only basic sanitaion available with onsite sanitation systems, |
| Changes relating to weather or other seasonal conditions | Demand for pit emptying also increases during heavy rainfall events, but flooding makes access to some areas difficult. Sewers overflow during severe rain events as a result of high flow and blockages caused by solid waste. |

## HAZARDOUS EVENTS, ASSESSMENT OF EXISTING CONTROL MEASURES AND RISK EXPOSURE

Specifying each step in the system is essential in determining potential hazards and hazardous exposure events for the exposure groups. This leads to the assessment of the risks and the extent at which these are controlled. The result of this step in the SSP is to ensure that subsequent efforts and investments on the system monitoring and improvements respond to actual rather than perceived health risks.

3.1 Identification of Hazard and Hazardous Exposure Events

The hazards and hazardous exposure events are identified for the different processes in the system. Table 3.1 shows these identified hazards and hazardous exposure events. This helps in identifying who may be placed at risk and how the exposure risk occurs. Also included in Table 12 are the existing controls, validation of control, exposure groups, likelihood, severity, risk score and risk rating.

3.2 Identification of Exposure Groups and Exposure Routes

Identified hazard and hazardous exposure events must be related to its corresponding exposure groups and the routes by which these groups may be affected. Below is the list of exposure groups and its corresponding code.

| W | Sewer Service Connection Personnel | W1 |
| --- | --- | --- |
|  | Sewer Network Maintenance Personnel | W2 |
|  | Treatment plant operator/ sample collector | W3 |
|  | TP Maintenance and cleaning Personnel | W4 |
|  | VT operators -Drivers and helpers | W5 |
|  | Staff of Water reclamation center (monitors/inspectors the operation) | W6 |
|  | Fleet Maintenance Personnel | W7 |
| L | Live adjacent local community exposed during IC installation or pipe laying | L1 |
|  | Customers/households | L2 |
|  | Local community of live adjacent to Treatment plant | L3 |
|  | Local community living downstream from the TP | L4 |
|  | Local community exposed to waste/sludge during transportation | L6 |
|  | Plant visitors | L7 |
| F | Farmer who uses sludge as fertilizer(gardening) | F |
| C | Consume products irrigated with effluent | C1 |
|  | Consume crops grown with sludge as fertilizer | C2 |

Table 3.1.1.  Definition for Likelihood and Severity for Risk Ranking

| Descriptor | Description |  |
| --- | --- | --- |
| Likelihood (L) |  |  |
| 1 | Very Unlikely | Has .t happened in the past and it is highly improbable it will happen in the next 12 months (or a.ther reasonable period). |
| 2 | Unlikely | Has .t happened in the past but may occur in exceptional circumstances in the next 12 months (or a.ther reasonable period). |
| 3 | Possible | May have happened in the past and/or may occur under regular circumstances in the next 12 months (or a.ther reasonable period). |
| 4 | Likely | Has been observed in the past and/or is likely to occur in the next 12 months (or a.ther reasonable period). 5 Almost Certain Has often been observed in the past and/or will almost certainly occur in most circumstances in the next 12 months (or a.ther reasonable period). |
| 5 | Almost Certain | Has been observed in the past and/or is likely to occur in the next 12 months (or a.ther reasonable period). 5 Almost Certain Has often been observed in the past and/or will almost certainly occur in most circumstances in the next 12 months (or a.ther reasonable period). |
| Severity (S) |  |  |
| 1 | Insignificant | Hazard or hazardous event resulting in . or negligible health effects compared to background levels |
| 2 | Minor | Hazard or hazardous event potentially resulting in minor health effects (e.g. temporary symptoms like irritation, nausea, headache). |
| 4 | Moderate | Hazard or hazardous event potentially resulting in a self-limiting health effects or mi.r illness (e.g. acute diarrhea, vomiting, upper respiratory tract infection, mi.r trauma). |
| 8 | Major | Hazard or hazardous event potentially resulting in illness or injury (e.g. malaria, schistosomiasis, food-borne trematodiases, chronic diarrhea, chronic respiratory problems, neurological disorders, bone fracture); and/or may lead to legal complaints and concern; and/or major regulatory. n-compliance. |
| 16 | Catastrophic | Hazard or hazardous event potentially resulting in serious illness or injury, or even loss of life (e.g. severe poisoning, loss of extremities, severe burns, drowning); and/or will lead to major investigation by regulator with prosecution likely |

### Table3.1.2: Semi-quantitative risk assessment matrix

|  |  |  | SEVERITY (S) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Insignificant | Minor | Moderate | Major | Catastrophic |
|  |  |  | 1 | 2 | 4 | 8 | 16 |
| LIKELIHOOD (L) | Very Unlikely | 1 | 1 | 2 | 4 | 8 | 16 |
|  | Unlikely | 2 | 2 | 4 | 8 | 16 | 32 |
|  | Possible | 3 | 3 | 6 | 12 | 24 | 48 |
|  | Likely | 4 | 4 | 8 | 16 | 32 | 64 |
|  | Almost certain | 5 | 5 | 10 | 20 | 40 | 80 |
| Risk score R = L × S |  |  | <6 | 7-12 |  | 13-32 | >32 |
| Risk level |  |  | Low risk | Medium Risk |  | High Risk | Very High Risk |

3.3 Identification and Assessment of Existing Control Measures

For each of the hazard exposure event identified in Table 3.1, Septage and Sewage treatment identified existing control measures to mitigate the risk of such hazardous exposure events. The said control measures were then assessed on their effectivity in reducing the risk of the exposure event.

The risks were assessed based on its likelihood and severity as shown on Table 3.2. The computed level of risk and likelihood equates to the ranking of risks as shown in Table 3.3 wherein the risk level was identified. Furthermore, the risk category is described in Table 3.4 to highlight the actions to be taken upon assessing the risk

S- Sanitation Step

HE – Hazardous Event

H- Hazard

E- Exposure Group

N- Number of People at risk

D- Description

V- Validation

SC – Score

D – Drought

R – More intense precipitation

| COMPONE NT | HAZARD IDENTIFICATION |  |  |  | EXISTING CONTROLS |  | RISK ASSESSMENT |  |  |  |  |  | COMMENTS JUSTIFYING RISK ASSESSMENT (Under current conditions, climate change scenarios, or effectiveness of the control) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Under current conditions L=Likelihood; S=Severity; R=Risk (H=High; M=Medium; VH=Very high) |  |  |  | Under the most likely climate change scenarios + means increased risk - means decreased risk = means the same risk |  |  |
| S | HE | H | E | N | D | V | L | S | SE | R | D | R |  |
| P1: Toilet and IC of houses, commercial establishments. | Ingestion after contact with wastewater in overflowing toilets or Ics | All microbial pathogens | U1 | 1000 | Preventive maintenance in the sewage collecting network | Visual observation by field visit | 3 | 4 | 12 | MR | - | + | This worsens during heavy rains, possibility of ingestion is high. |

| T1: Conveyance of Sewer through the network and pumping stations | Ingestion after contact with excreta during the repair work of blocking pipe lines and damaged pipe lines. | All microbial pathogen s | W1 | 10 | Wearing personal protective equipment during the work | Visual observation/ | 5 | 4 | 20 | HR | = | + | With the rain blockage will increase. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1: Conveyance of Sewer through the network and pumping stations | Wastewater, over flow from the manholes are directed to the Storm water system of the housing scheme and contaminate the Boo ela. | All microbial pathogen s | L5 | Unkn own | Proper maintenance of sewage network system and proper storm water management prevent the overflow of waste water. | Visual observation | 5 | 2 | 10 | MR | - | + | This worsens during heavy rains, due to increase in overflow. |
| T1: Conveyance of Sewer through the network and pumping stations | Contaminate the ground water with wastewater due to the leaks in the pipes of collecting network | All microbial pathogen s | L6 | No clear idea | Repairing leaks in the pipe lines in the collecting network | Visual Observations/ Surveys | 4 | 2 | 8 | MR | = | + | In rainy season ground water table comes up. Then contamination is high. |
| T1: Cleaning of Coarse Screen | Ingestion after contact with excreta during the cleaning | All microbial pathogen s | W2 | 6 | Following SOP and Wearing personal protective equipment | Monitoring at the field | 5 | 4 | 20 | MR | = | = | no effect from climate change. |

|  | work in lifting well. |  |  |  | during the work |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1: Cleaning of Pump Well | Ingestion after contact with wastewater and Injury to the body, possible asphyxiation, caused by falling into tanks. Suffocation due to lack of oxygen | All microbial pathogens | W2 | 2 | Following SOP and Wearing personal protective equipment | Improvement is required to seel the inflow. | 2 | 16 | 32 | VH R | = | + | During heavy rains, likelihood is increased. |
| T1: Transporting Screenings from pump house to WWTP. | Ingestion after contact with wastewater during the cleaning | All microbial pathoges | W2 | 2 | Wearing personal protective equipment during the work | Visual observation | 5 | 2 | 10 | MR |  |  |  |
| T1: Transporting Screenings from pump house to WWTP. | Ingestion after contact with wastewater and screenings | All Microbial pathogens | L | 40000 | Public aware | Visual Inspection | 1 | 4 | 4 | Minor |  |  |  |
| P3: Cleaning of Grit Channel | Ingestion after contact with wastewater during the cleaning | All microbial pathogen s | W2 | 6 | Wearing personal protective equipment during the work | Visual observation | 5 | 2 | 10 | MR | = | = | no effect from climate change. |
| P4: Cleaning work in treatment plant. | Injury to the body, possible asphyxiation, caused by falling into tanks | All microbial pathogen s | W2 | 3 | Wearing personal protective equipments during the work | Visual observation | 2 | 8 | 16 | HR | = | + | with rain walls will be more slippery. |

| P4: Secondary Treatment Process in the Aeration. | Wastewater is sprinkling and they will be ingested to the body. | All microbial pathogens | W2 | 5 | Wearing personal protective equipment during the work | Visual observation | 5 | 2 | 10 | MR | = | = | no effect from climate change. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P4: Secondary Treatment Process in the settling tank. | Due to breakdown in sludge scraper treatment process will be disturbed and Water of Boo ela will be contaminated with partially treated wastewater. | All microbial pathogens | L7 | Unkn own | Preventive maintenance and following SOP in the system | Visual observation normally once a six month,. | 3 | 4 | 12 | MR | = | = | no effect from climate change. |
| P2: Toilet and containment-stor age/treatment with septic tanks, soak pits. | Ingestion after contact with excreta (faeces/urine) due to blocking/ overflowing | All microbial pathogens | U2 | 500 | Properly functioning flush toilets with water supply | Visual observation /Survey | 3 | 4 | 12 | MR | - | + | Periodicaly empting the pits |
| P2: Disposal of liquid fraction by infiltration | Ingestion of contaminated groundwater due to | Faecal pathogens | L1 | 1000 0 | Safe distance from wells would have to be considered. | Technical Officers In Pradeshiya sabha | 5 | 4 | 20 | HR | + | + | Likelihood of ground water use is expected to increase during |

| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with raw sewage during vacuum tanker operation | All microbial pathogen s | W1 | 50 | Wearing gloves during work | NA | 5 | 4 | 20 | HR | = | + | Wearing gloves, and handwashing with soap or detergent after emptying is not widely practised. In flooding conditions, the likelihood will increase. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with faecal sludge caused by spillage during transport. | All microbial pathogen s | L4 | 6 | Procedures to deal spillage | Observations/ Interviews | 3 | 2 | 6 | MR | = | = | No effect from climate change. |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with faecal sludge while entering or falling into soak pits or septic tanks | All microbial pathogen s | W1 | 20 | Nil | NA | 1 | 8 | 8 | MR | = | + | Flooding can cause collapse of unline septic tanks, and soakage pits. And stability of the tank can be affected by flooding. |
| T2: Conveyance of FS by vacuum trucks | Injury to the body, possible asphyxiation, caused by entering or falling into soak pits or septic tanks | Injury to the body, including asphyxiat ion | W1 | 20 | Nil | NA | 1 | 8 | 8 | MR | = | = | Flooding can cause collapse of unline septic tanks, and soakage pits. And stability of the tank can be affected by flooding. |

| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with raw sewage during vacuum tanker operation | All microbial pathogen s | W1 | 50 | Wearing gloves during work | NA | 5 | 4 | 20 | HR | = | + | Wearing gloves, and handwashing with soap or detergent after emptying is not widely practised. In flooding conditions, the likelihood will increase. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with faecal sludge caused by spillage during transport. | All microbial pathogen s | L4 | 6 | Procedures to deal spillage | Observations/ Interviews | 3 | 2 | 6 | MR | = | = | No effect from climate change. |
| Pipe laying and IC installation | Potential health risk due to exposure to Wastewater during existing sewerage line handling | All microbial pathogen | W1 | 20 | Wearing personal protective equipment during the work | Observations/ Interviews | 8 | 2 | 16 | MR | = | + | Flooding can cause collapse of trenches and make it difficult for the construction work |
| De Clogging of IC | Potential health risk due to exposure to Wastewater during existing sewerage line handling | All microbial pathogen | W1 | 20 | Wearing personal protective equipment during the work | Observations/ Interviews | 8 | 2 | 16 | MR | = | + | Flooding can result to blocking of network |

### Assessment and Prioritization of Risk Exposure

The risks were assessed based on its likelihood and severity. The computed level of risk and likelihood equates to the ranking of risks. Where in the risk level was identified. Furthermore, the risk category which require additional control or improvements were listed and described here to highlight the actions to be taken to reduce the risks.

Table 3.2 Prioritized Hazardous events with very high risk

| Sanitation step | Hazardous event | Exposu re group | Numb er of peopl e at risk | Risk | Projection of changers in risk with climate change scenarios | Priority Given |
| --- | --- | --- | --- | --- | --- | --- |
| T1: Cleaning of collecting well | Ingestion after contact with wastewater and Injury to the body, possible asphyxiation, caused by falling into tanks. Suffocation due to lack of oxygen | W1 | 10 | VHR | Increases in heavy rains | Very High |

Table 3.3 Prioritized Hazardous events with high risk

| Sanitation step | Hazardous event | Exposure group | Number of people at risk | Risk | Projection of changers in risk with climate change scenarios | Priority Given |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P4: Cleaning of treatment plant (Near the chlorination Area) | Injury to the body, possible asphyxiation, caused by entering or falling into tanks in the treatment plant. | W1 | 2 | VHR | Increases in heavy rains | Very High |  |
| P4: Cleaning work in stabilization tank etc. | Injury to the body, possible asphyxiation, caused by entering or falling into tanks | W1 | 3 | HR | No effect from climate change. | High |  |
| T1: Conveyance of Sewer through the network and pumping stations. | Ingestion after contact with excreta during the repair work of blocking network and pipe lines and damaged pipe lines | U1 | 10 | HR | No effect from climate change. | High |  |
| T1: Conveyance of Sewer through the network and pumping stations. | Ingestion after contact with Contaminated ground water with waste water due to the leaks in the pipes of collecting network. | L6 | No clear idea | HR | Increases in heavy rains | Medium |  |
| T2: Conveyance of FS by vacuum trucks. | Ingestion after contact with raw sewage during vacuum tanker operation | W1 | 50 | HR | Increases in heavy rains | Medium |  |
| P22: Disposal of liquid fraction by infiltration | Ingestion of contaminated groundwater due to infiltration from septic tanks and soak pits into shallow groundwater. | U1 | 10000 | HR | Increases in heavy rains | Low |  |
| T2: Conveyance of FS by vacuum trucks. | Ingestion after contact with fecal sludge caused by spillage during emptying and transport | U2 | 6 | HR | Increases in heavy rains | Low |  |

| T1: Conveyance of Sewer through the network and pumping stations | Ingestion after contact with excreta during the repaire work of blocking pipe lines and damaged pipe lines. | All microbial pathogens | W1 | 10 | Wearing personal protective equipments during the work | Visual observation/ further improvements are required | 5 | 4 | 20 | HR | = | + | With the rain blockage will increace. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1: Conveyance of Sewer through the network and pumping stations | Waste water, over flow from the manhalls are directed to the Storm water system of the housing scheme and contaminate the Dandugam Oya. | All microbial pathogens | L5 | Unkn own | Proper maitainance of serage network system and proper storm water management prevent the overflow of waste water. | Visual observation | 5 | 2 | 10 | MR | - | + | This worsens during heavy rains, due to increase in overflow. |
| T1: Conveyance of Sewer through the network and pumping stations | Contaminate the ground water withwaste water due to the leaks in the pipes of collecting network | All microbial pathogens | L6 | No clear idea | Repairing leaks in the pipe lines in the collecting network | Visual Observations/Su rveys | 4 | 2 | 8 | MR | = | + | In rainy season ground water table comes up. Then contaimination is high. |
| T1: Cleaning of Primary lifting | Ingestion after contact with excreta during the cleaning | All microbial pathogens | W2 | 6 | Following SOP and Wearing personal protective equipments | Monitoring at the feild | 5 | 2 | 10 | MR | = | = | no effect from climate change. |
| T1: Cleaning of collecting well | If the sand bags are loosen, the people who working in the collecting tank may submerge in the waste water during the cleaning work in the collecting well. | All microbial pathogens | W2 | 2 | seal the inlet with sand bags. | Improvement is required to seel the inflow. | 3 | 16 | 48 | VH R | = | + | During heavy rains, likelihood is increased. |
| T1: Cleaning of Screening basket in collecting well. | Ingestion after contact with wastewater during the cleaning | All microbial pathogens | W2 | 2 | Wearing personal protective equipments during the work | Visual observation | 5 | 2 | 10 | MR |  |  |  |
| P3: Cleaning of Grit Channel | Ingestion after contact with wastewater during the cleaning | All microbial pathogen s | W2 | 6 | Wearing personal protective equipments during the work | PPE are supplied by NWSDB by annualy and as per the reqirement | 5 | 2 | 10 | MR | = | = | no effect from climate change. |
| P4: Cleaning work in treatment plant. | Injury to the body, possible asphyxiation, caused by falling into tanks | All microbial pathogen s | W2 | 3 | Wearing personal protective equipments during the work | PPE are supplied by NWSDB by annualy and as per the reqirement. / further improvements are required | 2 | 8 | 16 | HR | = | + | with rain walls will be more slipery. |
| P4: Secondary Treatment Process in the Aeration. | Waste water is sprinkling and they will be ingested to the body. | All microbial pathogen s | W2 | 5 | Wearing personal protective equipments during the work | Visual observation | 5 | 2 | 10 | MR | = | = | no effect from climate change. |
| P4: Secondary Treatment Process in the settling tank. | Due to breakdown in sludge scraper treatment process will be disturbed and Water of Dandugam Oya is contaminated with partialy treated waste water. | All microbial pathogens | L7 | Unkn own | Preventive maintainance and keeping standby gear Box and other neccesary equipments. | Visual observation nomally once a six month, weels should be rplaced. | 3 | 4 | 12 | MR | = | = | no effect from climate change. |
| P2: Toilet and containment-stor age/treatment with septic tanks, soak pits. | Ingestion after contact with excreta (faeces/urine) due to blocking/ overflowing | All microbial pathogens | U2 | 500 | Properly functioning flush toilets with water supply | Visual observation /Survey | 3 | 4 | 12 | MR | - | + | Periodicaly empting the pits |
| P2: Disposal of liquid fraction by infiltration | Ingestion of contaminated groundwater due to infiltration from septic tanks and soak pits into shallow groundwater | Faecal pathogens | L1 | 1000 0 | Safe distance from wells would have to be considered. | Technical Officers In Pradeshiya sabha | 5 | 4 | 20 | HR | + | + | Likelihood of ground water use is expected to increase during drought periods. Severity will increase with more and prolonged flooding due to climate change |
| P2: Toilet and containment-stor age/treatment with septic tanks and soak pits | Ingestion after contact with wastewater during the flooding. | All microbial pathogens | U3 | 500 | N/A | Field visits and interviews | 3 | 4 | 12 | MR | - | + | Due to shallow ground water table, poor drainage of storm water and rain intencity likelihood of flooding have been increaced. |
| P2: Toilet and containment-stor age/treatment with septic tanks and soak pits | Ingestion after contact with wastewater during the flooding. | All microbial pathogens | L2 | 100 |  |  | 3 | 2 | 6 | LR | - | + | Due to shallow ground water table, poor drainage of storm water and rain intencity likelihood of flooding have been increaced. |
| P2: Disposal of faecal sludge without treatment or Deep Row Entrenchment | Ingestion after contact with faecal sludge discharged without treatment, contaminated surface water or shallow ground water | All microbial pathogens | L3 | 1500 | Nil | NA | 3 | 4 | 12 | MR | = | + | Generally higher risk during heavy rains. |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with raw sewage during vacuum tanker operation | All microbial pathogens | W1 | 50 | Wearing gloves during work | NA | 5 | 4 | 20 | HR | = | + | Wearing gloves, and handwashing with soap or detergent after emptying is not widely practised. In flooding conditions, the likelihood will increase. |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with faecal sludge caused by spillage during transport. | All microbial pathogen s | L4 | 6 | Procedures to deal spillage | Observations/ Interviews | 3 | 2 | 6 | MR | = | = | No effect from climate change. |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with faecal sludge while entering or falling into soak pits or septic tanks | All microbial pathogen s | W1 | 20 | Nil | NA | 1 | 8 | 8 | MR | = | + | Flooding can cause collapse of unline septic tanks, and soakage pits. And stability of the tank can be affected by flooding. |
| T2: Conveyance of FS by vacuum trucks | Injury to the body, possible asphyxiation, caused by entering or falling into soak pits or septic tanks | Injury to the body, including asphyxiat ion | W1 | 20 | Nil | NA | 1 | 8 | 8 | MR | = | = | Flooding can cause collapse of unline septic tanks, and soakage pits. And stability of the tank can be affected by flooding. |

### 3.4 Assessment and Prioritization of Risk Exposure

The risks were assessed based on its likelihood and severity. The computed level of risk and likelihood equates to the ranking of risks. Where in the risk level was identified. Furthermore, the risk category which require additional control or improvements were listed and described here to highlight the actions to be taken to reduce the risks.

Table 3.2 Prioritized Hazardous events with very high risk

| Sanitation step | Hazardous event | Exposure group | Numb er of people at risk | Risk | Projection of changers in risk with climate change scenarios | Priority Given |
| --- | --- | --- | --- | --- | --- | --- |
| T1: Cleaning of collecting well | If the sand bags are loosen, waste water will enter in to the collecting well and the people who working in the collecting well, may submerge in the waste water during the cleaning work in the collecting well. | W1 | 2 | VHR | Increases in heavy rains | Very High |

Table 3.3 Prioritized Hazardous events with high risk

| Sanitation step | Hazardous event | Exposure group | Number of people at risk | Risk | Projection of changers in risk with climate change scenarios | Priority Given |
| --- | --- | --- | --- | --- | --- | --- |
| P4: Cleaning of treatment plant (Near the chlorination Area) | Injury to the body, possible asphyxiation, caused by entering or falling into tanks in the treatment plant. | W1 | 2 | VHR | Increases in heavy rains | Very High |
| P4: Cleaning work in stabilization tank etc. | Injury to the body, possible asphyxiation, caused by entering or falling into tanks | W1 | 3 | HR | No effect from climate change. | High |
| T1: Conveyance of Sewer through the network and pumping stations. | Ingestion after contact with excreta during the repair work of blocking pipe lines and damaged pipe lines. | U1 | 10 | HR | No effect from climate change. | High |
| T1: Conveyance of Sewer through the network and pumping stations. | Ingestion after contact with Contaminated ground water with waste water due to the leaks in the pipes of collecting network. | L6 | No clear idea | HR | Increases in heavy rains | Medium |
| T2: Conveyance of FS by vacuum trucks. | Ingestion after contact with raw sewage during vacuum tanker operation | W1 | 50 | HR | Increases in heavy rains | Medium |
| P22: Disposal of liquid fraction by infiltration | Ingestion of contaminated groundwater due to infiltration from septic tanks and soak pits into shallow groundwater. | U1 | 10000 | HR | Increases in heavy rains | Low |
| T2: Conveyance of FS by vacuum trucks. | Ingestion after contact with fecal sludge caused by spillage during emptying and transport | U2 | 6 | HR | Increases in heavy rains | Low |

## DEVELOPMENT AND IMPLEMENTATION OF INCREMENTAL IMPROVEMENT PLAN

### 4.1 Consideration of Options to Control Identified Risks

In this step, variety of ways to control risks, including technology upgrades, changes in management and operation, behavior change measures, and policy and regulatory measures, covering all steps of the sanitation chain were considered.

Table 4.1 Improvement Option at T1: Cleaning of collecting well.

| Step of the sanitation service chain: T1: Cleaning of collecting well. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: If the sand bags are loosen, waste water will enter in to the collecting well and the people who working in the collecting well, may submerge in the waste water during the cleaning work in the collecting well. |  |  |  |  |  |
| Exposure group: About 2 people engaged in Cleaning Work in Collecting wells. |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effectiveness | Level of resources | Effectiveness under climate | Comments/discussion | Priority |
|  |  |  | scenarios |  | for |
|  |  |  |  |  | improvement |
| Use blowers to ventilate the pump wells and deep manholes when carrying out cleaning activities | High | High | Effective | Plan to purchace the | Immediate |
|  |  |  |  | . |  |
| Carry out cleaning work | Medium | Low | Medium | Untill receive permanent | Immediate |
| with full harness safety kit |  |  |  | solution, even simple |  |
| to lift up the persons at |  |  |  | solution should be |  |
| emergency. Keep wokers |  |  |  | applied. |  |
| stand by for any |  |  |  |  |  |
| emergency situation. |  |  |  |  |  |

Table 4.2 Improvement Option at P4: Cleaning work in Chlorination Area, Stabilization area etc.

| Step of the sanitation service chain: P4: Cleaning work in Aeration, stabilization etc. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: Injury to the body, possible asphyxiation, caused by entering or falling into tanks |  |  |  |  |  |
| Exposure group: About 5 workers during the cleaning of treatment plant structures such as stabilization tank. |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effectiveness | Level of resources | Effectiveness ss under climate change scenarios | Comments/discussion | Priority for improvement plan |
| Wearing personal Protective equipment during the operation. | High | Low | Effective | Workers should be encourage to were the PPE | Immediate |
| Follow the standard Operational procedures in cleaning work. | High | Medium | Effective | SOPs should be Translated in to Sinhala language. | Medium Term |
| Improve the safety fence Where can be exposed to the risks. | High | High | Effective | In most of areas safety fence are not available | Medium Term |
| Placing safety ring buoyance in the Aeration tank, stabilization tank etc. . | High | Medium | Effective | Plan to purchase the Safety ring buoyance in fourth quarter of the 2024. | Medium Term |

Table 4.3 Improvemment Option at P4: Clening work in Settling Tank and aeration tank.

| Step of the sanitation service chain: P4: Clening work in Settling Tank area. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: Injury to the body, possible asphyxiation, caused by entering or falling into tanks |  |  |  |  |  |
| Exposure group: 19 labourers and operators . |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effectiveness | Level of resources | Effectiveness under climate change scenarios | Comments/discussion | Priority for improvement plan |
| Prepaire a safety buket and fixing it on the craper bridge. so that the person being cleaned can stand safely. | High | Medium | Effective | If any physical discomfort occurs also it will provides safe to the worker. | High |
| Wearing personal protective equipments during the operation. | High | Low | Effective | Workers should be encouraged to were the PPE | Immediate |

Table 4.2 Improvemment Option at T1: Conveyance of Sewer through the network and pumping

| Step of the sanitation service chain: T1: Conveyance of Sewer through the network and pumping stations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: Ingestion after contact with excreta during the repaire work of blocking pipe lines and damaged pipe lines. |  |  |  |  |  |
| Exposure group: About 10 workers who engaged with maintainance works. |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effectiveness | Level of resources | Effectiveness under climate change scenarios | Comments/discussion | Priority for improvement plan |
| Implementation of regular | High | Medium | Effective | NWSDB, Preventive | Short |
| preventive mainainance |  |  |  | maintenance will reduce | Term |
| programmes covering the |  |  |  | the high risk with |  |
| network. |  |  |  | minimum cost. |  |
| Identify the highly | High | High | In rainy | NWSDB, Current sewer | Medium |
| vulnerable areas and |  |  | season | network is older than 48 | Term |
| refurbishment the |  |  | difficult to | years |  |
| network. |  |  | engaged |  |  |
| Encourage to wear | Medium | Low | Effective | NWSDB provide necessary | Short |
| personal protective |  |  |  | PPE to the workers. | Term |
| equipments during the |  |  |  |  |  |
| maintainance work. |  |  |  |  |  |

Table 4.1 Improvement Option at T2: Conveyance of FS by vacuum trucks

Step of the sanitation service chain: T2: Conveyance of FS by vacuum trucks

Description of hazardous event: Injury to the body, possible asphyxiation, caused by entering or falling into soak pits or septic tanks

Exposure group: About 20 people engaged in fecal sludge transpot to the waste water treatment plants..

IMPROVEMENT OPTIONS

| Option | Effectiveness | Level of resources | Effectiveness under climate change scenarios | Comments/discussion | Priority for improvement plan |
| --- | --- | --- | --- | --- | --- |
| Awareness-raising campain targeting vacume truck operators to avoid the accidents near the septic tank. | Medium | Low | Effective | Awareness programmes Should be organized and carried out in the future. | Immediate |
| providing training on standard operating procedures among service providers | High | Medium | Effective | Training on SOPs should be given. | Medium Term |
| Wearing personal protective equipments during the operation. | High | Medium | Effective | Private service providers are not use PPE and they should be encouraged. | Immediate |

Climate Adaptation Option for a Specific Sanitation Step

| Sanitation Technology | Most Probable Climate Change Scenario | Effect on Sanitation System | Hazardous Event | Adaptation Option |
| --- | --- | --- | --- | --- |
| Wastewater treatment | More intensive storm | Disturb the treatment process by diluting the wastewater and high inflows | Untreated waste water/partially treated wastewater diverted to the boo ela |  |
| Wastewater Treatment | More intensive storm | It disturbs the sedimentation tank. | Untreated waste water/partially treated wastewater diverted to the boo ela |  |

### Development of an Incremental Improvement Plan

The incremental improvement plan was prepairded prioroticing the control measures which did not required major funds or time for the next 3 years

| Improvement measure |  | Cost Rs. |  | Source of funds | Lead organization | Year 1 |  |  |  |  |  |  |  |  |  |  |  | Year 2 |  |  |  | Year 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 1 0 | 1 1 | 1 2 | Q 1 | Q 2 | Q 3 | Q 4 | Q 5 | Q 6 | Q 7 | Q 8 |  |
| T2: Conveyance of FS by vacuum trucks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Awareness-raising campaign targeting vacume truck operators to becareful near the septic tank/ standard operating procedures |  | 15,000 |  | O& M | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wearing personal protective equipments during the operation. |  | 25,000 |  | Gully bowser owners | Gully bowser owners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T1: Conveyance of Sewer through the network and pumping stations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 100,000 |  | O& M | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Implementation of regular preventive mainainance programmes covering the network. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Installation of gate valve in collective well |  | 1,200,0 00 |  | O& M | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify the highly vulnerable areas and refurbishment the network. |  | 500,000 |  | O& M | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Encourage to wear personal protective |  | 10,000 |  | O& M | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| equipments during the maintainance work. |  |  |  |  | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P4: Cleaning work in Aeration, stabilization, settling tank and pump housesetc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rectifying decayed locations in concrete structures |  | 2,000,000 |  | O & M | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wearing personal protective equipment during the operation. |  | 10,000 |  |  | WR-NW Manager office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Implementation of the Improvement Plan

SSP implementation was challenging, and building upon the initial commitments and motivation of all stakeholders required continuous follow-up. The SSP team leader organized meetings with the SSP task forces on a regular basis to review progress and discuss challenges.

## 5.MONITORING OF CONTROL MEASURES AND VERIFICATION OF PERFORMANCE

### Definition and Implementation of Operational Monitoring

Monitoring and verification plans were developed using the Tool 5.1. Tool 5.1 helped to decide which control measures should be monitored to ensure each operation.

Table 5.1 Operational monitoring overview

| Sanitation step | Control measures for a detailed operational monitoring plan |
| --- | --- |
| T2: Conveyance of FS by vacuum trucks | - Wearing Personal Protective Equipment |
| P4: Cleaning work in Aeration, stabilization, settling tank etc. | - Constructing safety fences covering the areas with a possible risk of falling into the tanks - Wearing personal protective equipment during the operation. |

Table 5.2 Operational monitoring plan for T2: Conveyance of FS by vacuum truc

| Operational monitoring plan for: Wearing personal protective equipment during the operation. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
| Vacuum truck operators who discharge fecal sludge to WR-NW | What is monitored? | Whether operators are wearing PPE | What action is to be taken? | Imposing penalty charges |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | At the fecal sludge unloading point of WR-NW | Who takes the action? | Engineering Assistant who approve for discharge |
|  | Who monitors it? | Engineering Assistant/Lab Assistant who approve discharge | When is it taken? | Immediately |
|  | When is it monitored? | Every time they discharge to the WRC | Who needs to be informed of the action? | Regional Manager (WR NW) |

Table 5.4 Operational monitoring plan for T1: Conveyance of Sewer through the network and pumping stations

| - Operational monitoring plan for: Wearing personal protective equipments during the operation and maintainance work. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
|  | What is monitored? | Whether workers are wearing PPE | What action is to be taken? | Apply penalties to workers who do not wear the PPE. |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | Sewer network at the water reclamation center | Who takes the action? | Engineering Assistant who supervised the operation and maintenance work. |
|  | Who monitors it? | Engineering Assistant who supervised the operation and maintenance work. | When is it taken? | After warning 3 times |
|  | When is it monitored? | Every time they are engaged with the O&M works. | Who needs to be informed of the action? | Manager WR |
| All the workers who engage in Operation and maintainance work should wear the PPE. | What is monitored? | Whether workers are wearing PPE | What action is to be taken? | Apply penalties to workers who do not wear the PPE. |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | Sewer network at the water reclamation center | Who takes the action? | Engineering Assistant who supervised the operation and maintenance work. |
|  | Who monitors it? | Engineering Assistant who supervised the operation and maintenance work. | When is it taken? | After warning 3 times |
|  | When is it monitored? | Every time they are engaged with the O&M works. | Who needs to be informed of the action? | Managerr |

Table 5.5 Operational monitoring plan for P4: Cleaning work in Aeration, stabilization, settling tank etc.

| - Operational monitoring plan for: Improve the safety fence where can be exposed to the risks. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
| Collecting well inflow should be stopped during the cleaning works of final well | What is monitored? | Covering all the unsafe areas with safety fence | What action is to be taken? | Fixing safety fence |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | In the Water reclamation center | Who takes the action? | OIC – EA (Civil) |
|  | Who monitors it? | Engineering Assistant | When is it taken? | As soon as possible. (according to the fund availability) |
|  | When is it monitored? | Daily | Who needs to be informed of the action? | Manager |

Table 5.6 Operational monitoring plan for P4: Cleaning work in Aeration, stabilization, settling tank etc

| - Operational monitoring plan for: Wearing personal protective equipments during the Cleaning and operation work. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
| All the workers who engage in Operation and maintenance work should avoid accidents | What is monitored? | Operators are wearing PPE | What action is to be taken? | Apply penalties to workers how do not wear the PPE. |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | At the water reclamation center and sewer network | Who takes the action? | OIC - |
|  | Who monitors it? | Engineering Assistant who supervises the operation and maintenance work. | When is it taken? | After warning 3 times |
|  | When is it monitored? | Every time the workers are engaged with the operation and maintenance work. | Who needs to be informed of the action? | Manager |

  - Verification of System Performance

| Sanitation Step | Verification |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | What | Limit | When | Who | Method |
| T1: Conveyance of Sewer through the network and pumping stations. | the time taken to attend a leak | < 48 hours | Monthly | Manager | Monthly progress report & monthly progress meeting. |
| T1: Conveyance of Sewer through the network and pumping stations. | % of No. of complaints attended within 48 hours. | > 98% | Monthly | Manager | Monthly report & monthly progress meeting |
| T2: Conveyance of FS by vacuum trucks | pH Value, Colour, Odor and Oil | Ph – 6-8 Color and Oder- acceptable level according to the visual observation Oil – Not Allow | Before discharge | Pump Operator | Field testing & Visual observations |
| P4: Treatment Processe | BOD, COD,TSS, Colour Level in the influent, and the effluent of the treatment plant. | COD <250mg/l BOD < 30mg/l TSS < 50 mg/l | Once a week | Lab Assistant | By sample testing |
| P4: Treatment Processe | BOD, COD,TSS, Colour Level in the influent, and the effluent of the treatment plant. | COD <250mg/l BOD < 30mg/l TSS < 50 mg/l | Once a week | Lab Assistant | By sample testing |

  - Auditing the System

It was decided to review and audit the sanition saftety planning of Kurunegala WRC internally once in an year and externally once in an year.

## DEVELOPMENT OF SUPPORTING PROGRAMMES AND REVIEW PLANS

### Identification and Implementation of Supporting Programmes

### Programme to enhance the protection during cleaning of Main collecting well. The ssp was target to improve the safety of workers during the cleaning of main well at Kurunegala wastewater treatment plant. There are is no perfect closing system for stop the entering of waste water to the main collecting well during the preventive maintenance work. Therefore, it was decided to install a gate at manhole prior to the main collecting well. Currently, the preparation of suppliers list is being progressed.

### Programme to enhance the safety on the treatment Plant

### At present, there is no perfect security fence on the treatment plant. Hence, it was decided to install a perfect fence on the plant in order to secure the safety of worker and visitors during walking on the treatment plant floor. Currently, procurement of material for the above is being processed.

- Programme to enhance the coverage with vacuum pump service providers.

The SSP was to target the Kurunegala wastewater treatment plant and the vacuum track service providers who discharge the fecal sludge to the Kurunegala wastewater treatment plant. By conducting the programmes connected with the Environment police, it will be able to increase the number of the private vacuum truck service providers who get the service from Kurunegala wastewater treatment plant. It will ensure the safe sewage disposal in the areas around the Kurunegala by providing a safe end use option.

### Programme to reduce the storm water collecting to the sewer conveyance system.

During the rainy period, storm water is diverted to the sewer collecting network in the Kurunegala Municipal Limits. Therefore, wastewater is diluted and the biological treatment process is disturbed. Therefore, supportive programme to be conducted with Kurunegala Municipal limits to implement a suitable solution such as refurbishment the storm water network.

### Programme to ensure the quality of the treatment process and the effluent by improving the onsite testing facility in the Kurunegala network area.

The quality of the treatment process is verified by testing the influent and effluent at once a week by the water reclamation center laboratory. By increasing the onsite testing facility in the Kurunegala wastewater treatment plant itself, it will be helpful to ensure the quality of the treatment process. It will ensure the safe sewage disposal in the areas around the Kurunegala by providing a safe end use option.

### Map the location of the leak point/overflow, on the network map.

.

### Periodic Reviewing and Updating the SSP Outputs

It was decided to revise the SSP in 12 months according to the improvements of the proposed SSP and the new risk that arose.

Annexes

Planning Template	……

Introduction	3

The requirement for an emergency response plan	3

Section 1. System Information	4

Section 2. Chain of Command – Lines of Authority	5

Section 3. Events that Cause Emergencies	6

Section 4. Emergency Notification	7

Section 5. Effective Communication	71

Section 6. Response Actions for Specific Events	72

Section 7. Alternative Water Sources	14

Section 8. Plan Approval	15

# Introduction

NWSDB, Kurunegala water reclamation center has formulated an Emergency Response Unit. This Unit is planned to response all types of Emergencies in the Kurunegala WRC. The aim of this plan is to treat the waste water due to an emergency with in the 24 hours and to cater the minimum lifeline and waste water treat during emergency until supply is restored or normalcy is attained.

This plan involves mainly Disaster Management Center and other stakeholders of Kurunegala wrc such as Pradeshiya Sabha, Schools, Department of Health, Divisional Secretariat, Irrigation Depatment, CEB, Police and other local authorities.

Emergency Response Plan in Kurunegala wrc can be categories as follows,

- Response Plan

  - Major distribution main failure

  - Power outage in water supply system

  - Intake reservoir/Service tanks

  - Water quality issues

  - Protocol for managing threats to the waste water treat

  - Protocol for response to unauthorized entry to a water supply asset

  - Treat to the system infrastructure/Landslides

- Contingency Plans

  - Intermittent supply

  - Water main shutdown procedures

- Risk Management

- Customer complains

- Financial and Corporate Services

- Strategy and Communications

- People and Culture

- Infrastructure Services

# Section 01. System Information

System information

| Scheme Name and Address | Kurunegala WRC |  |  |  |
| --- | --- | --- | --- | --- |
| Basic Description and Location of System Facilities | Refer SSP |  |  |  |
| Location/Town | Kurunegala |  |  |  |
| Population Served and Service Connections from Division | 10000 people |  | 2237 connections |  |
| System Owner | National Water Supply and Drainage Board |  |  |  |
|  |  | 0114831468 |  | Office Phone |
| Name, Title, and Phone Number of Person Responsible for Maintaining and Implementing the Emergency Plan | Manager, C.J Ransingha | 0777633321 |  | Mobile |

# Section 02. Chain of Command – Lines of Authority

The first response step in any emergency is to inform the person at the top of this list, who is responsible for managing the emergency and making key decisions.

Chain of command – lines of authority

| Name and Title | Responsibilities During an Emergency | Contact Numbers |
| --- | --- | --- |
| OIC-Kurunegala (WRC) | Operation and Maintenance Plant | 0714436521 |
| EA (civil)-Raddoluga ma (WRC) | Operation and Maintenance Network system | 0714898653 |
| O&M Engineer or Area Engineer | Coordination, resource allocation, Operation and Maintenance | 0764583668 |
| Electrical EA | Coordination, resource allocation, Operation and Maintenance | 0773900557 |
| Mechanical EA | Coordination, resource allocation, Operation and Maintenance | 0716407880 |
| Chemist | Water quality analysis | 0771453791 |
| Manager | Coordination, resource allocation | 0777633321 |
| DGM | Authorization | 0772918534 |

# Section 03. Events that Cause Emergencies

The events listed below may cause water system emergencies. They are arranged from highest to lowest probable risk. The list shall be updated as the emergencies occurred in the same WSS and the other WSSs operated in Sri Lanka

Events that cause emergencies

| Type of Event | Probability or Risk (High-Med-Low) | Comments |
| --- | --- | --- |
| Power failure | Low to high |  |
| Chlorine leakage | Low to high |  |
| Mechanical equipment failure | Low to high |  |
| Electronic equipment failure | Low to high |  |
| Earth Slip / drought | Low to high |  |
| Terrorist attack | Low to high |  |
| Biological attack | Low to high |  |

# Section 04. Emergency Notification

Notification call-up lists - Use these lists to notify first responders of an emergency.

| Organization or Department | Name & Position | Telephone | Email |
| --- | --- | --- | --- |
| Local Law Enforcement | OIC-Kurunegala Police station | 0112292857 |  |
| Fire Department | Fire Brigade | 0112422222 |  |
| Emergency Medical Services | Ambulance service | 1990 |  |
| Chlorine leakage | Paranthen chemicals | 0112437556 |  |
| Neighboring Water Reclamation Center (not connected) | OIC- Ekala | 0707517860 |  |
| Electricity Board | C E B Ja ela | 0114344736 |  |
| Irrigation Department | Enginner | 0714443662 |  |
| Hospitals or Clinic(s) | MOH | 0714511732 |  |
| Chemist | T.A.D A. I Jaythilaka | 0771453791 |  |
| Mechanical/Electrical Section | Mech EA Ele Ea | 0716407880 0773900557 |  |
| RDA / PRDA / PS | RDA- EA | 0714472580 |  |
|  | PRDA-General | 0312233621 |  |
|  | PS-TO | 0773753087 |  |
| JCB | Operator | 0716169247 |  |
| Supply Section | Supply Officer | 0714482919 |  |

| Organization or Department | Name & Position | Telephone | Email |
| --- | --- | --- | --- |
| Newspaper - Local | Media notifications done by the Secretary of the Ministry. Only inform to DGM for further actions. |  |  |
| Newspaper – Regional/State/Tribal |  |  |  |
| Radio |  |  |  |
| Radio |  |  |  |
| TV Station |  |  |  |

### Media Notification

Notification procedures

Notify water system customers of potential water shortage

| Who is Responsible: | DGM(GCS)/Manager(Ekala) |
| --- | --- |
| Procedures: | Public Announcement, Media message |

Contact service and repair team

| Who is Responsible: | OIC Kurunegala WRC |
| --- | --- |

| Procedures: | Assigning working team with resources |
| --- | --- |

Contact neighboring wastewater systems, if necessary

| Who is Responsible: | O & M Engineer |
| --- | --- |

| Procedures: | Telephone call/messages |
| --- | --- |

Procedures for issuing a health advisory

| Who is Responsible: | Manager, Chemist, Sociologist, |
| --- | --- |
| Procedures: | Contact MOH office, PHI |

# Section 05. Effective Communication

Communication with customers, the news media, and the general public is a critical part of emergency response.

### Designated public spokesperson

Designate a spokesperson (and alternate) and contact your local primacy agency for delivering messages to the news media and the public.

### Designate a spokesperson and alternates

| Spokesperson | Alternate |
| --- | --- |
| General Manager | DGM-GCS |

Health advisories

During events when water quality and human health are in question, it may be necessary to issue a health advisory that gives advice or recommendations to water system customers on how to protect their health when drinking water is considered unsafe. These advisories are issued when the health risks to the consumers are sufficient, in the estimation of the water system, state or tribal, or local health officials, to warrant such advice.

# Section 06. Response Actions for Specific Events

In any event, there are a series of general steps to take:

- Analyze the type and severity of the emergency

- Take immediate actions to save lives

- Take action to reduce injuries and system damage

- Make repairs based on priority demand

- Return the system to normal operation.

The following tables identify the assessment, set forth immediate response actions, define what notifications need to be made, and describe important follow-up actions.

  - Power outage

| 7 Assessment | Finding out the details for power outage such as duration of outage, affected areas etc from the Electricity board |
| --- | --- |
| Immediate Actions | Telephone Call |
| Notifications | OIC Kurunegala (WRC), Pump operators |
| Follow-up Actions | Power supply from Generator |

  - Network line break

| 8 Assessment | Affected areas and consumers, priority customers, |
| --- | --- |
| Immediate Actions | Assigned team for rectification |
| Notifications | OIC Kurunegala (WRC), O&M Engineer(Ja Ela/Ekala) |
| Follow-up Actions | Immediate rectification / Corrective actions / study the situation / case studies |

  - Chlorine system failure

| 9 Assessment | Chlorine neutralization / Health assessment of infected workers |
| --- | --- |
| Immediate Actions | Contact health services / shut down of the plant / first – aids / rectification of the system / restore the security / commissioning of the plant |
| Notifications | Fire / police / health services / top management |
| Follow-up Actions | Case study / corrective actions |

  - Treatment equipment

| 10 Assessment | Quality of the treated water / PH, BOD,COD,TSS, |
| --- | --- |
| Immediate Actions | Rectify the problem and Recommissioning of plant after rectification |
| Notifications | Consumers / top management Media if situation lasts long |
| Follow-up Actions | Case study / corrective actions |

  - Source pump failure

| 11 Assessment | Quantity of waste water pumping from the plant |
| --- | --- |
| Immediate Actions | Inform Eng(O&M) and M&E group / operation of stand-by pumps |
| Notifications | Eng(O&M) / M&E over the phone / top management |
| Follow-up Actions | Case study / corrective measures / spares arrangement |

  - Microbial (coliform, E. coli) contamination

| 12 Assessment | Bacteriological reports of the treated water |
| --- | --- |
| Immediate Actions | Shut down of the plant / adjust disinfection system / disinfection of stored water / rectification and recommissioning |
| Notifications | OIC (Kurunegala / Eng (O&M) / Chemist |
| Follow-up Actions | Case study / corrective measures |

  - Chemical contamination

| 13 Assessment | Chemical composition report of treated water |
| --- | --- |
| Immediate Actions | Shut down the operation / identification of contaminant source / rectification and recommissioning |
| Notifications | OIC (Kurunegala / Eng(O&M)/Manager(WR-J/E)/ Chemist |
| Follow-up Actions | Case study / corrective measures / evaluating |

  - Vandalism or terrorist attack

| 14 Assessment | Chemical/ Bacteriological composition report of treated water and plant equipment report |
| --- | --- |
| Immediate Actions | Shut down the plant / contact security services |
| Notifications | Workers / public / top management / security services (police and forces) |
| Follow-up Actions | Case study / evaluating / mental condition survey |

I.	Earth Slip

| 15 Assessment | n/a |
| --- | --- |
| Immediate Actions | Shut down of the plant / Protection of M&E equipment / protection of the plant |
| Notifications | Disaster Management Centre / top management / fire |
| Follow-up Actions | Rectification and recommissionning |

- Hazardous materials spill in vicinity of sources or system lines

| 16 Assessment | Chemical / biological and other reports |
| --- | --- |
| Immediate Actions | Rectification and recommissioning |
| Notifications | Top management / consumers |
| Follow-up Actions | Analyzing the situation |

- Electronic equipment failure

| 17 Assessment | Water quality and quantity / responsiveness of the particular equipment |
| --- | --- |
| Immediate Actions | Inform to ME/EE/EA(Mech) |
| Notifications | Eng(O&M)/ R.Manager(WR-Jaela Ekala) |
| Follow-up Actions | R.Manager(WR-Jaela Ekala)Case study / corrective measures |

- Fire

| 18 Assessment | Water quality report of treated water and plant equipment report |
| --- | --- |
| Immediate Actions | Inform to relevant party(OIC/RM) , used fire extinguisher(training person) and go to “assemble point “ |
| Notifications | OIC/Eng(O&M) / R.Manager(WR-Jaela Ekala) |
| Follow-up Actions | Analyze situation |

R.Manager(WR-Jaela Ekala)

- Other

| 19 Assessment | As per the situation |
| --- | --- |
| Immediate Actions | Inform to relevant party / Eng(O&M) / R.Manager(WR-Jaela Ekala) |
| Notifications | Eng(O&M) / R.Manager(WR-Jaela Ekala) |
| Follow-up Actions | Analyze situation |

Section 7. Alternative Water Sources

Alternate source(s) of water

| Alternative Sources | Names | Phone | Availability | Is the Water Safe for Drinking? |
| --- | --- | --- | --- | --- |
| Tanker trucks in | NWSDB Jae | Eng (O&M) | Available | Safe |
| the area available | la/Ekala (WR), |  |  |  |
| to deliver bulk |  |  |  |  |
| water for potable |  |  |  |  |
| use |  |  |  |  |
| Tanker trucks in | Disaster Mgt Cen. AG Offices (PS) Irrigation Dept. | 0112136136 | Available | Safe |
| the area available |  | 031 2222836 |  |  |
| to deliver bulk |  | 0773753087 |  |  |
| water for potable |  | 0714443662 |  |  |
| use (with other |  |  |  |  |
| organizations) |  |  |  |  |
| Tanker trucks in | PRDA | 0312233621 | Available | Unsafe |
| the area available | Irrigation Dept. |  |  |  |
| to deliver bulk water for |  | 0714443662 |  |  |
| non-potable use |  |  |  |  |

Section 8. Plan Approval

This plan is officially in effect when reviewed, approved, and signed by the following people:

| Name/Title | Signature | Date |
| --- | --- | --- |
| G.D.M Nevil (DGM) |  |  |
| Eng.C.J Ranasingha (R.Manager(WR- Ekala/Jaela) |  |  |
| Eng.D.M.W.JB. Dissanayaka.(O &M Engineer) |  |  |
| A.W.A.D.J.R.Abesekara(OIC Kurunegala WRC) |  |  |