2.1 ………

MORATUWA - RATHMALANA

SANITATION SAFETY PLAN

# PREPARATION FOR SSP

## ESTABLISHING PRIORITY AREAS.

A Sanitation Safety Plan (SSP) is a comprehensive strategy and set of protocols designed to ensure the safe and efficient operation of a wastewater or sewage system. The key focus areas and activities outlined in this SSP involve the continued operation and maintenance of critical infrastructure, including the wastewater treatment plant, pump houses, and the sewer pipe network. Additionally, it prioritizes the expansion of the network by facilitating new connections and promptly addressing consumer concerns such as blockages and leaks.

## OBJECTIVES

The primary objectives of an SSP are to protect public health, the environment, and the integrity of the sewer system itself.

- Public Health Protection: To prevent the contamination of water sources and the spread of waterborne diseases by expanding the sewer network and encouraging household and industries to connect their waste to public sewer network.

- Environmental Protection: To minimize pollution and adverse effects on ecosystems, including rivers, lakes, and oceans by minimizing the disposal of septic waste to waterbodies by people and encourage to connect their waste to sewer network.

- Infrastructure Integrity: To ensure that the sewer network itself is maintained and operated safely to prevent issues like leaks, overflows, or system failures. The SSP includes strategies for regular inspection, maintenance, and repair of sewer infrastructure to prevent structural failures and protect the system's longevity.

## SYSTEM BOUNDARY AND LEAD ORGANIZATION

The SSP's boundaries will be explicitly defined, restricted to areas in the Moratuwa Rathmalana region where the sewer network is available. Specifically, within the context of the network, it will extend solely up to the IC chamber. Consumer responsibility will encompass internal plumbing and toilet hygiene facilities. Moreover, the SSP's scope will be confined to wastewater that adheres to influent standards and will exclusively involve NWSDB's registered gully bowser service providers for treating septic waste.

Lead Organization will be Manager office- Water Reclamation (Deh/Mt.Lav/Mor./Rath) of National Water Supply and Drainage Board

## SSP TEAM

A Sanitation Safety Plan (SSP) team is crucial for the successful development, implementation, and monitoring of a comprehensive sanitation safety plan. It typically consists of two main groups: the Core Team and the Multi-Stakeholder Team.

Core Team:

The Core Team is primarily responsible for the development, implementation, and management of the SSP. The team will be composed of dedicated NWSDB employees affiliated with the Dehiwala/Mt. Lavinia/Moratuwa/Rathmalana manager's office. Their responsibilities encompass the management and operation of the sewer network, including routine maintenance, facilitating new connections, identifying non-conformities, and addressing consumer concerns. Additionally, they will actively collaborate with other essential stakeholders in the sanitation chain, extending the SSP team through the identification and invitation of key participants.

Multi-Stakeholder Team:

The team represents a diverse group of individuals and organizations with a vested interest in sanitation safety,

| including Government | Authorities, | Community |
| --- | --- | --- |
| Representatives, Industry | Representatives, | Academics, |

Healthcare Professionals, and Environmental Advocates who

have a vested interest in the region. They will provide input, share concerns, and contribute to the development and implementation of the SSP.

Collaboratively, the Core Team and Multi-Stakeholder Team will strive for the successful implementation and ongoing maintenance of the SSP. Both teams will convene annually to conduct a comprehensive review, facilitating discussions and updates to ensure the continued relevance and effectiveness of the SSP.

| SSP Member | Key Role |
| --- | --- |
| Manager (WR) Deh/MtLavinia/Mor/Rath- NWSDB | Provide leadership to the team and coordinate with all stakeholders |
| Technical Staff –NWSDB Deh/MtLavinia/Mor/Rath | Maintaining and monitoring the network, treatment process and planning the network expansion. |
| Workers-NWSDB Deh/MtLavinia/Mor/Rat | Attending the repairs, blockages etc |
| Environmental Advocates | Identifying and providing insights about environmental risks |
| Academics | Conducting researches and providing consultancy services |
| Industry Representatives | Coordinating role between the business community and the SSP team |
| Community Representatives | Coordinating role between the community and the SSP team |
| Government Authorities | Supporting and guiding with their insights. |
| Healthcare Professionals | Providing insights about health concerns related to sanitation. |

# DESCRIBE THE SANITATION SYSTEM

## 2.1. MAP THE SYSTEM

Moratuwa-Rathmalana, located on Sri Lanka's southwestern coast in the Western province, is a densely populated area. It's bordered by Mt. Lavinia in the north and the Bolgoda River in the south. Covering around 22.5 square kilometers, Moratuwa has about 9,250 people per square kilometer, and Rathmalana has about 7,700 people per square kilometer, as of the 2020 Census. This region is known for various industries, including food production, clothing manufacturing, furniture making, rubber products, batteries, transformers, and wood crafts.

Moratuwa Rathmalana's water reclamation division manages providing new sewer connections, network maintenance, customer complaints, and maintaining treatment plant and its process. On the other hand, areas that cannot be connected to main sewer lines or the dwelling units that are not willing to be connected to the main sewer lines, but have septic tanks, are provided with sanitation services that include regular desludging of septic tanks, as well as the coordination works with the customers.

The SSP of Moratuwa Rathmalana encompasses the entire wastewater infrastructure managed by the Mor-Rathalana network. This includes household and industrial connections to the sewer system, pumping stations, the wastewater treatment plant (WWTP), treatment of WWTP sludge, and the disposal of treated wastewater into the shallow sea near Angulana. Additionally, the disposal of treated WWTP sludge is included. Since some residences use on-site systems like septic tanks, the faecal sludge management system operated by the service providers is also covered within the scope of the Sanitation Safety Plan (SSP).

The sanitation system of this area can be mapped as follows

### P1 & P2 & P2a : Wastewater Generation

Wastewater is basically added to the system from direct connections from the households & industries and from unloading septic tanks. Wastewater treatment plant receives an average monthly flow of 160,000 cubic meters. This flow comprises nearly 93,000 cubic meters from industrial and commercial connections, 64,000 cubic meters from domestic connections, and 3,000 cubic meters from vacuum trucks.

### T1 P2a & T2 : Transmission of Wastewater

The sewerage system encompasses a total of 5,200 connections, including over 250 designated for industrial use. The network comprises four main pumping stations and additional four sub- pumping stations, all operating collaboratively to convey wastewater to the treatment plant. Spanning a distance of 40 kilometers, this comprehensive network strategically incorporates more than 300 manholes along its route, simplifying maintenance and inspection processes. It consistently and reliably manages an average monthly flow of 160,000 cubic meters of wastewater. The pipeline infrastructure is constructed using a combination of materials, including HDPE, PE, and PVC pipes, ensuring the system's resilience and

operational efficiency.

The Water Reclamation Centre accepts gully bowers from households and industries that meet specific acceptance criteria. Septage from households and commercial units is transported using private gully bowers and pre-registered gully browsers, both from NWSDB and other local authorities. Before unloading, all gully browsers undergo laboratory checks for parameters such as oils, pH, and temperature.

### Treatment plant process description

The wastewater treatment plant, planned in two stages with a combined daily inflow capacity of 28,870 cubic meters, is currently in its initial operational phase, capable of treating up to 17,000 cubic meters of wastewater daily. This facility employs a comprehensive biological reverse A2O

treatment approach, addressing not only the removal of bio degradable matters, but also targeting the removal of nitrogen and phosphorus compounds from the incoming wastewater.

### P3 Preliminary Treatment

Within the inlet channel, a mechanical coarse screen, cleaned periodically, is installed. The water is then pumped to two parallel fine screens, followed by two parallel grit traps for effective removal of debris. The fine screens have free slots of 3 mm between the bars and their function are to remove rags, paper and other large particles to prevent clogging, blockages and similar in the downstream treatment facilities.

Removed screenings are transported by a screw conveyor to a screenings washer press. The screenings are washed, dewatered, compacted and disposed into a container. The screened water is led to two (four at final stage) mixed parallel grit traps, where sand and other heavy particles are removed.

### P7 Grit Removal

The settled grit is pumped to a sand separator. After the grit has been dewatered and rinsed in the sand separator, it is dumped in a container

### P4 & P5 Secondary Treatment & outfall

The biological treatment comprises two parallel trains, each equipped with denitrification, Bio-P (phosphorus removal), and nitrification basins, followed by individual clarifiers. The clarified water is subsequently directed toward the recipient, and there's the option to add Sodium Hypochlorite to the treated water before it leaves the plant for further disinfection in water borne disease pandemic situations. Effluent is finally released to sea through a short sea outfall.

### P6 Sludge Management

Regarding excess sludge generated during the activated sludge process, it's directed by gravity to two parallel sludge storage basins. Sludge thickened at these basins and pumped to a belt filter press for dewatering, reducing its moisture content. The dewatered sludge is then placed in containers and transported to solar sludge drying bed. Sludge will be agitated and dried at solar

sludge drying bed. Then store these sludge in sludge storage and transport away from the plant for final disposal, completing the wastewater treatment cycle.

| Code | Process/ equipment | Purpose | Waste stream & potential hazards |
| --- | --- | --- | --- |
| P1,T1 | Sewerage disposal from households and industries | Collection of wastewater to the network | Health hazards if blocked or leaked. |
| P2, T2 | Loading fecal sludge emptying truck | Emptying Septic tanks | Spillage, Gas inhalation, contact |
| P2A ,T2 | Fecal Sludge unloading at plant | Disposing the facal sludge to treatment plant | Spillage, Leakages, Contacting |
| P3 | Preliminary/physical Treatment - Coarse screen | Prevent large (>40mm) floating particles entering the treatment plant | Risk of contact with pathogens, helminth eggs, and solid waste during cleaning and maintenance work, odour related health risks |
| P3 | Preliminary/physical - Fine Screen | Prevent f (>4mm) floating particles entering the secondary treatment phase | Risk of contact with pathogens, helminth eggs, and solid waste during cleaning and maintenance work, odour related health risks |
| P3 | Preliminary/physical - Grit Chamber | Trap and remove grit/sand | Risk of contact with pathogens, helminth eggs, and solid waste |

| P4 | Anoxic Tank | Denitrification to remove Nitrogen | Drowning, contact with pathogens, helminth eggs, and solid waste |
| --- | --- | --- | --- |
| P4 | Anaerobic Tank | Removal of P | Drowning, contact with pathogens, helminth eggs, and solid waste |
| P4 | Aerobic Tank | Supply O2 for the influent | Drowning, contact with pathogens, helminth eggs, and solid waste |
| P5 | Sea Outfall |  | Contamination of sea with pathogens, heavy metals |
| P6 | Sludge management system | Dewatering the sludge, mix with polymer and drying | contact with pathogens, helminth eggs, heavy metals , and solid waste |
| P7 | Grit separator | Separating the grit from water | contact with pathogens, helminth eggs, and heavy metals |
| P7 | Grit Disposal | Transporting grit to dumping yards/Land fills | contact with pathogens, helminth eggs, and heavy metals |

## CHARACTERIZE THE WASTE FRACTIONS

- Septic Tank Effluent: This component primarily consists of water, excreta, and urine. It represents the liquid waste discharged from septic tanks.

- Greywater: Greywater encompasses all domestic wastewater generated in households, originating from bathrooms and kitchens, but does not typically contain fecal matter and urine.

- Septic Tank Sludge: Comprising solids and water settled at the bottom of septic tanks, septic tank sludge has the potential to contain, feminine hygiene products, sharps, and other foreign materials. Proper handling and disposal of septic tank sludge are crucial to prevent contamination.

- Industrial Wastewater: It originates from various industrial processes and can contain a multitude of contaminants, depending on the industry. These contaminants may include heavy metals, chemicals, toxins, and other pollutants specific to industrial activities.

| Physical Hazards |  |  |  | Biological Hazards |  |  |  |  | Chemical Hazards |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compone nts | Inorganic Material | Sharp Objects | Malodorous | Virus | Bacteria | Protozoa | Helminths | Vector Related Diseases | Heavy Metals | Toxic Chemicals |
| Liquid waste fractions |  |  |  |  |  |  |  |  |  |  |
| Diluted Excreta |  |  | √ | √ | √ | √ | √ |  |  |  |
| Human Urine |  |  | √ | √ | √ | √ | √ |  |  |  |
| Domestic waste water | √ | √ | √ | √ | √ | √ | √ | √ |  |  |
| Industrial wastewate r | √ | √ | √ |  |  |  |  |  | √ | √ |
| Solid waste fractions |  |  |  |  |  |  |  |  |  |  |
| Fecal sludge | √ | √ | √ | √ | √ | √ | √ | √ |  |  |
| WWTP sludge | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |

| Organic domestic waste |  | √ | √ | √ |  |
| --- | --- | --- | --- | --- | --- |
| Inorganic domestic waste | √ | √ |  | √ | √ |

## IDENTIFY POTENTIAL EXPOSURE GROUPS

| Symbol | Short Name | Short Description |
| --- | --- | --- |
| S | S | NWSDB Staff that are involved with the O&M and extension projects within the region |
| W | Workers | The division's workforce includes employees responsible for the operation and maintenance of the network and treatment plant, as well as those who handle customer inquiries and concerns. |
| G | Gully Service Providers | Service providers responsible for transporting septic waste from septic tanks to the treatment plant |
| C | Domestic Consumers | Individuals connected to the public sewer network and people who utilize gully services to empty their septic tanks. |
| I | Industry Owners and employees | Industries that are connected to the public sewer network and who utilize gully services to empty their septic tanks. |

| P | Public | Public residing, visiting, and working in areas served by the sewer network and in proximity to the treatment plant. |
| --- | --- | --- |
| U | End users | People who take away removed grit and sludge. |
| V | Visitors | External parties who visit the WWTP including students, foreigners and researchers. |

  - GATHER COMPLIANCE AND CONTEXTUAL INFORMATION

Laboratory functioning at Soysapura WWTP test and monitor different parameters in the influent and effluent to ensure that the receiving wastewater and discharging treated water comply with the standards set by Central Environmental Authority (CEA). Compliance is checked accordance to Gazette No:2264/17, 2022/1/27. Summary of test reports done at inlet, outlet, from gully bowsers and from industries are given below.

Weekly test reports at inlet and outlet of the WWTP

| No | Parameter |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Inlet |  | Outlet |  |
|  |  | Tolerance Limits For Discharge of Effluents Into Public Sewers with Central Treatment Plants. | Annual Average Test Result | Tolerance Limits for the discharge of wastewater or effluent into a short sea outfall | Annual AverageTest Result |
| 1 | Temperature (℃) | 45 | 27.7 | 35 | 28.1 |
| 2 | pH | 5.5-9.0 | 7.2 | 5.5-9.0 | 7.3 |
| 3 | DO (mg/l) |  | * |  | * |
| 4 | Colour (m-1) Y R B | 7 / 5 / 3 | 1.6/0.9/0.6 | 7 / 5 / 3 | 1.0/0.6/0.3 |
| 5 | BOD5 (mg/l) | 400 | 207.2 | 75 | 8.1 |
| 6 | COD (mg/l) | 800 | 568 | 400 | 49 |
| 7 | Total P (mg/l) | 3.5 | ** |  | ** |
| 8 | Total N (mg/l) |  | 20.7 |  | 15.4 |
| 9 | Dissolved Phosphate as P (mg/l) |  |  | 5 |  |
| 10 | NH4- N (mg/l) | 50 | - | 50 | - |
| 11 | NO3-N (mg/l) |  | 0.4 |  | 3.2 |
| 12 | Fluorides (as F) (mg/l) | 10 | 0.4 | 2 | 0.1 |

Inlet BOD

500

450

400

350

300

250

200

150

100

50

0

Data

04.01.2022

19.01.2023

02.02.2023

13.02.2023

01.03.2023

17.03.2023

30.03.2023

10.04.2023

27.04.2023

10.05.2023

25.05.2023

08.06.2023

21.06.2023

07.07.2023

19.07.2023

04.08.2023

16.08.2023

28.08.2023

14.09.2023

26.09.2023

12.10.2023

26.10.2023

08.11.2023

22.11.2023

07.12.2023

21.12.2023

| 13 | TSS (mg/l) | 500 | 402 | 50 | 9.1 |
| --- | --- | --- | --- | --- | --- |
| 14 | MLSS (mg/l) |  | - |  | - |
| 15 | SV30 (ml/l) |  | - |  | - |
| 16 | SVI (ml/g) |  | - |  | - |
| 17 | Total Solids (mg/l) |  | - |  | - |
| 18 | Total Dissolved Solids (mg/l) | 3000 | 680 | - | 711 |
| 19 | MLVSS (mg/l) |  | - |  | - |
| 20 | F/M Ratio (kg BOD5/kgSS,d) |  | - |  | - |
| 21 | Oil & Grease | 20 | 16.6 | 12 | 0.5 |

Inlet COD

2000

1500

1000

500

0

Data

04.01.2022

19.01.2023

02.02.2023

13.02.2023

01.03.2023

17.03.2023

30.03.2023

10.04.2023

27.04.2023

10.05.2023

25.05.2023

08.06.2023

21.06.2023

07.07.2023

19.07.2023

04.08.2023

16.08.2023

28.08.2023

14.09.2023

26.09.2023

12.10.2023

26.10.2023

08.11.2023

22.11.2023

07.12.2023

21.12.2023

SANITATION SAFETY PLAN

Inlet Total N (mg/l)

30.0

25.0

20.0

15.0

10.0

5.0

0.0

Data

Outlet BOD

20

18

16

14

12

10

8

6

4

2

0

Data

MORATUWA-RATHMALANA

17

04.01.2022

19.01.2023

02.02.2023

13.02.2023

01.03.2023

17.03.2023

30.03.2023

10.04.2023

27.04.2023

10.05.2023

25.05.2023

08.06.2023

21.06.2023

07.07.2023

19.07.2023

04.08.2023

16.08.2023

28.08.2023

14.09.2023

26.09.2023

12.10.2023

26.10.2023

08.11.2023

22.11.2023

07.12.2023

21.12.2023

SANITATION SAFETY PLAN

Outlet COD

100

90

80

70

60

50

40

30

20

10

0

Data

OUTLET TOTAL N

20.0

18.0

16.0

14.0

12.0

10.0

8.0

6.0

4.0

2.0

0.0

Data

Tolerance Limit

MORATUWA-RATHMALANA

18

Testing of Gully Bowsers before discharging

| No | Parameter | Name of the Gully Service , Date & Time of collection, Lab Reg. No. |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MDK | M/C Moratuwa | Sara Lanka | Sriyani Enterprises |
|  |  | 5/B, Horana Road, Pokunuwita | 3/26, Ihala Indibedda, Moratuwa | 37, Pasal Mawatha, Pannipitiya | 53, Niudawa Road, Niudawa, Panadura |
|  |  | 06/10/2023 at 10:35 Hrs 2023/WW/ 329 | 06/10/2023 at 10:50 Hrs 2023/WW/ 330 | 06/10/2023 at 11:05 Hrs 2023/WW/ 331 | 18/10/2023 at 12:15 Hrs 2023/WW/ 349 |
| 1 | pH | 6.2 | 7.4 | 7.5 | 7.8 |
| 2 | Temperature (℃) | 26.5 | 27.2 | 27.4 | 28.9 |
| 3 | BOD5 (mg/l) | 4320 | 120 | 2610 | 173 |
| 4 | COD (mg/l) | 11213 | 768 | 14592 | 718 |
| 5 | TSS (mg/l) | 24300 | 960 | 18420 | 820 |

Testing of influent from Industries

| No | Parameter | Tolerance limits for the discharge of effluents into public sewer network connected to a common treatment plant. | Name of the Factory , Date & Time of collection, Lab Reg. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FAB | Oceanic Knitters | Janasiri (Pvt) Ltd. | Brandix (Pvt) Ltd. | Sigiri Weavin g (Pvt) |
|  |  |  | 12.10.2023 at 11:00 Hrs. 2023/WW/334 | 12.10.2023 at 11:10 Hrs. 2023/WW/335 | 26.10.2023 at 11:30 Hrs. 2023/WW/351 | 26.10.2023 at 11:40 Hrs. 2023/WW/352 | 27.10.2023 at 11:20 Hrs. 2023/WW/355 |
| 1 | pH | 5.5-9.0 | 6.3 | 6.9 | 7.8 | 7.5 | 7.6 |
| 2 | Temperature (℃) | 45 | 28.5 | 28.4 | 30.4 | 30.1 | 35.9 |
| 3 | BOD5 (mg/l) | 400 | 755 | 114 | 18 | 15 | 39 |
| 4 | COD (mg/l) | 800 | 2445 | 323 | 199 | 199 | 627 |
| 5 | TSS (mg/l) | 500 | 568 | 7 | 82 | 86 | 110 |
| 6 | TDS (mg/l) | 3000 | 1269 | 2164 | 4331 | 1541 | 5085 |
| 7 | Colour (m-1) Y/R/B | 7 / 5 / 3 | 1.2/0.5/0.3 | 1.4/0.7/ 0.4 | 10.7/4. 7/2.5 | 1.3/0.6 /0.2 | 5.3/2.9/ 2.5 |

## VALIDATE THE SYSTEM DESCRIPTION

### Sewer Network Inspection

To evaluate the sewer network's condition, we conduct field investigations to detect any indications of infiltration, damage, or blockages. Our dedicated team employs specialized tools and equipment, including CCTV cameras, sewer rods, and a vacuum truck unit (VTU), to swiftly and precisely identify any anomalies. Furthermore, we perform regular preventive maintenance on the sewer network to uphold its ongoing reliability and efficiency.

### Environmental Compliance of the Water Reclamation Section

The Water Reclamation division is committed to conforming to the environmental quality standards set forth by the Central Environmental Authority and THE NATIONAL ENVIRONMENTAL ACT, No. 47 of 1980. All waste components identified throughout the entire sanitation system must adhere to these rigorous standards before undergoing disposal or, where applicable, potential reuse. This dedication ensures the observance of environmental regulations and the promotion of sustainable practices.

# HAZARD IDENTIFICTION

Network

| Hazard | Risk | Safety Measures |
| --- | --- | --- |
| Confined Spaces (Manholes, Sewer Lines) | Entering confined spaces within the sewer network | Confined space entry protocols ( Check O2 levels, operating exhaust fans etc ), proper training, and safety equipment |
| Toxic Gases in Sewer Atmosphere | Accumulation of toxic gases (e.g., hydrogen sulfide) | Gas detection systems, proper ventilation, and worker training |
| Falls into Open Manholes | Open manholes have a fall hazard | Proper covering of manholes, barriers, and warning signs |
| Exposure to Contaminated Water | Contact with contaminated water during maintenance activities | Use of PPE, including gloves, boots and waterproof clothing |
| Biological Hazards in Sewage | Exposure to pathogens in raw sewage | Hygiene practices, PPE, and vaccination programs |
| Traffic Hazards | Working near roads during sewer maintenance | Traffic control measures, reflective clothing, and proper signs |
| Equipment Hazards | Use of heavy equipment for maintenance activities | Training for equipment operators, regular equipment maintenance, and safety checks |
| Collapse of Sewer Structures | Structural failures leading to collapses | Regular inspections and proper maintenance |
| Chemical Contaminants | Presence of hazardous chemicals in the sewer system | Hazard identification, proper handling procedures, and PPE |
| Drowning Hazards | Working in areas with standing or flowing water | Adequate training, use of life jackets, and emergency response plans |

Wastewater Treatment Plant:

| Hazard | Risk | Safety Measures |
| --- | --- | --- |
| Pathogen Exposure | Handling raw sewage and wastewater | Adequate PPE, hygiene practices, and vaccination programs |
| Chemical Exposure | Use of chemicals for treatment processes | Proper training, PPE, and safe storage and handling practices for chemicals |
| Exposure to odorous and hazardous gasses | Inhalation of gasses | Ventilation systems, respiratory protection, and regular air quality monitoring |
| Slips, Trips, and Falls | Wet and slippery surfaces in the treatment plant | Non-slip flooring, handrail systems, and proper signage |
| Confined Spaces | Tanks and chambers may be confined spaces | Confined space entry protocols, training, and safety equipment like gas detectors and harnesses |
| Noise Exposure | High noise levels from machinery and equipment | Hearing protection, noise control measures, and regular noise exposure assessments |
| Mechanical Hazards | Moving machinery and equipment in the plant | Guarding of machinery, training, and regular maintenance |
| Electrical Hazards | Electrical equipment used in the plant | Proper installation and maintenance of electrical systems, worker training in electrical safety |
| Chemical Storage and Handling | Improper storage and handling of treatment chemicals | Secure chemical storage, secondary containment, and spill response training |
| Failures in Treatment Process | Disruptions in the treatment process leading to hazards | Monitoring systems, regular inspections, and contingen |

|  | Component | Hazard Identification |  |  |  |  | Existing Control(s) |  | Risk Assessment |  |  |  |  |  | Comments justifying risk assessment, under current conditions or climate change scenarios, or effectiveness of the control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Under current conditions L=Likelihood; S=Severity; R=Risk |  |  |  | Under the most likely climate change scenarios: |  |  |
|  | Sanitation step | Hazardous event | Hazard | Exposure Groups | No.of people at risk |  | Description of existing control | Validation of control | L | S | Score | R | Rain | Drought |  |
|  | Wastewater Generation Leaks & Overflowing of ICs &MHs | Vector breading | Physical, Biological | W,P |  | Vector borne with flies | PPM | observation | 3 | 8 | 8 | M | + | - |  |
|  | Wastewater Generation Internal leakages at bathrooms and toilets | Exposure to raw sewage | Biological | P |  | Skin contact | Awareness programmes | observation | 3 | 2 | 8 | M | + | - |  |
|  | T1 P2a & T2 : Transmission of Wastewater Block Clearance | Exposure to raw sewage | Biological | W,P,G,I,C |  | Inadvertent ingestion Dermal contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 5 | 4 | 20 | H | + | - |  |

|  | (IC, MH & Pipe lines) | Inhalation of toxic Gases/ Odour | Biological | W,P,G,I,C |  | Respiratory Tract | Confined space entry protocols, training,PPE, Gas detection systems, proper ventilation | observation | 3 | 4 | 12 | M | + | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Falls into Open Manholes, confine space | Physical | W,P,G,I,C |  | Skin contact | Proper covering of manholes, barriers, and warning signs | observation | 3 | 8 | 24 | H | = | = |  |
|  |  | Traffic Hazards | Physical | W,P |  | Skin contact | Traffic control measures, reflective clothing, and proper signs | observation | 3 | 8 | 24 | H | = | = |  |
|  |  | Equipment Hazards | Physical | W |  | Skin contact | Training for equipment operators, regular equipment maintenance, and safety checks | observation | 3 | 8 | 24 | H | = | = |  |

|  |  | Collapse of Sewer Structures | Physical | W,C,P |  | Inadvertent ingestion, Skin contact | Regular inspections and proper maintenance | observation | 2 | 16 | 32 | H | = | = |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Chemical Contaminants | Chemical | W,G,I |  | Respiratory Tract | Hazard identification, proper handling procedures, and PPE | observation | 1 | 8 | 8 | M | = | = |  |
|  | T1 P2a & T2 : Transmission of Wastewater Repair & Maintenance of Net work and Pumphouses including Screen cleaning, sand remval, M&E PPMs | Exposure to raw sewage during repairs and PPMs | Biological | W,G,C,I,P |  | Inadvertent ingestion Dermal contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 5 | 4 | 20 | H | + | - |  |
|  |  | Inhalation of toxic Gases/ Odour during repairs and PPMs | Biological | W, G,I |  | Respiratory Tract | Confined space entry protocols, training,PPE, Gas detection systems, proper ventilation | observation | 4 | 4 | 16 | H | + | - |  |
|  |  | Falls into Open Manholes, confine space during repairsand PPMs | Physical | W,C,P |  | Skin contact | Proper covering of manholes, barriers, and warning signs | observation | 3 | 8 | 24 | H | = | = |  |

|  |  | Traffic Hazards during repairs and PPMs | Physical | W,P |  | Skin contact | Traffic control measures, reflective clothing, and proper signs | observation | 2 | 8 | 16 | H | = | = |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Equipment Hazards | Physical | W |  | Skin contact | Training for equipment operators, regular equipment maintenance, and safety checks | observation | 2 | 8 | 16 | H | = | = |  |
|  |  | Collapse of Sewer Structures | Physical | W,C,P |  | Inadvertent ingestion,Ski n contact | Regular inspections and proper maintenance | observation | 2 | 8 | 16 | H | = | = |  |
|  |  | Chemical Contaminants | Chemical | W,G,I |  | Respiratory Tract | Hazard identification, proper handling procedures, and PPE | observation | 1 | 8 | 8 | M | = | = |  |
|  | T1 P2a & T2 : Transmission of Wastewater |  | Biological (Pathogen s, | W |  | Inhalation Dermal contact | Use of PPE, including gloves, boots | observation | 5 | 4 | 32 | H |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | + | - |  |

|  | Loading, transporting and unloading | Exposure to raw septage during loading | helminth) Chemical (heavy metal), Physical |  |  | Inadvertent ingestion | and waterproof clothing |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Traffic Hazards | Physical | W,P |  | Skin contact | Traffic control measures, reflective clothing, and proper signs,competeta nt drivers | observation | 2 | 8 | 16 | H | = | = |  |
|  |  | Falls into Open Manholes, confine space( septic tanks | Physical | W,C,P |  | Skin contact | Proper covering of manholes, barriers, and warning signs | observation | 2 | 16 | 32 | H | = | = |  |
|  | T1 P2a & T2 : Transmission of Wastewater Overflowing of MHs due to intense rainfall | Exposure to raw sewage | Biological | W,G,C,I,P |  | Inadvertent ingestion Dermal contact | Awareness, Capacity enhancing | observation | 5 | 4 | 20 | H | + | - |  |
|  | T1 P2a & T2 : Transmission of Wastewater Providing ne | Exposure to raw sewage | Biological | W,G,C,I,P |  | Inadvertent ingestion Dermal contact | Awareness, Capacity enhancing | observation | 5 | 4 | 20 | H | + | - |  |

|  | connecions & Extension projects | Traffic Hazards | Physical | W,P |  | Skin contact | Traffic control measures, reflective clothing, and proper signs,competeta nt drivers | observation | 2 | 8 | 16 | H | = | = |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mechanical Hazards | Physical | W |  | Skin contact | Guarding of machinery, training, and regular maintenance | observation | 3 | 8 |  | H | = | = |  |
|  | P3 Preliminary Treatment Cleaning of Screens,removal of grit, PPMs, Repairs | Exposure to raw sewage during repairs and PPMs | Biological | W,G,C,I,P |  | Inadvertent ingestion Dermal contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 4 | 4 | 16 | M | + | - |  |
|  |  | Inhalation of toxic Gases/ Odour during repairs and PPMs | Biological | W, G,I |  | Respiratory Tract | Confined space entry protocols, training,PPE, Gas detection systems, proper ventilation | observation | 3 | 4 | 12 | M | + | - |  |
|  |  | Health risk due to exposure to grit, |  | W |  | ingestion Inhalation | Introduce PPE issuance/Conduc | observation | 2 | 4 | 12 | M | = | = |  |

|  |  | wastewater during grit removal | Physical, Chemical, Biological |  |  |  | ting Awareness programs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Health risk due to exposure to dried sludge or solid waste | Chemical (heavy metals), Biological (Pathogen s), Physical | W,U |  | Dermal contact Inadvertent ingestion | Use of PPE, giving awareness | observation | 3 | 4 | 12 | M | = | = |  |
|  | P4 Secondary Treatment | Inhalation of toxic Gases/ Odour | Biological | W, G,I |  | Respiratory Tract | PPE, Gas detection systems, proper ventilation | observation | 2 | 8 | 16 | H | + | - |  |
|  |  | Falls into Open Manholes, confine space | Physical | W,C,P |  | Skin contact | Proper covering of manholes, barriers, and warning signs | observation | 2 | 16 | 32 | H | = | = |  |
|  |  | Mechanical Hazards | Physical | W |  | Skin contact | Guarding of machinery, training, and regular maintenance | observation | 3 | 8 | 24 | H | = | = |  |

|  |  | Electrical Hazards | Physical | W |  | Skin contact | Proper installation and maintenance of electrical systems, worker training in electrical safety | observation | 3 | 16 | 48 | H | + | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Health risk due to Inhalation of aerosol during plant operation | Biological, Chemical, Physical | W |  | Inhalation | Introduce PPE issuance/Conduc ting Awareness programs | observation | 3 | 2 | 8 | M | + | - |  |
|  |  | High noise levels from machinery and equipment | Physical | W |  | Skin contact | Hearing protection, noise control measures, and regular noise exposure assessments | observation | 1 | 2 | 2 | L | = | = |  |
|  | P4 Secondary Treatment Handling Chemicals | Riskof storing and handling of treatment chemicals | Chemical | W |  | Dermal contact | Secure chemical storage, secondary containment, and spill response training | observation | 3 | 4 | 12 | M | = | = |  |

|  | P4 Secondary Treatment Sample collectiong | Inhalation of aerosol | Biological, Chemical | W |  | Inhalation | Introduce PPE issuance/Conduc ting Awareness programs | observation | 3 | 4 | 8 | M | + | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Exposure to raw sewage | Biological | W,G,C,I,P |  | Inadvertent ingestion Dermal contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 4 | 4 | 16 | H | + | - |  |
|  |  | Trips, Slips and Falls | Physical | W,C,P |  | Skin contact | Proper covering of manholes, barriers, and warning signs | observation | 4 | 8 | 32 | H | = | = |  |
|  | P4 Secondary Treatment Process | Disruptions in the treatment process leading to hazards | Biological | W,G,I,P |  |  | Monitoring systems, regular inspections, and contingency planning | observation | 3 | 8 | 24 | H | + | - |  |
|  |  | Variability in raw sewage characteristics, impact on treatment processes | Biological | W |  |  | Monitoring of water quality parameters in the sewage network. | observation | 3 | 4 | 12 | M | = | = |  |

|  |  | Risk of exceeding plant capacity due to Increased Intensity and Frequency of Storm Events | Biological | W |  |  | Contingency Plan | observation | 2 | 4 | 8 | M | + | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P5 Outfall | Not complying to the standards ( Affecting the ocean ecosystem) | Biological |  |  |  |  |  | 2 | 4 | 8 | M | = | = |  |
|  | P5 Out fall Sample collectiong | Inhalation of aerosol | Biological, Chemical | W |  | Inhalation | Introduce PPE issuance/Conduc ting Awareness programs | observation | 3 | 2 | 8 | M | + | - |  |
|  |  | Exposure to treated sewage | Biological | W,G,C,I,P |  | Inadvertent ingestion Dermal contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 3 | 2 | 6 | M | + | - |  |
|  |  | Trips, Slips and Falls | Physical | W,C,P |  | Skin contact | Proper covering of manholes, barriers, and warning signs | observation | 4 | 8 | 32 | H | = | = |  |

|  | P5 Out fall Effluent reuse at plant for cleaning of machineries, vacuum trucks, sucker pump, jetting machine | Health risk due to exposure effluent during cleaning | Biological (Pathogen s, helminth) Chemical (heavy metal), Physical | W |  | Inhalation Dermal contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 3 | 4 | 6 | L | = | = |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P5 Out fall Repairs and maintenance in dischage line | Exposure to effluent | Biological | W |  | Inadvertent ingestion Dermal contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 3 | 4 | 12 | M | = | = |  |
|  |  | Trips, Slips and Falls | Physical | W |  | Skin contact | Proper covering of manholes, barriers, and warning signs | observation | 4 | 8 | 32 | H | = | = |  |
|  | P5 Out fall Effluent reuse at SpTP for cleaning of machineries, vacuum trucks and dump truck | Exposure to effluent | Physical | W |  | Skin contact | Use of PPE, including gloves, boots and waterproof clothing | observation | 3 | 2 | 6 | L | = | = |  |

|  | P6 Sludge Management Belt press & drying bed operations | Health risk due to exposure sludge during loading, transporting and unloading and at dryingbed operations | Physical(o dor), Chemical, Biological | W |  | Dermal contact Inhalation Inadvertent ingestion | Use of PPE, including gloves, boots and waterproof clothing | observation | 3 | 8 | 32 | H | = | = |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Health risk due to exposure polymer during operating and maintenances | Chemical, Biological | w |  | Dermal contact Inhalation Inadvertent ingestion | Use of PPE, including gloves, boots and waterproof clothing | observation | 3 | 4 | 16 | H | = | = |  |
|  | P6 Sludge Management Usage of dried sludge by external parties | Health risk due to exposure to dewatered sludge or solid waste | Chemical (heavy metals), Biological (Pathogen s), Physical | W,U |  | Dermal contact Inadvertent ingestion | Use of PPE, giving awareness | observation | 3 | 4 | 12 | M | = | = | = |
|  |  | Consumption of contaminated crops | Biological | P,U |  | Ingestion | Awareness programmes | observation | 2 | 4 | 12 | L | = | = |  |

# DEVELOP AND IMPLEMENT AN INCREMENTAL IMPROVEMENT PLAN

  - : Consider options to control identified risks

| Step of the sanitation service chain: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of the hazardous event: Exposure to raw sewage |  |  |  |  |  |
| Exposure group: |  |  |  |  |  |
| Improvement options |  |  |  |  |  |
| Option of new or modified control measures for this hazardous event | What is the likely effectiveness of this control measure option? | What is the level of resources required? | To what extent will this control measure be effective under the most likely climate change scenarios? | Comments/ | Priority for improvement plan |
|  | (High, medium, low) | (Including financial, human resources, political support: high, medium, low) | (Effective, ineffective, detrimental) | discussion | (Immediate, short term, medium term, long term) |
| Purchasing Chest wadders and full body safety kits | High | Medium | Effective |  | Immediate |
| Step of the sanitation service chain: |  |  |  |  |  |
| Description of the hazardous event: Falls into Open Manholes, confine space |  |  |  |  |  |
| Exposure group: |  |  |  |  |  |
| Improvement options |  |  |  |  |  |
| Option of new or modified control measures for this hazardous event | What is the likely effectiveness of this control measure option? | What is the level of resources required? | To what extent will this control measure be effective under the most likely climate change scenarios? | Comments/ | Priority for improvement plan |
|  | (High, medium, low) | (Including financial, human resources, political support: high, medium, low) | (Effective, ineffective, detrimental) | discussion | (Immediate, short term, medium term, long term) |
| Purchasing of safety harnesses & confine space entry tripod | High | Medium | Effective |  | Immediate |

| Step of the sanitation service chain: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of the hazardous event: Traffic Hazards during repairs and PPMs |  |  |  |  |  |
| Exposure group: |  |  |  |  |  |
| Improvement options |  |  |  |  |  |
| Option of new or modified control measures for this hazardous event | What is the likely effectiveness of this control measure option? | What is the level of resources required? | To what extent will this control measure be effective under the most likely climate change scenarios? | Comments/ | Priority for improvement plan |
|  | (High, medium, low) | (Including financial, human resources, political support: high, medium, low) | (Effective, ineffective, detrimental) | discussion | (Immediate, short term, medium term, long term) |
| Purchasing of night lights, cones and sign boards | High | Medium | Effective |  | Immediate |
| Step of the sanitation service chain: |  |  |  |  |  |
| Description of the hazardous event: Collapse of Sewer Structures |  |  |  |  |  |
| Exposure group: |  |  |  |  |  |
| Improvement options |  |  |  |  |  |
| Option of new or modified control measures for this hazardous event | What is the likely effectiveness of this control measure option? | What is the level of resources required? | To what extent will this control measure be effective under the most likely climate change scenarios? | Comments/ | Priority for improvement plan |
|  | (High, medium, low) | (Including financial, human resources, political support: high, medium, low) | (Effective, ineffective, detrimental) | discussion | (Immediate, short term, medium term, long term) |
| Conduct structural integrity assessments, reinforce vulnerable structures,Implement regular inspection and maintenance | High | High | Effective |  | Long term |

| Step of the sanitation service chain: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| escription of the hazardous event: Risk of exceeding plant capacity due to Increased Intensity and Frequency of Storm Events |  |  |  |  |  |
| Exposure group: |  |  |  |  |  |
| Improvement options |  |  |  |  |  |
| Option of new or modified control measures for this hazardous event | What is the likely effectiveness of this control measure option? | What is the level of resources required? | To what extent will this control measure be effective under the most likely climate change scenarios? | Comments/ | Priority for improvement plan |
|  | (High, medium, low) | (Including financial, human resources, political support: high, medium, low) | (Effective, ineffective, detrimental) | discussion | (Immediate, short term, medium term, long term) |
| Operating the second train and commencing stage 2 of the plant, Implement regular inspection of sewer lines for infiltration | High | High | Effective |  | Long term |
| Step of the sanitation service chain: |  |  |  |  |  |
| escription of the hazardous event: Inhalation of toxic Gases/ Odour |  |  |  |  |  |
| Exposure group: |  |  |  |  |  |
| Improvement options |  |  |  |  |  |
| Option of new or modified control measures for this hazardous event | What is the likely effectiveness of this control measure option? | What is the level of resources required? | To what extent will this control measure be effective under the most likely climate change scenarios? | Comments/ | Priority for improvement plan |
|  | (High, medium, low) | (Including financial, human resources, political support: high, medium, low) | (Effective, ineffective, detrimental) | discussion | (Immediate, short term, medium term, long term) |
| Purchase gas detection systems, and respiratory protection equipment | High | High | Effective |  | Long term |

  - : Develop an incremental improvement plan

| Improvement measure | Cost | Source of funds | Lead organization | Year 1 |  |  |  |  |  |  |  |  |  |  |  | Year 2 |  |  |  | Year 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | Q1 | Q2 | Q3 | Q4 | Q1 | Q2 | Q3 | Q4 |
| Improvement measures to control Exposure to raw sewage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Purchasing Chest wadders | 60,000 | O&M | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Purchasing full body safety kits | 500,000 | O&M | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvement measures to control Falls into Open Manholes, confine space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| & confine space entry tripod | 50,000 | O&M | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Purchasing of confine space entry tripod | 500,000 | O&M | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvement measures to control Traffic Hazards during repairs and PPMs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Purchasing of night lights, cones and sign boards | 150,000 | O&M | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvement measures to control Collapse of Sewer Structures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conduct structural integrity assessments, reinforce vulnerable structures, Implement regular inspection, maintenance and repairs |  | RH | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvement measures to control Risk of exceeding plant capacity due to Increased Intensity and Frequency of Storm Events |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating the second train of WWTP |  | RH | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Commencing stage 2 of the plant |  | RH | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Implement regular inspection of sewer lines for infiltration |  |  | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvement measures to control isInhalation of toxic Gases/ Odour |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Purchase gas detection systems |  | RH | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Purchase respiratory protection equipment |  | RH | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 5: Monitor control measures and verify performance

### : Define and implement operational monitoring

| Prioritized hazardous event | Sanitation step | Choose one control measure that will have a detailed operational monitoring plan |
| --- | --- | --- |
| Exposure to raw sewage |  | Use of PPE, including gloves, boots and waterproof clothing |
| Falls into Open Manholes, confine space |  | Proper covering of manholes, barriers, and warning signs |
| Traffic Hazards during repairs and PPMs |  | Traffic control measures, reflective clothing, and proper signs,competetant drivers |
| Collapse of Sewer Structures |  | Regular inspections and proper maintenance |
| Risk of exceeding plant capacity due to Increased Intensity and Frequency of Storm Events |  | Infiltration Control |
| Inhalation of toxic Gases/ Odour |  | Confined space entry protocols, training,PPE, Gas detection systems, proper ventilation |

  - Operational monitoring plan for the chosen control measures

| Operational monitoring plan |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational monitoring plan for: | Control exposure to raw sewage |  |  |  |
| Operational limits | Operational monitoring of the control measure: |  | Corrective action when the operational limit is exceeded |  |
|  | Control measure: |  |  |  |
| 100% compliance to use of PPE, including gloves, boots and waterproof clothing | What is monitored? | Whether Safety practices are followed | What action is to be taken? | Provide training and awareness, impose panelties |
|  | How is it monitored? | Random Inspection |  |  |
|  | Where is it monitored? | onsite | Who takes the action? | Eng/OIC |
|  | Who monitors it? | Eng/OIC | When is it taken? | Within a month |
|  | When is it monitored? | Weekly | Who needs to be informed of the action? | Manager |
| Operational monitoring plan |  |  |  |  |
| Operational monitoring plan for: | Control Falls into Open Manholes, confine space |  |  |  |
| Operational limits | Operational monitoring of the control measure: |  | Corrective action when the operational limit is exceeded |  |
|  | Control measure: |  |  |  |
| 100% compliance to safety protocols and SOPs | What is monitored? | Whether Safety practices are followed | What action is to be taken? | Provide training and awareness, |
|  | How is it monitored? | Random Inspection |  |  |

|  |  |  |  | Address the root cause |
| --- | --- | --- | --- | --- |
|  | Where is it monitored? | onsite | Who takes the action? | Eng/OIC |
|  | Who monitors it? | Eng/OIC | When is it taken? | Within a month |
|  | When is it monitored? | Weekly | Who needs to be informed of the action? | Manager |
| Operational monitoring plan |  |  |  |  |
| Operational monitoring plan for: | Control Traffic Hazards during repairs and PPMs |  |  |  |
| Operational limits | Operational monitoring of the control measure: |  | Corrective action when the operational limit is exceeded |  |
|  | Control measure: |  |  |  |
| 100% compliance to safety protocols and SOPs | What is monitored? | Whether Safety practices are followed | What action is to be taken? | Provide training and awareness, Address the root cause |
|  | How is it monitored? | Random Inspection |  |  |
|  | Where is it monitored? | onsite | Who takes the action? | Eng/OIC |
|  | Who monitors it? | Eng/OIC | When is it taken? | Within a month |
|  | When is it monitored? | Weekly | Who needs to be informed of the action? | Manager |
| Operational monitoring plan |  |  |  |  |

| Operational monitoring plan for: | Collapse of Sewer Structures |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure: |  | Corrective action when the operational limit is exceeded |  |
|  | Control measure: |  |  |  |
|  | What is monitored? | Cracks, settlements | What action is to be taken? | Repair or srengthening the structures |
|  | How is it monitored? | Visual Inspections |  |  |
|  | Where is it monitored? | On site | Who takes the action? | Eng/OIC |
|  | Who monitors it? | OIC/Eng | When is it taken? | Within a month |
|  | When is it monitored? | Monthly | Who needs to be informed of the action? | Manager |
| Operational monitoring plan |  |  |  |  |
| Operational monitoring plan for: | Risk of exceeding plant capacity due to Increased Intensity and Frequency of Storm Events |  |  |  |
| Operational limits | Operational monitoring of the control measure: |  | Corrective action when the operational limit is exceeded |  |
|  | Control measure: |  |  |  |
|  | What is monitored? | Flow levels, Infiltration percentages | What action is to be taken? | Operate By- pass route |
|  | How is it monitored? | Flow Meter readings, |  |  |
|  | Where is it monitored? | SCARDA | Who takes the action? | Eng/OIC |
|  | Who monitors it? | Technicians,OIC, Eng | When is it taken? |  |

|  | When is it monitored? | Daily | Who needs to be informed of the action? | Manager |
| --- | --- | --- | --- | --- |
| Operational monitoring plan |  |  |  |  |
| Operational monitoring plan for: | Inhalation of toxic Gases/ Odour |  |  |  |
| Operational limits | Operational monitoring of the control measure: |  | Corrective action when the operational limit is exceeded |  |
|  | Control measure: |  |  |  |
| 100% compliance to safety protocols and SOPs | What is monitored? | Whether Safety practices are followed | What action is to be taken? | Provide training and awareness, Address the root cause |
|  | How is it monitored? | Random Inspection |  |  |
|  | Where is it monitored? | onsite | Who takes the action? | Eng/OIC |
|  | Who monitors it? | Eng/OIC | When is it taken? | Within a month |
|  | When is it monitored? | Weekly | Who needs to be informed of the action? | Manager |

6: DEVELOP SUPPORTING PROGRAMMES AND REVIEW PLANS

|  | Supporting program 1 | Supporting program 2 |
| --- | --- | --- |
| Title of the program | Awareness to NWSDB staff | Awareness to external Stakeholders |
| Objective of the program | Make NWSDB staff aware about the SSP and train them on the procedures to follow | Make external stakeholders aware about the importance of SSP to the community and involve them with the process. |
| Description of the program | Providing training and awareness on the safety procedures to follow and records to maintain. | Involvement of external parties to implement the SSP |
| Key partners to implement the program | SSP Team | SSP Team |