# SANITATION SAFETY PLANNING SSP IN RADDOLUGAMA AREA

### Prepared By:

Regional Manager - Water Reclamation Raddolugama Office

National Water Supply and Drainage Board

MAY 2024

Table of Contents

INTRODUCTION	2

- PREPARATION OF SANITATION SAFETY PLAN (SSP)	4

  - The SSP Area and the Lead Organization	4

  - The SSP Team	4

  - Establishment of SSP Priorities	17

- SYSTEM DESCRIPTION	20

  - System Map	Error! Bookmark not defined.

  - Characterize System Flows	30

  - Identify Exposure Groups	32

  - Supporting Information	38

  - Confirmation of System Descrfiption	Error! Bookmark not defined.

- HAZARDOUS EVENTS, ASSESSMENT OF EXISTING CONTROL MEASURES AND RISK EXPOSURE39

  - Identification of Hazards and Hazardous Events	Error! Bookmark not defined.

  - Identification and Assessment of Existing Control Measures	40

  - Assessment and Prioritization of Risk Exposure	47

  - ​

1

Abbreviation

| BOD | Biochemical Oxygen Demand |
| --- | --- |
| CEA | Central Environmental Authority |
| CMR | Compliance Monitoring Report |
| COD | Chemical Oxygen Demand |
| DA | Department of Agriculture |
| FOG | Fat, Oil, and Grease |
| LA | Local Authority |
| MOH | Ministry of Health |
| NWSDB/ NWS&DB | National Water Supply and Drainage Board |
| PHI | Public Health Instructor |
| PPE | Personal Protection Equipment |
| PS | Pump Station |
| RSC | Regional Support Center |
| SSP | Sanitation Safety Plan |
| SSC | Sewer Service Connection |
| STP | Sewage Treatment Plant |
| TP | Treatment Plant |
| TSS | Total Suspended Solids |
| UDA | Urban Development Authority |
| VT | Vacuum Truck |
| VTU | Vacuum Truck Unit |
| WR | Water Reclamation |
| WWTP | Wastewater Treatment Plant |
| WWTS | Wastewater Treatment System |

2

Definition of Terms

Biosolids – Organic materials resulting from the treatment of sewage and septage sludge; residue generated during the treatment of domestic sewage and septage in a treatment facility

NWS&DB – National Water Supply and Drainage Board

Desludging — The process of cleaning or removing the accumulated domestic sludge or septage.

Dewatering — This is the process of reducing the moisture content of sludge to lessen the volume and odor, e.g. vacuum, filter, centrifuge, belt filter press, filter press, sludge drying beds, and lagoons.

Disinfection — This is the process of destroying pathogenic organisms either by physical (e.g. application of heat) or chemical (e.g. chlorine application) means.

Effluent – A general term denoting any wastewater, partially or completely treated, or in its natural state, flowing out of a manufacturing plant, industrial plant, or treatment plant.

Exposure— Contact of a chemical, physical or biological agent with the outer boundary of an organism (e.g. through inhalation, ingestion, or dermal [skin] contact).

Hazard—A biological, chemical, or physical constituent that can cause harm to human health Helminth— Helminths are a broad range of organisms that include intestinal parasitic worms: trematodes (flatworms, also commonly known as flukes, e.g. Schistosoma), nematodes (roundworms, e.g. Ascaris, Trichuris and the human hookworms) or cestodes (tapeworms, e.g. Taenia solium, the "pork tapeworm")

Influent – Waste water flowing through the sewer lines/conveyance into a treatment plant

Pathogens – Disease-causing organisms (e.g. bacteria, helminths, protozoa, or viruses).

Risk – The likelihood and consequences that something with a negative impact will occur.

Septage - Also known as Domestic Sludge or the solid particle of domestic sewage, which settles at the bottom of the sedimentation tank and is digested by anaerobic bacteria, purely from domestic sources, exclusive of industrial and hazardous wastes.

Sewage - or also known as Domestic Sewage — wastewater composed of raw liquid and solid waste coming from residential and commercial uses, exclusive of industrial and hazardous waste.

Wastewater – Sewage and faecal sludge

1

## INTRODUCTION

In today’s increasing number of challenges brought about by the impacts of human diseases due to the scarce or improper use of sanitation facilities, communities must establish a system to address these challenges, mitigate and reduce the effects. Aside from the adequacy of sanitation facilities, situations such as inadequate containment, improper treatment and handling of excreta and wastewater need to be addressed as well as contribute to the problems. The National Water Supply and Drainage Board (NWS&DB) initiates the development of a Sanitation Safety Plan to cover the entire country. In line with this, Raddolugama Sanitation Safety Plan is developed to ensure the safety of personnel related to sanitation activities in the Katana, Minuwangoda, Gampaha divisional secretariats.

The coverage area includes mainly the Raddolugama housing scheme. In additionally fecal sluge treatment covers the Minuwangoda, Katana, Jaela, Seeduwa, and Gampaha DS divisions.

The main types of sanitation system in the are:

- Sanitation system 1 – Raddolugama Water Reclamation System

### Raddolugama Water Reclamation System

Raddolugama WWTS is a combined sewage and faecal sludge treatment facility, which receives sewage coming from the sewered area (National Housing Scheme-NHS) of Raddolugama, and faecal sludge from the surrounding unsewered areas. A total of 2,237 utilities are already connected to the sewer lines of the Raddolugama existing sewerage system and more than 10,000 residential population and 100 floating population are benefited. These sewers are connected to the wastewater reclamation plant through a system that contains 1 lifting station (2 km of pumping main) and 250 manholes spanning 30 km of sewers. Following figures show water reclamation center and coverage area by the sewer network which served by the treatment plant.

2

With this SSP, NWS&DB aims to identify all potential hazards and quantify the risk associated with exposure to the sewage and faecal sludge throughout the sanitation chain including exposure to the end-products produced by the treatment facilities, particularly the effluent and the dried bio-solids.

The coverage area includes Ja-ela, Katana, Minuwangoda, Gampaha DS divisions. This SSP was done for the above mentioned wastewater management system.

3

## PREPARATION FOR SANITATION SAFETY PLAN (SSP)

### The SSP Area and the Lead Organization SSP area

The area of the system consisted of the entire wastewater infrastructure managed by the

NWSDB, including household connections to the Raddolugama sewer networks, pumping stations, wastewater treatment plants (WWTPs), treatment and disposal of sludge, reuse and disposal of treated wastewater. Because, some houses are served with on-site systems (e.g. Septic tanks and soakage arrangements), the faecal sludge management system, where workers, users, and community exposes to the wastewater (Sewage and faecal sludge) during collection, transport, treatment and disposal, was also included. Fecal sluge treatment covers the Minuwangoda, Katana, Jaela, Seeduwa, and Gampaha DS divisions.

### Lead organization

National Water Supply and Drainage Board (NWSDB) is the SSP lead organization.

### The SSP Team

SSP team leader

Regional manager-Jaela/Ekala, water reclamation division of NWSDB was appointed as the SSP team leader, having many years of experience in developing sanitation projects, and has resources, knowledge and management skills to lead project implementation.

4

Table 01: SSP Supportive Team

|  | Name | Job Title | Role | Responsibility | Contact No |
| --- | --- | --- | --- | --- | --- |
| 01 | Eng. Danesh Gunathilake | Additional General Manager – Water reclamation (NWSDB) | Facilita tor | Assist to implement SSP by allocating financial and Human resources. Reporting the progress to the top management. Monitoring the functionality of the SSP and coordinate with top management of relevant stakeholder organizations. | 0773856724 |
| 02 | Eng. G.D.N. Nevile | Deputy General Manager – (WR) (NWSDB) | Facilitator | Assist to implement SSP by allocating financial and Human resources. Reporting the progress to the top management. Monitoring the functionality of the SSP and coordinate with top management of relevant stakeholder organizations. | 0772918634 |
| 03 | Eng. Jayawardana | Waterrreclamatio n Specialist (NWSDB) | Memb er | Guidance on water quality issues, and wastewater treatment. Advise to provide new improvements. | 0764503487 |
| 04 | Eng. S.N. Fernando | Assistant General Manager - GCWR (NWSDB) | Facilitator | Monitoring the functionality of the SSP and coordinate with top management of relevant stakeholder organizations. | 0772386182 |
| 05 | Eng. K. Kuruppu | Assistant General Manager WR (NWSDB) | Facilitator | Monitoring the functionality of the SSP and coordinate with top management of relevant stakeholder organizations. | 0773440686 |

5

### Table 02: The SSP Operation Team

| NAME/JOB | Section | TITLE | Expertice | ROLE IN SSP TEAM | CONTACT INFORMATION |
| --- | --- | --- | --- | --- | --- |
| C.J. Ranasinghe | WR ( Ja Ela/Ekala Region) | Regional Manager | Charted Engineer (Civil) | Team Leader | 777633321 |
| D.M.W.J.B.Dissanayake | WR ( Ja Ela/Ekala Region) | Engineer (Civil) | Engineer (Civil) | Facilitator | 764583668 |
| Eshani Jayathilake | Laboratory (Soysapura WRC) | Chemist (Moratuwa/R athmalana) | Water Quality | Water quality testing | 0771453791 |
| W. Athulathmudali | GCWR | Accountant (GCWR) | Financial | Financial Facilitator | 0718066289 |
| Amali Jayakodi | WR Section | Sociologist | SSP Moderator | Conducting meetings and awareness programmes | 0715220242 |
| A.D.J.R.Abeysekara | Raddoluga ma WRC | OIC (Raddolugama ) | Civil Engineering | Deputy Team Leader. O&M of WRC & coordination. | 777764226 |
| J.P.Koralage | O&M | EA (Civil) | Civil Engineering | O&M of WRC & coordination. | 714898653 |
| G.R.D.M.Iddamalgoda | WR ( Ja Ela/Ekala Region) | EA(Mech.) | Mechanicall Engineering | O&M of mechanical section | 716407880 |
| H.L.P.K.Gunawardana | O&M | EA (Electrical) | Electrical Engineering | O&M of electrical section | 773900557 |
| H.D.M.P ERERA | O&M | EA (Civil) | Civil Engineering | O&M of documentation & procumentation. | 0763570891 |
| H.K.S Randila | WR ( Ja Ela/Ekala Region) | Lab Assistant | Sample Testing | Sample testing and quality contral | 0776799245 |
| T.G.C.M. Perera | O&M | Electrician | Electrical O&M | O&M work (Elactrical) | 0751517412 |
| A.A. Ranil Jayantha | O&M | Pump Operator | Pump Operation | Plant operation | 0769039817 |
| K.D.R. Perera | O&M | Pump Operator (Covering) | Pump Operation | Plant operation | 0770535619 |
| W.M.C. Jayashantha | O&M | Pump Operator (Covering) | Pump Operation | Plant operation | 0754785193 |
| W.H.P.M. Jayasekara | O&M | Labour | O&M work | Assisting for Maintainance Work | 0723233017 |
| M.D.A. Fernando | O&M | Labour | O&M work | Assisting for Maintainance Work | 0761100270 |

6

| S.T.W. Samarathunga | O&M | Driver | Driving | Driving vehicles | 0718001828 |
| --- | --- | --- | --- | --- | --- |
| A.N.Vipun | O&M | Labour | Documentat ion | Data entry | 0701687834 |
| A.D.S. Fernando | O&M | Labour | O&M work | Assisting for Maintainance Work | 0722056668 |
| J.A.C.S. Jayakodi | O&M | Labour | O&M work | Assisting for Maintainance Work | 0771555803 |
| A.D.M. Dharmathilake | O&M | Labour | O&M work | Assisting for Maintainance Work | 0781046816 |
| M.S. Chandrarathna | O&M | Labour | O&M work | Assisting for Maintainance Work | 0767519027 |

7

### Table 03: The SSP Stakeholder Team

| SANITATION STEP | SSP TEAM MEMBERS AND REPRESENTATION |
| --- | --- |
| Toilet and Containment – (Storage/treatment) | PHI - Seeduwa-Katunayeke Municipal Councils Work Supersisor - Minuwangoda Urban Councils Secretary - Ja-Ela Pradeshiya sabha Technical Assistant - Katana Pradeshiya sabha |
|  | Katana Area officer - Central Environmental Authority |
|  | Katana Divisional Secretory – Katana Divisional Secratariet |
|  | Seeduwa Medical officer of health – Seeduwa(MOH) officer |
|  | Public health investigator (PHI) – Seeduwa(MOH) officer Public health investigator (PHI) – Raddolugama(MOH) officer Public health investigator (PHI) – Minuwangoda(MOH) officer Public health investigator (PHI) – Ja Ela(MOH) officer Public health investigator (PHI) – Katana(MOH) officer |
|  | Chairman - Welfaire Oganizations in Raddolugama Housing Scheme |
| Conveyance (Emptying and transport of sewage and/or faecal sludge) | SSP team leader – NWSDB O&M Engineer – NWSDB OIC (Raddolugama-WRC) – NWSDB EA (Civil) (Raddolugama-WRC) – NWSDB EA (M&E), (Raddolugama-WRC) – NWSDB EA (Electrical) (Ja Ela/Ekala WRC) – NWSDB |
|  | Fecal Sludge suplier - AB Hydrotech - Dinuth cey food (pvt) Ltd - Yehara Enterprises - Hettiarachchi Cleaners - Mahela Cleaners - Didula Cleaners - Rusiru Enterprises - Envirotech System Service - Sena Enterprises - Limo Logistic - Golden Bird Confectionary - St Anthony's Auto Service - Eco Service - Super Suck Gully Cleaners - Chaminda Enterprises - Crown Investment - D W Transport Service - Amashan Enterprises - DP Waste Management - H&M Western (pvt) Ltd |

8

|  | Authority for traffic law enforcement and licences |
| --- | --- |
| Treatment and disposal | SSP team leader – NWSDB O&M Engineer – NWSDB OIC (Raddolugama-WRC) – NWSDB EA (Civil) (Raddolugama-WRC) – NWSDB EA (M&E) (Raddolugama-WRC) – NWSDB EA (Electrical) (Ja Ela/Ekala WRC) – NWSDB Lab Assistant |
|  | Central Environmental Authority (CEA) |
| Reuse | Water reclamation center (WRC- Raddolugama) |
|  | Irrigation department |
|  | Department of agriculture |

9

### Stakeholder analysis

Because of the size and complexity of the sanitation systems in Raddolugama, the SSP core team decided to conduct a stakeholder analysis to ensure that all relevant stakeholders were engaged. After a brainstorming session, a list of stakeholders representing the entire sanitation service chain identified and analysed.

### Table 04: Stakeholder Analysis

| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influence/Power | Partici tion Requir |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Entire sanitation | NWSDB | Direct control and oversight of | SSP will help the mto identify the right sanitation | Too much effort and insufficient resources | High | High | Empo ment |
| chain |  | implementation of | interventions. |  |  |  |  |
|  |  | national guidelines and |  |  |  |  |  |
|  |  | standard sateach step of |  |  |  |  |  |
|  |  | the chain. |  |  |  |  |  |
| Entire sanitation chain | District/ Divisional Secretariat | Influence–coordinate with other government organization to ensure | Providing safe sanitation services is a requirement for healthy communities. | Low knowledge about their role in the sanitation sector. | Low | High | Collab tion |
|  |  | that sanitation systems |  |  |  |  |  |
|  |  | do not pose public |  |  |  |  |  |
|  |  | healthrisks.` |  |  |  |  |  |
| Entires anitation chain | Public health official MOH, PHI | Ensuring the public health and providing awareness about the sanitation. | They have been developing studies on water borne diseases. | The scope of their job role is wider. As a result, they have limited time to involve in sanitation sector. | High | Low | Consu ion |

| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influence/Po wer | Partici tion Requir |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Entire sanitation chain | Secretary of Local authority (PS, UC,MC) | Direct control–Support from the legislative branch of the local | They have the legislative power to address the sanitation safety within their | Lack of experience and interest in sanitation. | High | High | Collab tion |
|  |  | government. | boundaries. |  |  |  |  |
| Entire sanitation chain | Chairman of PS | Alocation of funds from local government. | Commitment to the government to improve their cities in every aspect. | Allocation of lesser funds for sanitation improvements and preferring government funds to be allocated to other sectors. | Low | High | Collab tion |
| Entire sanitation chain | Waste and wastewater service workers | Influence – representatives of formal and informal sanitation | Workers along the chain will be responsible for implementing proposed | Limited representation of workers | High | Medium | Collab tion |
|  |  | workers advocating for | system improvements and |  |  |  |  |
|  |  | fair working conditions | monitoring, and can evaluate |  |  |  |  |
|  |  | and pay. | the practicality and safety for |  |  |  |  |
|  |  |  | workers in the system. |  |  |  |  |

| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influence/Power | Partici tion Requir |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sewered system (flush toilet with sewerage and off-site wastewater treatment) |  |  |  |  |  |  |  |
| Toilet | Residents of Raddolugama housing scheme. | Functioning the waste water system in their premises correctly. |  |  | High | Low | Collaborati |
| Conveyance and treatment and disposal | O&M Team Raddolugama, WRC | Operate and maintain the wastewater conveyance, treatment and disposal system. | Lead organization in sanitation safety plan implementation. | Rising the cost of spare parts. | High | High | Informatio |
| Conveyance | RDA/PRDA | Approving and providing estimates for road damages due to excavations for leak repires and new connections. Reinstatement of roads. |  | Resistance to approve the road damagers. | High | High | Collaborati |
| Enduse/ disposal | CEA | Enforcing laws and regulations related to treated effluent discharge and sludge disposal | Possible improvements to sludge management process. | Resources and capacities to monitor dried sludge quality are lacking. | High | High | Collaborati |
| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influenc e/ Power | Participati Required |

| Enduse | Department of Agriculture | Develop official guidelines and quality standards for agricultural products,and conduct training and projects with farmers. | Use of treated wastewater and dried sludge for, agricultural purposes. | Resources and staff are lacking. | High | Low | Collaborati |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Enduse | Farmers | Use treated wastewater which is discharged to the Dandugam Oya. | Using dried sludge for cultivations as a fertilizer. | Less of knowladge on sanitation. | High | Low | Consultatio |
| On-site system (flush toilets with septic tanks and effluent infiltration, and off-site faecal sludge disposal) |  |  |  |  |  |  |  |
| Toilet, containment –storage/tre atment | House Owners | In charge of maintaining their own on-site systems and should cover the costs of refurbishment and upgrading. | They would be interested in increasing property values. | They are concerned about the costs of repairing and refurbishing their on-site system. | Low | Low |  |
| Conveyance | Vacuum tanker operators –private and public providers | Emptying cesspools and septic tanks. | They are motivated by legalization and recognition. | Lack of competent workers. Some discharging points are faraway from their collection sites. Increace in the tranpotation cost. | High | High | Consultatio |
| Sanitation step | Stake holder | Role of stake holder | Motivating factors | Constraing Factors | Importance | Influenc e/ Power | Participati Required |

| Disposal | CEA | have a surveillance team that monitors illegal waste discharges to the environment. However, they have not given enough focus on vacuum tankers. | SSP supports their overall aim of providing leadership in environmental protection. | This could represent new tasks that have not been included instrategic planning. | High | High | Empowerm nt |
| --- | --- | --- | --- | --- | --- | --- | --- |

Establishment of steering committee

Following the results of the stakeholder analysis, the SSP core team decided to invite the following people to form the SSP steering committee:

- District Secretariat – Gampaha

- Divisional Secretaries– Ja-Ela/Minuwangoda/Katana/Gampaha

- Members	of	Local	Authorities	(MCs/UCs/PSs)	Chairman/Secretary (Ja-Ela/Minuwangoda/Katana/Gampaha/Seeduwa)

- Head of Central Environment Authoruty Gampaha

- Resident Engineer – Road Development Authority Gampaha

- Resident Engineer – Provintial Road Development Authority Negombo

- Head of Water Reclamation Section of NWSDB

- Head of UDA – Gampaha

- OIC - Envoronmental Police Ja E

- Head of Regional Health Department (MOH) – Gampaha

- Head of Agricultural Department - Gampaha

- SSP Team Leader in NWSDB

### Establishment of SSP Priorities

It was not feasible to develop SSP for all sanitation systems in Raddolugama, Minuwangoda, Gampaha, etc. because of limited resources. The SSP core team hosted a kick-off meeting with members of the SSP steering committee. The main objective of the meeting was to decide which sanitation systems were to be prioritized by SSP and which target systems are likely to pose the greatest health risks.

Excreta flow diagrams (SFDs) are a simple and effective way of visualizing the service types in a city and the fate of different excreta streams. Green arrows represent the proportions of excreta that are “safely managed” along the sanitation chain. Red arrows show where the excreta flows are not safely managed.

Members of the steering committee decided to consider following priority areas in SSP;

- Sewered Areas in Raddolugama WRS

- Unsewered Areas – Serviced by Faecal Sludge Management (Those who are having only onsite sanitation facilities)

### Sewered areas

In Raddolugama sewerage scheme, almost all of the household units are connected to the sewer network.

There are also instances, interception chambers from households connected to the sewers or manholes overflow due to blockages. A sewerage system which is properly

operating and maintaining, could capture domestic wastewater from sources and direct them to a wastewater treatment facility. Eventually, treated effluent is reused, and	excess discharges to a water body according to CEA discharge standards.

### Unsewered areas

The most unsewered areas within and outside the limits of the wastewater treatment plants of Raddolugama, sewage is handled by onsite sanitation systems such as septic tanks and soakage pits. Houses with septic tanks and soakage arrangements located in these areas are regularly served by Raddolugama water reclamation center through a NWSDB own vacuum truck or 3rd party vacuum trucks (Such as pradeshiya sabha or private party).

Water reclamation centers utilizes vacuum truck units (VTUs) to collect faecal sludge from customers in unsewered areas. The VTUs have to service selected areas each day

to    ensure septic tanks of all customers are covered once in every 3 - 5 years.

There are also instances, when customers request immediate desludging (e.g., overflowing septic tanks) which are classified as unplanned activities, water reclamation centers will cater to such requests having spare capacity. Such desludging requests are	handled	by either NWSDB or 3rd party vacuum trucks.

Customers have to support the desludging activities. Once, VTUs have reached the premises which requires desludging, the personnel prepare the hose to be used, opens the septic tank cover slab or MH cover and proceed the desludging work. After	desludging work finished, VTU personnel will cover the septic tank and customer has to	seal it. Then the VTU and personnel will move and attend to another desludging work	until  the  VTU  reach  to  its

maximum capacity. Finally, the VTU will depart to the	wastewater	treatment	plant	for emptying.

## SYSTEM DESCRIPTION

The regional manager, water reclamation section of Ja-Ela/Ekala of NWS&DB handles the provision of sewer service connections, maintenance and repair of the sewer network, handling of customer complaints, operation and maintenance of the wastewater facilities (Sewage and faecal sludge), and planning for sewerage coverage expansion. For the sanitation services, its main tasks include the provision of sewage and faecal sludge management, handling customer complaints, operation and maintenance of the wastewater facilities, and planning for sanitation coverage expansion in Raddolugama water reclamation system.

Sewered areas are provided with sewerage services that include the operation, and maintenance of sewage treatment facilities (Raddolugama), conveyance systems, the sewer network installation , and new sewer service connections. On the other hand, areas that cannot be connected to the sewers, but have septic tanks, are provided with faecal sludge management services that include desludging and treatment of septic tank wastewater on request, as well as coordination works with the customers.

### System Map

Raddolugama water reclamation system

The system starts with households where wastewater is generated (Domestic wastewater). Households are connected to the sewers. In addition to households, wastewater from commercial customers is accommodated as long as their wastewater is domestic in nature.

#### Water reclamation center

The water reclamation system of Raddolugama consists with a sewage treatment facility (Raddolugama WRC) and Raddolugama WRC is a combination of preliminary treatment, secondary treatment, and disinfection. Wastewater is accepted by the WRC for treatment and safely disposal to comply with National Environmental standards. The WRC of Raddolugama treats wastewater from the sewered area of Raddolugama NHS. The wastewater treatment plant at the center removes pollutants from the influent wastewater. These techniques and technologies include physical, chemical, and biological processes and procedures.

#### P1: Sewage from households, and commercial establishments

The system starts with households where wastewater is generated. At present around 2046 households are connected to the sewer network. In addition, wastewater from 95 commercial, 60 government quarters, 16 board quarters, and 20 other establishments are accommodated as long as their wastewater meets the discharge standards to	public sewer. This is verified by the Water Reclamation Center Raddolugama through field investigations.

#### T1: Sewer network and pumping station

The wastewater generated by consumers, flows under gravity to sewer manholes and eventually collected at a Pumping Station (PS). The sewer network comprises with around 30km of gravity sewers, and 2km of pumping sewers, which lies in Raddolugama National Housing Scheme (NHS). Further, the system comprises with 250 nos. of manholes. This sewer network enables the transportation of sewage to the treatment plant via 1 lifting (Pumping) station. The pump station consist of	underground sump, a pump house, a generator room.

Manually cleaned coarse screen is available at the inlet collecting well. The screen have free slot of 75 mm between the bars. The screen remove floating particles greater than 75mm to prevent clogging of the pumps. The screenings are collected in a bucket-screen and subsequently lifted up for further treatment at WRC. The influent is collected in underground sump. The raw water levels are monitored using sensors and reported in real-time to the WRC.

The sewage flows through sewers to pumping station (PS-1) under gravity and Pump stations 1 (PS-1) is located besides temple road.

#### P2: Toilet and containment – storage/treatment with septic tanks and soakage pits

At present Water Reclamation Center accepts fecal sludge from Katana and surrounding local authorities areas such as Seeduwa and Minuwangoda.

#### T2: Conveyance of fecal sludge by vacuum trucks

The fecal sludge from household and commercial utilities are transported to WRC through VTU owned by NWSDB or 3rd party (Private or local authority own) VTUs upon request by the households.

#### P3: Pre-treatment of wastewater

The wastewater collected from the network is subjected to pretreatment. The pre-treatment system consists of a screening bucket in the main collecting well.

Collecting well (Lifting station):

Wastewater from PS-1 directly pumps sewage to collecting well in WRC. From the collecting well, wastewater pumps to the grit channel. Collecting well is 2.5m diameter and	6.3m deep. Three submersible pumps and screening bucket are installed inside the	collecting well.

#### T3: Pumping sewer to treatment plant

sewer collected to the collecting well is pumped to the treatment plant by using pumps located in the collecting well.

#### P3’: Pre-treatment for faecal sludge

There is a bar rack to remove coarse particles (screenings) from both sewer and faecal sludge prior to the grid channel at the WRC. After the screening of faecal sludge, liquid portion with sewage (Wastewater) is passed to the grid channel. The screen remove rags, paper and other large particles to prevent clogging in other equipments.

Bar rack

The bar rack (screen) made with alluminium alloy at the inlet of grit channel at 45 degrees to remove particles which are gearter than 25mm. The screenings and other solid wastes are washed, and handed over from the faecal sludge and sewage treatment plant to the local authority.

Grit channel

Grid channel is Design to remove sand, gravel, and other dense mineral particles, which are settled in the grit channel, when wastewater flows along the grit channel. Grit channel is 7.8m in length and 0.4m wide.

#### T4: Wastewater send to the aeration tank.

The screened waste water is sended to the biological treatment process by gravity from the grit channel.

#### P4: Secondary treatment: Biological treatment

Secondary treatment includes biological treatment and clarification of sludge. The biological treatment is carried out according to the activated sludge method. The concept is that the living conditions for the microorganisms normally present as decayers  in the wastewater are optimized. The biological degradation that would take	place in the recipient and consume oxygen in the water is now completed in the	treatment plant, thus preserving the environment of the recipient. At the Raddolugama wastewater treatment plant the biological treatment consists of aeration zone.

Aerobic zone

Under aerobic conditions the water contains dissolved oxygen. At a wastewater treatment plant this condition is created by blowing air into the water. The micro-organisms use oxygen and release carbon dioxide and water. For the growth, the micro-organisms use organic matter as an energy source, but they also need nutrients. Air is distributed to the	wastewater with two surface aerators fixed underneath of the beam above the aeration basin. The aeration system is the single biggest energy consuming system at the wastewater treatment plant and its efficiency is of greatest

Importance to minimize the energy consumption. Aeration tank volume is around 592m3.

Settling tank (Clarifier)

In the sedimentation basins, the activated sludge flocs are separated from the treated wastewater by gravity forces. The sludge is lifted by siphon action to a longitudinal sludge channel. From a small pump pit, the sludge is lifted by two frequency controlled pumps and is led in two separate pipes to the beginning of the stabilization tank.

Stabilization tank

Excess sludge is drawn out from the settling tank and conveyed to the stabilization tank. Sludge volume index (SV30) shall be between 40%-90% and the volume of the tank is 296m3.

#### T5: Conveyance of partially treated water

The surfaced water in the settling tank separated and sent for chlorination by gravity.

#### P5: Disinfection: Chlorination

Clarified effluent flows through a chlorine contact tank with baffle walls, to achieve the effluent requirements regarding fecal bacteria. The biologically treated effluent is disinfected by dosing calcium hypochlorite at the inlet of this tank. The chlorination dosage is controlled manually. Around 2 kg of calcium hypochlorite with the strength of 30% is used per day. Chlorine contact time is around 15 minutes.

#### T6: Conveyance of effluent up to the River

Treated wastewater is conveyed through a storm water drainage system to discharge to the D outfall under gravity according to the central environmental authority regulations. P6: Discharge of effluent to stream

The outfall of the facility is located at a river named as Dandugam Oya. Some part of the treated effluent is used to water the gardening area of water reclamation premises. Excess treated effluent is discharged to Dandugam Oya through the storm water drainage system.

#### P6’: Reuse of effluent

Some part of the treated effluent is used to water the gardening area of water reclamation premises.

#### T7: Conveyance of sludge

When the sludge concentration in the stabilization tank gets higher than the 90%, the sludge in the stabilization tank is directed to sludge drying beds.

#### P7: Sludge treatment: Sludge drying beds

Bottom of the sludge driying bed is consists with 20cm thick gravel layer (Made with 15mm-25mm gravel particles) and top of the drying bed is made up with 10cm thick sand layer (with 0.5mm-1.5mm sand).

#### T8: Collecting and storing dried sludge

When sludge is dried well, they are collected to the wheel borrows manually and dumped in the dried sludge storage area within the treatment plant premises.

#### P8: Usage of dried sludge (bio-solids) as a soil conditioner

All the dried sludge (bio-solids) is taken by the 3rdparty, whom used dried sludge as soil conditioner in coconut cultivated land or used to produce compost.

#### P9: Grit collection

Deposited grit in the grit channel is collected to a wheelbarrow and transported to the side of the WRC land. washed and screened grit will be used for landfilling and backfilling work of repairing water and wastewater pipe laying trenches.

#### T9: Grit conveyance

Collected grit/sand are transported to the grit storage area by the wheel barrow.

#### P10: Grit and dried sludge storage area

Grit, and dried sludge (biosolids) from the solar drying beds are transported to the land behind the solar drying beds (Within the water reclamation center premises).

#### T10: Solid particles collecting

The solid particles retain in the fines screen and collecting well are collected.

#### P11: wash the solid particles and keep to dry

The screenings are washedand dryed to handed over to local authority.

#### T11: Transpotation of Solid waste

Cleaned solid wastes are handed over to the local authority. Later on, they transported for land fillings.

#### P12: Disposal of polythine and plastic

Dried polythene and plastic are used for landfilling by local authorities.

SS1=Receipt of sewage from households, commercial and industrial establishments SS2= Conveyance of WW to WWTP via sewer network and pumping stations SFS1=Faecal sludge collected in septic tanks and soak pits

SLF1=Liquid fraction that percolates from the pits

SSWF1=Solid waste fraction that screened out from faecal sludge, and then keep to dry. SSWF2=Solid waste fraction that screened out while going through the fine screen.

SSF1=Sludge fraction separated from treated wastewater and collected in bin .

SSF2=Sludge fraction collected into dump truck from collection bin and transported to solar drying beds.

SWW1= Secondary treated wastewater.

SWW2= Disinfected wastewater send to the Dandugam Oya and dilute with river water. SWW3= Disinfected wastewater used for watering for gardening and flushing purposes. SGF1=Collected grit fraction transported, from grit container to grit storage area.

SSWF3=Dried screening fraction from solar drying beds, transported to grit and dried sludge storage area and eventually incinerated.

SSF3=Sludge fraction (sludge cake) dumped in and spread for treatment in solar drying beds.

SSF4=Sludge treated in solar drying beds transported to sludge storage area using dump trucks

SSFGF=Sludge fraction and grit fraction dumped and handled by spreading in storage area behind solar drying beds

SSF5=Dried Sludge fraction taken out by 3rd party for usage in coconut cultivated lands as a soil conditioner

### Characterize System Flows

Based on the information available, the SSP team used tool 2.1 to characterize the system flows and to collect key quantitative information, and information on the microbiological, physical and chemical hazards.

### Table 2.1 – Characterization of system flow

| Sanitation Step | Description of the System Flow | Key Information of | Expected Variations (Seasonal | Type of |
| --- | --- | --- | --- | --- |
|  | (Excreta-related flow such as | the System Flow | variations or unusual events, such | Potential |
|  | ww or sludge, and other waste | (Volume, flow, | as accidentally mixed | Hazard |
|  | streams when relevant to | concentration, etc.) | components or climate events) | (Biological, |
|  | sanitation system) |  |  | chemical, or |
|  |  |  |  | physical) |
| P1: Toilet and IC of houses, | SS1=Receipt of sewage from households, commercial | About 1500 m3 per day WW is collected. | The sewage could contain faeces and urine (Black water), bathing | Biological Chemical |
| commercial | establishments | BOD could reach 300 | water (Grey water), kitchen WW |  |
| establishments |  | mg/l | through grease traps. |  |
| T1: Sewer network and pumping | SS2=Transfer (Conveyance) of WW from WW generation | About 1500 m3 per day WW is collected. | Strength of WW is reduced due to dilution | Biological Physical |
| stations | stage to WWTP via sewer | BOD could reach 200 |  | Chemical |
|  | network and pumping stations | mg/l |  |  |
| P2: Toilet and containment-stora | SFS1=Faecal sludge collected in septic tanks and soak pits | About 100 m3 per day WW is collected. BOD | The sludge could contain anal cleansing materials, menstrual | Biological |
| ge/treatment with | [Faecal sludge-solids and water | could reach 600 mg/l | hygiene products, sharp objects | Physical |
| septic tanks and | that are collected in |  | and other foreign material. It may | Chemical |
| soak pits | underground tanks] |  | also contain chemicals present in |  |
|  |  |  | grey water.Depends on the soil |  |
|  |  |  | type/ ground water table/Climate |  |

| T2: Conveyance FS by vacuum trucks | SFS2=Fecal sludge emptied into vacuum trucks and transported to the WWTP | About 75 m3 of fecal sludge is received every day. | expected variations : High ph, visual observation grease and oil contamination | Chemical Biologica |
| --- | --- | --- | --- | --- |
| P3’ : Pre-treatment of faecal sludge at carse screen | SSWF1=Solid waste fraction is screened out by the bar screen while pumping out the fecal sludge. The screenings are kept to dry beside the pumping stations. | About 1 kg of solid waste is removed each time | The sludge could contain anal cleansing materials, menstrual hygiene products, sharp objects and other foreign material. Disposal of these materials is a big problem to the WRC. | Biological Chemical Physical |
| P3 and T1: Pre-treatment of wastewater | SSWF2=Solid waste fraction is screened out while going through the bucket screen. The screenings are kept to dry beside the pumping stations. | About 100g of solid waste is removed each time | Reduction of water content in washed screening waste | Biological Chemical |
| P4: Secondary treatment (Biological treatment and clarification) | SSF1=Sludge fraction separated from treated wastewater in the stabilization tanks. | No evidence on quantity | No expected variations. | Physical |
| T4: Conveyance of sludge fraction into solar drying beds. | SSF2=Sludge fraction is sent to the solar drying beds by gravity. | About 10m3 twice a week | During the southeast monsoon, drying of sludge fraction takes more time. | Biological Physical |
| P5: Disinfection | SWW1= Chemicals are added to Surface water extracted from settling tanks after the secondary treatment. | Around 3 kg of Bleaching powder is needed for a day | No expected variations. | Chemical |

| P6: Discharge of effluent (treated wastewater) to stream | SWW2= Disinfected wastewater (Disinfected using Bleaching Powder) pumped into Dandugama Oya and dilute with river water | About 1500 m3 per day WW is pumped. BOD of pumped ww is below 30 mg/l | Heavy rainfall leads to heavy dilution of wide range of constituents | Biological |
| --- | --- | --- | --- | --- |
| P6’: Reuse of effluent | SWW3= Disinfected wastewater used for watering for gardening and flushing purposes | About 0.5 m3 per day WW is used. | With heavy rainfall, there is no need to water the gardens. In dry seasons, more WW is required for watering of garden areas. | Biological |
| T3: Conveyance of grit by dump truck | SGF1=Collected grit fraction transported, from grit container to grit storage area | About 5 kg of grit is removed each time | No expected variations. | Biological |
| T4’: Conveyance of washed screenings | SSWF3=Dried screening fraction are collected and eventually incinerated. | About 1 kg of solid (Dried screenings) waste is collected and handed over to the local authority weekly. | With heavy rainfall, solid waste/incinerated residues end up in open drains/mix with surface or groundwater. | Biological Chemical |
| P7 : Sludge treatment | SSF3=Sludge fraction (sludge cake) pumped in and spread for treatment in solar drying beds. Water evaporates and the rest of the water is drained. | About 20m3 of sludge is dumped and spread each time | No expected variations. equipments | Biological |
| T5: Conveyance of dried sludge by dump truck | SSF4=Sludge fraction treated in solar drying beds transported to sludge storage area using dump trucks | About 850kg of sludge is transported each time | No expected variations. | Biological Physical |
| P8: Grit and dried sludge storage | SSFGF=Sludge fraction and grit fraction dumped and handled by spreading in storage area behind solar drying beds | About 5 kg of grit and 850kg of sludge is dumped and spread each time | With heavy rainfall, fecal matter could end up in open drains/mix with surface or groundwater. | Biological |

| P10: Reuse of biosolids | SSF5=Dried Sludge fraction taken out by 3rd party for usage in coconut cultivated lands as a soil conditioner | About 850kg removed from one dring bed. | With heavy rainfall, fecal matter could contaminate surface or groundwater. | Biological |
| --- | --- | --- | --- | --- |

### Identify Exposure Groups

As a next step, the SSP team identified the exposure groups and used tool 2.2 to identify who they are, how many there are, where they are and how exposure occurs. According to the 2018 WHO Guidelines on sanitation and health (WHO, 2018), the people most likely to be exposed to hazards during hazardous events at different steps of the sanitation service chain are as follows.

| Sanitation Step | Exposure Group | Who are the Exposure Groups? (Description of these people) | How many are there? (Actual numbers, if known; otherwise estimate) | What are they doing? (Circumstances under which they might be exposed to hazards in the system flow) | What are they exposed to? (Which system flows and which types of hazards they have contact with) | How often are they exposed to this? (Exposure frequency: daily, weekly, once a year, etc.) |
| --- | --- | --- | --- | --- | --- | --- |
| P1: Toilet and IC of | U1 | Users of flush | 675 utilities | ICs are outside the | They could have | Overflow and |
| houses, |  | toilets | [Including bulk | house. Children play | contact with | contamination |
| commercial, and |  | connected to | meter connection | and adults perform | wastewater during | could happen |
| industrial |  | sewer network | of Millennium | different activities in | overflows. | every 2 years, but |
| establishments |  | through ICs | City, which has | the vicinity. |  | is more frequent |
|  |  |  | 820 units’/ bulk |  |  | during heavy |
|  |  |  | meter connection of Nivasipura, |  |  | rains. Hookworms…?? |
|  |  |  | which has 1,200 |  |  | ? |
|  |  |  | units] |  |  |  |
| P2: Toilet and | U2 | Users of flush | 20,000 | Septic tanks and soak | They could have | Overflow and |
| containment-stora |  | toilets | households | pits are usually outside | contact with | contamination |
| ge/treatment with |  | connected to | (Around 60,000 | the house. Children | wastewater during | could happen |
| septic tanks and |  | septic tanks and | people). About | play and adults | overflows from ICs or | every 2 years, but |
| soak pits |  |  | 35% are children | perform different | toilets. | is more frequent |

|  |  | soak pits in their |  | activities in the vicinity |  | during heavy |
| --- | --- | --- | --- | --- | --- | --- |
|  | L1 | properties |  | of the tank. |  | rains. Hookworms…?? |
|  |  | Families living in |  |  |  | ? |
|  |  | areas where | 5,000 households |  |  |  |
|  |  | septic tank | (Around 15,000 | They usually have | Wells might |  |
|  |  | effluent and | people). About | shallow dug wells, | contaminate with ww | Seasonal, could |
|  |  | soak pits | 25% are children | rather than deep tube | especially where | be both during |
|  |  | infiltrate to the |  | wells | seasonal high ground | dry and rainy |
|  |  | ground water |  |  | water table is | periods |
|  |  |  |  |  | presence, and | depending on |
|  |  |  |  |  | distance to the well | situations |
|  |  |  |  |  | from unlined septic |  |
|  |  |  |  |  | tank or soakage pits is |  |
|  |  |  |  |  | small. They are |  |
|  |  |  |  |  | exposed to microbial |  |
|  |  |  |  |  | pathogens (Ex: E. |  |
|  |  |  |  |  | Coli). |  |
| T2: Conveyance FS by vacuum trucks | W1 | NWSDB or 3rd party (PS, UC,MC, or private party etc.) vacuum truck operators | NWSDB – 4 operators 2 trucks PS – 4 operators 2 trucks UC – 4 operators 2 trucks MC – 4 operators 2 trucks Private – 4 operators 2 trucks | They open the underground tanks (Manhole covers or cover slabs), insert the hose of vacuum truck and empty the lined or unlined septic tanks and the soak pits. | They are in direct contact with faecal sludge, which contains pathogens. Possible bursts due to bio-gas formation in sealed septic tanks, especially where vent pipes have not been provided. | Every day Rare |

| T1: Sewer network and pumping stations | W2 | WWTP Operators | 3Operators (Working of shift of 1 peeson) | They usually do not come into contact with ww, sludge, screening waste, or grit | They are not direct contact with ww, sludge, screening waste, or grit | Not applicable |
| --- | --- | --- | --- | --- | --- | --- |
| P2’: Pre-treatment of faecal sludge at WRC | W2 | WWTP Operators | 3 Operators (Working of shift of 1 peeson | They usually do not come into contact with ww, sludge, screening waste, or grit | They are not direct contact with ww, sludge, screening waste, or grit | Not applicable |
| P3: Pre-treatment of wastewater | W2 | WWTP Operators | 2Treatmnt plant cleaning labours | Cleaning of strainer and the bar screen | They might possible to contact with ww, sludge, screening waste, or grit | Daily |
| P4: Secondary treatment (Biological treatment and clarification) | W2 | WWTP Operators | 1Treatmnt plant cleaning labours | Cleaning of walls and channels | He might possible to contact with ww, sludge | Daily |
| T4: Conveyance of stabilized sludge by gravity | W2 | WWTP Operators | 1Treatmnt plant cleaning labours | Opening and closing the the sludgr valve | He might not possible to contact with stabilized sludge | Weekly |
| P5: Disinfection | W2 | WWTP Operators | 1Treatmnt plant cleaning labours | He usually do not come into contact with treated eflurnt | He might not possible to contact with treated eflurnt | Weekly |
| P6: Discharge of effluent (treated wastewater) to stream | L2 | People who consume fish (Fish caught from downstream of the treated effluent | 500 people | They eat fish without much care | Pathogens | Rare and Only in Rainy season |

|  |  | discharge location of Dandugam oya and lagoon |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| P6’: Reuse of effluent | W2 | Operators who use the treated effluents (ww) to gardening the WRC premises | 4 Operators (Working of shift of 2 people) | Patrolling within the WRC premise, to carry out daily routine works | They can exposed to aerosol which can contain pathogenic microorganisms | Daily |
| T3: Conveyance of grit by wheel barrow | W2 | WWTP Operators | 4 Operators (Working of shift of 2 people) | They usually do not come into contact with ww, sludge, screening waste, or grit | They are not direct contact with ww, sludge, screening waste, or grit… | Not applicable |
| T4’: Conveyance of stabilised sludge by geavity | W2 | WWTP Operators | 2Treatmnt plant cleaning labours | Observation and operation of valves | They are not direct contact with ww, | two or three days per week |
| P7: De watering and driying of Sludge ????? | W2 | N\A | N\A | N\A | N\A | N\A |
| T5: Conveyance of dried sludge | W2 | WWTP Operators | Treatmnt plant cleaning labours | Collecting dried sludge by shavels | sludge is not fully treated and can contain pathogenic microorganisms. They can expose to microorganisms. | one a week |
| P8: Grit and dried sludge storage | L3 | Families living in the surrounding area of the Raddolugama WRC premises | 200 households ( | They live surrounding areas of WRC and some houses are close by the surface water canal | Mosquito breeding in grit and sludge storage areas. Pathogens from partially treated sludge, grit, and toxic | Often During rainy season |

|  |  |  |  |  | substances from the incinerated screenings might contaminate surface water canal and ultimately ground water (Probable contamination of shallow dug wells) |  |
| --- | --- | --- | --- | --- | --- | --- |
| P9: Reuse of bio-solids | F C | Farmers/coconut cultivators Consumers of farm products | 10 No information | Usage of partially treated dried sludge as a bio solid/soil conditioner/or use to produce compost They eat the products without much care (Probably the leaves) | Occasional bad smell Pathogenic microorganisms Sharp objects in dried sludge Pathogens | Specially during the dry season Specially during the dry season |

U Sanitation system users: all people who use a toilet.

L Local community: people who live and/or work nearby (who are not necessarily users of the sanitation system) and may be exposed.

W Sanitation workers: all people – formally employed or informally engaged – responsible for maintaining, cleaning or operating (e.g. emptying) a toilet or equipment (e.g. pumps, vehicles) at any step of the sanitation service chain.

WC Wider community: the wider population (e.g. farmers, communities in lower-lying areas) who are exposed to sanitation end-use products (e.g. through recreation or flooding), use sanitation end-use products, or consume products (e.g. fish, crops) that are produced using sanitation end-use products, intentionally or unintentionally. Sanitation end-use products include compost, faecal sludge and wastewater.

Depending on the sanitation service chain to which SSP applies, it might be necessary to treat the following exposure groups separately because they are exposed to very specific hazardous events during the end-use step (e.g. reuse in agriculture or aquaculture, consumption of products).

F Farmers: people who use sanitation end-use products (e.g. untreated, partially treated or fully treated wastewater, biosolids, faecal sludge).

C Consumers: anyone who consumes or uses products (e.g. crops, fish, compost) that are produced using sanitation product

L44

L44

The letters U, L, W, WC, F and C are used as symbols to identify the exposure groups in maps and tables, facilitating the health risk assessment in the subsequent modules.

The broad exposure groups (U, F, C, etc.) can be refined and defined into subgroups to aid the detailed hazard risk assessment, as shown in tool 2.2. For instance, the exposure group “U: sanitation system users” can be divided into U1: users of pit latrines, U2: users of flush toilets with a septic tank, and U3: users of toilets connected to the sewer system. It is important to estimate the number of individuals in each subgroup, how they come into contact with system flows (e.g. wastewater, excreta) and the frequency of exposure.

Although some exposure groups, such as formal workers, are relatively easy to identify, others will be more difficult – for example, communities accessing nearby groundwater sources, seasonal and informal workers, and people living in informal settlements or immigrant populations. Demographics of the exposure groups, such as gender, age and potential social exclusion, should be noted. Climate change or climate variability may increase or decrease the frequency of exposure.

| Symbal | Description |
| --- | --- |
| U1 | All people who use a toilet in sewer system in Raddolugama NHS. |
| U2 | All people who use a toilet having onsite sanitation facilities. |
| U3 | All people who affected by flooding the onsite sanitation facility (septic tank and soackage pit). |
| W1 | All people who formally employed or informally engaged with cleaning or operating (e.g. emptying) a toilet or equipment (e.g. pumps, vehicles) at unsewered onsite toilets. |
| W2 | All people who employed in NWSDB and engaged in operation and maintainance work or equipment (e.g. pumps, vehicles) at any step of the sanitation service chain. |
| L1 | All the Local community who affected by contaminated groundwater due to infiltration from septic tanks and soak pits into shallow groundwater |
| L2 | All the Local community who affected by flooding the surrounding septic tanks and sockage pits. |

| L3 | All the Local community who affected by contaminated surface water or shallow ground water due to discharge of faecal sludge without treatment . |
| --- | --- |
| L4 | All the Local community who affected by ingesting faecal sludge due to spillage during transport. |
| L5 | All the local community who use the road where manhole over flowing occure. |
| L6 | All the local community who affected Contaminated the ground water withwaste water due to the leaks in the pipes of collecting network |
| L7 | All the Local community who use the water in the Dandugam Oya and lives down of the place where treated waste water and storm water are discharged. |

### Supporting Information

    - Regulatory Requirements Water quality standards

The National Environmental (Protection and Quality) regulations, No. 1 of 2008 published in the Gazette No. 1534/18 of February 1, 2008 gives the standards for discharge of wastewater or effluent into inland surface waters.

Table 2.3 – Supporting Information

| Information Sources | Summary of Key Observations |
| --- | --- |
| Standards and regulations |  |
| Central environmental authority (CEA) effluent dischange standards 2022 | BOD, COD, TSS, pH, TDS, E. coli limit, Faecal Coliform (FC), Helminth eggs, |
| Information Related to system Management and Performance |  |
| Demographics and land-use patterns | Population near Ekala industrial area, Population near low-lying flood-prone areas where sanitation conditions are poor, Population where there are only basic sanitaion available with onsite sanitation systems, |
| Changes relating to weather or other seasonal conditions | Demand for pit emptying also increases during heavy rainfall events, but flooding makes access to some areas difficult. Sewers overflow during severe rain events as a result of high flow and blockages caused by solid waste. |

## HAZARDOUS EVENTS, ASSESSMENT OF EXISTING CONTROL MEASURES AND RISK EXPOSURE

### Identification of Hazards and Hazardous Events

Identification of hazards in sanitation safety planning is a critical step in ensuring the safety and effectiveness of wastewater management systems. This process involves systematically recognizing potential sources of contamination and risks associated with wastewater collection, treatment, and disposal. Common hazards include microbial pathogens such as bacteria, viruses, and parasites, which can cause waterborne diseases if not properly managed. Chemical hazards, including heavy metals and toxic compounds, may originate from industrial discharges and household chemicals, posing risks to both human health and the environment. Physical hazards, such as debris and blockages in the sewage system, can lead to system failures and overflows. Additionally, infrastructural vulnerabilities, like aging pipelines and treatment facilities, can exacerbate these risks. Identifying these hazards requires a thorough assessment of the entire sanitation system. By pinpointing these hazards, sanitation safety planning can implement targeted control measures, enhancing the safety and reliability of the wastewater management system.

Table 3.1 .1: Risk definitions for semi-quantitative risk assessment

|  | Descriptor | Description |
| --- | --- | --- |
| Likelihood (L) |  |  |
| 1 | Very Unlikely | Has not happened in the past and it is highly improbable it will happen in the next 12 months (or another reasonable period). |
| 2 | Unlikely | Has not happened in the past but may occur in exceptional circumstances in the next 12 months (or another reasonable period). |
| 3 | Possible | May have happened in the past and/or may occur under regular circumstances in the next 12 months (or another reasonable period). |

| 4 | Likely | Has been observed in the past and/or is likely to occur in the next 12 months (or another reasonable period). |
| --- | --- | --- |
| 5 | Almost certain | Has often been observed in the past and/or will almost certainly occur in most circumstances in the next 12 months (or another reasonable period). |
| Severity (S) |  |  |
| 1 | Insignificant | Hazard or hazardous event resulting in no or negligible health effects compared with background levels. |
| 2 | Minor | Hazard or hazardous event potentially resulting in minor health effects (e.g. temporary symptoms of irritation, nausea, headache) |
| 4 | Moderate | Hazard or hazardous event potentially resulting in self-limiting health effects or minor illness (e.g. acute diarrhoea, vomiting, upper respiratory tract infection, minor trauma). |
| 8 | Major | Hazard or hazardous event potentially resulting in illness or injury (e.g. malaria, schistosomiasis, food-borne trematodiases, chronic diarrhoea, chronic respiratory problems, neurological disorders, bone fracture), and/or may lead to legal complaints and concern, and/or major regulatory noncompliance |
| 16 | Catastrophic | Hazard or hazardous event potentially resulting in serious illness or injury, or even loss of life (e.g. severe poisoning, loss of extremities, severe burns, drowning), and/or will lead to major investigation by regulator, with prosecution likely |

### Table3.1.2: Semi-quantitative risk assessment matrix

|  |  |  | Severity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Insignificant | Minor | Moderate | Major | Catastroph ic |
|  |  |  | 1 | 2 | 4 | 8 | 16 |
| L | Very Unlikely | 1 | 1 | 2 | 4 | 8 | 16 |
| i | Unlikely | 2 | 2 | 4 | 8 | 16 | 32 |
| k | Possible | 3 | 3 | 6 | 12 | 24 | 48 |
| e | Likely | 4 | 4 | 8 | 16 | 32 | 64 |
| li |  |  |  |  |  |  |  |
| h |  |  |  |  |  |  |  |
| o |  |  |  |  |  |  |  |
| o d | Almost certain | 5 | 5 | 10 | 20 | 40 | 80 |
| ( |  |  |  |  |  |  |  |
| L |  |  |  |  |  |  |  |
| ) |  |  |  |  |  |  |  |
| Risk score R = L × S |  |  | <6 | 6-12 |  | 13-32 | >32 |

| Risk level | Low risk | Medium Risk | High Risk | Very High Risk |
| --- | --- | --- | --- | --- |

### Identification and Assessment of Existing Control Measures

For each of the hazard and hazardous event identified. Existing control measures have been identified to mitigate the risk of such hazards and hazardous exposure events. The said control measures were then assessed on their effectiveness in reducing the risk of the exposure event.

| COMPONE NT | HAZARD IDENTIFICATION |  |  |  | EXISTING CONTROLS |  | RISK ASSESSMENT |  |  |  |  |  | COMMENTS JUSTIFYING RISK ASSESSMENT (Under current conditions, climate change scenarios, or effectiveness of the control) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Under current conditions L=Likelihood; S=Severity; R=Risk (H=High; M=Medium; VH=Very high) |  |  |  | Under the most likely climate change scenarios + means incresed risk - means decresed risk = means the same risk |  |  |
| Sanitatio n step | Hazardous event | Hazar d | Ex po su re gr ou p | Nu m be r of pe op le at ris k | Description | Validation | L | S | Sco re (Lx S) | R | D r o u g h t | More intens e precipi tation |  |
| P1: Toilet and IC of houses, commercial establishments. | Ingestion after contact with wastewater in overflowing toilets or Ics | All microbial pathogens | U1 | 1000 | Preventive maintaine in the sewage collecting network | Visual observation by field visit | 3 | 4 | 12 | MR | - | + | This worsens during heavy rains, possibility of ingestion is high. |

| T1: Conveyance of Sewer through the network and pumping stations | Ingestion after contact with excreta during the repaire work of blocking pipe lines and damaged pipe lines. | All microbial pathogen s | W1 | 10 | Wearing personal protective equipments during the work | Visual observation/ further improvements are required | 5 | 4 | 20 | HR | = | + | With the rain blockage will increace. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1: Conveyance of Sewer through the network and pumping stations | Waste water, over flow from the manhalls are directed to the Storm water system of the housing scheme and contaminate the Dandugam Oya. | All microbial pathogen s | L5 | Unkn own | Proper maitainance of serage network system and proper storm water management prevent the overflow of waste water. | Visual observation | 5 | 2 | 10 | MR | - | + | This worsens during heavy rains, due to increase in overflow. |
| T1: Conveyance of Sewer through the network and pumping stations | Contaminate the ground water withwaste water due to the leaks in the pipes of collecting network | All microbial pathogen s | L6 | No clear idea | Repairing leaks in the pipe lines in the collecting network | Visual Observations/Su rveys | 4 | 2 | 8 | MR | = | + | In rainy season ground water table comes up. Then contaimination is high. |
| T1: Cleaning of Primary lifting | Ingestion after contact with excreta during the cleaning | All microbial pathogen s | W2 | 6 | Following SOP and Wearing personal protective equipments | Monitoring at the feild | 5 | 2 | 10 | MR | = | = | no effect from climate change. |

|  | work in lifting well. |  |  |  | during the work |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1: Cleaning of collecting well | If the sand bags are loosen, the people who working in the collecting tank may submerge in the waste water during the cleaning work in the collecting well. | All microbial pathogen s | W2 | 2 | seal the inlet with sand bags. | Improvement is required to seel the inflow. | 3 | 16 | 48 | VH R | = | + | During heavy rains, likelihood is increased. |
| T1: Cleaning of Screening basket in collecting well. | Ingestion after contact with wastewater during the cleaning | All microbial pathogen s | W2 | 2 | Wearing personal protective equipments during the work | Visual observation | 5 | 2 | 10 | MR |  |  |  |
| P3: Cleaning of Grit Channel | Ingestion after contact with wastewater during the cleaning | All microbial pathogen s | W2 | 6 | Wearing personal protective equipments during the work | PPE are supplied by NWSDB by annualy and as per the reqirement | 5 | 2 | 10 | MR | = | = | no effect from climate change. |
| P4: Cleaning work in treatment plant. | Injury to the body, possible asphyxiation, caused by falling into tanks | All microbial pathogen s | W2 | 3 | Wearing personal protective equipments during the work | PPE are supplied by NWSDB by annualy and as per the reqirement. / further improvements are required | 2 | 8 | 16 | HR | = | + | with rain walls will be more slipery. |

| P4: Secondary Treatment Process in the Aeration. | Waste water is sprinkling and they will be ingested to the body. | All microbial pathogen s | W2 | 5 | Wearing personal protective equipments during the work | Visual observation | 5 | 2 | 10 | MR | = | = | no effect from climate change. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P4: Secondary Treatment Process in the settling tank. | Due to breakdown in sludge scraper treatment process will be disturbed and Water of Dandugam Oya is contaminated with partialy treated waste water. | All microbial pathogen s | L7 | Unkn own | Preventive maintainance and keeping standby gear Box and other neccesary equipments. | Visual observation nomally once a six month, weels should be rplaced. | 3 | 4 | 12 | MR | = | = | no effect from climate change. |
| P2: Toilet and containment-stor age/treatment with septic tanks, soak pits. | Ingestion after contact with excreta (faeces/urine) due to blocking/ overflowing | All microbial pathogens | U2 | 500 | Properly functioning flush toilets with water supply | Visual observation /Survey | 3 | 4 | 12 | MR | - | + | Periodicaly empting the pits |
| P2: Disposal of liquid fraction by infiltration | Ingestion of contaminated groundwater due to | Faecal pathogens | L1 | 1000 0 | Safe distance from wells would have to be considered. | Technical Officers In Pradeshiya sabha | 5 | 4 | 20 | HR | + | + | Likelihood of ground water use is expected to increase during |

|  | infiltration from septic tanks and soak pits into shallow groundwater |  |  |  |  |  |  |  |  |  |  |  | drought periods. Severity will increase with more and prolonged flooding due to climate change. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P2: Toilet and containment-stor age/treatment with septic tanks and soak pits | Ingestion after contact with wastewater during the flooding. | All microbial pathogen s | U3 | 500 | N/A | Field visits and interviews | 3 | 4 | 12 | MR | - | + | Due to shallow ground water table, poor drainage of storm water and rain intencity likelihood of flooding have been increaced. |
| P2: Toilet and containment-stor age/treatment with septic tanks and soak pits | Ingestion after contact with wastewater during the flooding. | All microbial pathogen s | L2 | 100 |  |  | 3 | 2 | 6 | LR | - | + | Due to shallow ground water table, poor drainage of storm water and rain intencity likelihood of flooding have been increaced. |
| P2: Disposal of faecal sludge without treatment or Deep Row Entrenchment | Ingestion after contact with faecal sludge discharged without treatment, contaminated surface water or shallow ground water | All microbial pathogen s | L3 | 1500 | Nil | NA | 3 | 4 | 12 | MR | = | + | Generally higher risk during heavy rains. |

| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with raw sewage during vacuum tanker operation | All microbial pathogen s | W1 | 50 | Wearing gloves during work | NA | 5 | 4 | 20 | HR | = | + | Wearing gloves, and handwashing with soap or detergent after emptying is not widely practised. In flooding conditions, the likelihood will increase. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with faecal sludge caused by spillage during transport. | All microbial pathogen s | L4 | 6 | Procedures to deal spillage | Observations/ Interviews | 3 | 2 | 6 | MR | = | = | No effect from climate change. |
| T2: Conveyance of FS by vacuum trucks | Ingestion after contact with faecal sludge while entering or falling into soak pits or septic tanks | All microbial pathogen s | W1 | 20 | Nil | NA | 1 | 8 | 8 | MR | = | + | Flooding can cause collapse of unline septic tanks, and soakage pits. And stability of the tank can be affected by flooding. |
| T2: Conveyance of FS by vacuum trucks | Injury to the body, possible asphyxiation, caused by entering or falling into soak pits or septic tanks | Injury to the body, including asphyxiat ion | W1 | 20 | Nil | NA | 1 | 8 | 8 | MR | = | = | Flooding can cause collapse of unline septic tanks, and soakage pits. And stability of the tank can be affected by flooding. |

### Assessment and Prioritization of Risk Exposure

The risks were assessed based on its likelihood and severity. The computed level of risk and likelihood equates to the ranking of risks. Where in the risk level was identified. Furthermore, the risk category which require additional control or improvements were listed and described here to highlight the actions to be taken to reduce the risks.

Table 3.2 Prioritized Hazardous events with very high risk

| Sanitation step | Hazardous event | Exposu re group | Numb er of peopl e at risk | Risk | Projection of changers in risk with climate change scenarios | Priority Given |
| --- | --- | --- | --- | --- | --- | --- |
| T1: Cleaning of collecting well | If the sand bags are loosen, waste water will enter in to the collecting well and the people who working in the collecting well, may submerge in the waste water during the cleaning work in the collecting well. | W1 | 2 | VHR | Increases in heavy rains | Very High |

Table 3.3 Prioritized Hazardous events with high risk

| Sanitation step | Hazardous event | Exposure group | Number of people at risk | Risk | Projection of changers in risk with climate change scenarios | Priority Given |
| --- | --- | --- | --- | --- | --- | --- |
| P4: Cleaning of treatment plant (Near the chlorination Area) | Injury to the body, possible asphyxiation, caused by entering or falling into tanks in the treatment plant. | W1 | 2 | VHR | Increases in heavy rains | Very High |
| P4: Cleaning work in stabilization tank etc. | Injury to the body, possible asphyxiation, caused by entering or falling into tanks | W1 | 3 | HR | No effect from climate change. | High |
| T1: Conveyance of Sewer through the | Ingestion after contact with excreta during the repair work of blocking | U1 | 10 | HR | No effect from climate change. | High |

| network and pumping stations. | pipe lines and damaged pipe lines. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| T1: Conveyance of Sewer through the network and pumping stations. | Ingestion after contact with Contaminated ground water with waste water due to the leaks in the pipes of collecting network. | L6 | No clear idea | HR | Increases in heavy rains | Medium |
| T2: Conveyance of FS by vacuum trucks. | Ingestion after contact with raw sewage during vacuum tanker operation | W1 | 50 | HR | Increases in heavy rains | Medium |
| P22: Disposal of liquid fraction by infiltration | Ingestion of contaminated groundwater due to infiltration from septic tanks and soak pits into shallow groundwater. | U1 | 10000 | HR | Increases in heavy rains | Low |
| T2: Conveyance of FS by vacuum trucks. | Ingestion after contact with fecal sludge caused by spillage during emptying and transport | U2 | 6 | HR | Increases in heavy rains | Low |

## DEVELOPMENT AND IMPLEMENTATION OF INCREMENTAL IMPROVEMENT PLAN

### Consideration of Options to Control Identified Risks

In this step, variety of ways to control risks, including technology upgrades, changes in management and operation, behavior change measures, and policy and regulatory measures, covering all steps of the sanitation chain were considered.

Table 4.1 Improvemment Option at T1: Cleaning of collecting well.

| Step of the sanitation service chain: T1: Cleaning of collecting well. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: If the sand bags are loosen, waste water will enter in to the collecting well and the people who working in the collecting well, may submerge in the waste water during the cleaning work in the collecting well. |  |  |  |  |  |
| Exposure group: About 2 people engaged in Cleaning Work in Collecting wells. |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effecti | Level of | Effective | Comments/discussion | Priority |
|  | venes | resource | ness |  | for |
|  | s | s | under |  | improve |
|  |  |  | climate |  | ment plan |
|  |  |  | change |  |  |
|  |  |  | scenario |  |  |
|  |  |  | s |  |  |
| Installing a mechanically | High | High | Effective | Plan to purchace the | Long Term |
| operated Gate Valves to |  |  |  | Gate valve under the |  |
| control the in flow. |  |  |  | O&M budget of Regional |  |
|  |  |  |  | Manager(Water |  |
|  |  |  |  | Reclamation) JaEla/Ekala |  |
|  |  |  |  | in 2025. |  |
| Carry out cleaning work | Medi | Low | Medium | Untill receive permanent | Immediat |
| with full harness safety kit | um |  |  | solution, even simple | e |
| to lift up the persons at |  |  |  | solution should be |  |
| emergency. Keep wokers |  |  |  | applied. |  |
| stand by for any |  |  |  |  |  |
| emergency situation. |  |  |  |  |  |

Table 4.2 Improvement Option at P4: Cleaning work in Chlorination Area, Stabilization area etc.

| Step of the sanitation service chain: P4: Cleaning work in Aeration, stabilization etc. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: Injury to the body, possible asphyxiation, caused by entering or falling into tanks |  |  |  |  |  |
| Exposure group: About 5 workers during the cleaning of treatment plant structures such as stabilization tank. |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effect venes s | Level of resour ces | Effectiveness ss under climate change scenarios | Comments/discussion | Priority for improve ment plan |
| Wearing personal Protective equipment during the operation. | High | Low | Effective | Workers should be encourage to were the PPE | Immediat e |
| Follow the standard Operational procedures in cleaning work. | High | Mediu m | Effective | SOPs should be Translated in to Sinhala language. | Medium Term |
| Improve the safety fence Where can be exposed to the risks. | High | High | Effective | In most of areas safety fence are not available | Medium Term |
| Placing safety ring buoyance in the Aeration tank, stabilization tank etc. . | High | Mediu m | Effective | Plan to purchase the Safety ring buoyance in fourth quarter of the 2024. | Medium Term |

Table 4.3 Improvemment Option at P4: Clening work in Settling Tank area.

| Step of the sanitation service chain: P4: Clening work in Settling Tank area. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: Injury to the body, possible asphyxiation, caused by entering or falling into tanks |  |  |  |  |  |
| Exposure group: About 01 worker during the cleaning in Settling tank area. |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effecti venes s | Level of resour ces | Effectivene ss under climate change scenarios | Comments/discussion | Priority for improve ment plan |
| Prepaire a safety buket and fixing it on the craper bridge. so that the person being cleaned can stand safely. | High | Mediu m | Effective | If any physical discomfort occurs also it will provides safe to the worker. | High |

| Wearing personal protective equipments during the operation. | High | Low | Effective | Workers should be encourage to were the PPE | Immediat e |
| --- | --- | --- | --- | --- | --- |

Table 4.2 Improvemment Option at T1: Conveyance of Sewer through the network and pumping

| Step of the sanitation service chain: T1: Conveyance of Sewer through the network and pumping stations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of hazardous event: Ingestion after contact with excreta during the repaire work of blocking pipe lines and damaged pipe lines. |  |  |  |  |  |
| Exposure group: About 10 workers who engaged with maintainance works. |  |  |  |  |  |
| IMPROVEMENT OPTIONS |  |  |  |  |  |
| Option | Effecti | Level | Effectiven | Comments/discussion | Priority |
|  | venes | of | ess under |  | for |
|  | s | resour | climate |  | improve |
|  |  | ces | change |  | ment plan |
|  |  |  | scenarios |  |  |
| Implementation of regular | High | Mediu | Effective | NWSDB, Preventive | Short |
| preventive mainainance |  | m |  | maintainance will reduce | Term |
| programmes covering the |  |  |  | the high risk with |  |
| network. |  |  |  | minimum cost. |  |
| Identify the highly | High | High | In rainy | NWSDB, Current sewer | Medium |
| vulnerable areas and |  |  | season | network is older than 48 | Term |
| refurbishment the |  |  | difficult to | years |  |
| network. |  |  | engaged |  |  |
| Encourage to ware | Medi | Low | Effective | NWSDB provide necessary | Short |
| personal protective | um |  |  | PPE to the workers. | Term |
| equipments during the |  |  |  |  |  |
| maintainance work. |  |  |  |  |  |

Table 4.1 Improvement Option at T2: Conveyance of FS by vacuum trucks

Step of the sanitation service chain: T2: Conveyance of FS by vacuum trucks

Description of hazardous event: Injury to the body, possible asphyxiation, caused by entering or falling into soak pits or septic tanks

Exposure group: About 20 people engaged in fecal sludge transpot to the waste water treatment plants..

IMPROVEMENT OPTIONS

| Option | Effecti venes s | Level of resour ces | Effectiven ess under climate change scenarios | Comments/discussion | Priority for improve ment plan |
| --- | --- | --- | --- | --- | --- |
| Awareness-raising campain targeting vacume truck operators to avoid the accidents near the septic tank. | Medi um | Low | Effective | Awareness programmes Should be organized and carried out in the future. | Immediat e |
| providing training on standard operating procedures among service providers | High | Mediu m | Effective | Training on SOPs should be given. | Medium Term |
| Wearing personal protective equipments during the operation. | High | Mediu m | Effective | Private service providers are not use PPE and they should be encouraged. | Immediat e |

Climate Adaptation Option for a Specific Sanitation Step

| Sanitation Technology | Most Probable Climate Change Scenario | Effect on Sanitation System | Hazardous Event | Adaptation Option |
| --- | --- | --- | --- | --- |
| Waste water | More intensive | Disturb the | Untreated waste |  |
| treatment | storm | treatment process | water/partialy |  |
|  |  | by diluting the | treated waste |  |
|  |  | waste water with | water diverted to |  |
|  |  | stome water. | the Dandugam |  |
|  |  |  | Oya and |  |
|  |  |  | contaminate the |  |
|  |  |  | river water and |  |
|  |  |  | effect to the users |  |
|  |  |  | of river down |  |

| Waste water | More intensive | It disturb the |  |  |
| --- | --- | --- | --- | --- |
| Treatment | stomes | treated sludge |  |  |
|  |  | dring process by |  |  |
|  |  | solar power. |  |  |

### Development of an Incremental Improvement Plan

The incremental improvement plan was prepairded prioroticing the control measures which did not required major funds or time for the next 3 years

| Improvement measure |  | Cost Rs. | Sour ce of fund s | Lead organiz ation | Year 1 |  |  |  |  |  |  |  |  |  |  |  | Year 2 |  |  |  | Year 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 1 0 | 1 1 | 1 2 | Q 1 | Q 2 | Q 3 | Q 4 | Q 5 | Q 6 | Q 7 | Q 8 |
| T2: Conveyance of FS by vacuum trucks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Awareness-raising campaign targeting vacume truck operators to becareful near the septic tank/ standard operating procedures |  | 15,000 | O& M | NWSDB Manag er office (Ja ela/Eka la) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wearing personal protective equipments during the operation. |  | 25,000 | Gull y bow ser own ers | Gully bowser owners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T1: Conveyance of Sewer through the network and pumping stations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 100,000 | O& M | NWSDB Raddol ugama WRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Implementation of regular preventive mainainance programmes covering the network. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Installation of gate valve in collective well |  | 1,200,0 00 | O& M | NWSDB Raddol ugama WRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify the highly vulnerable areas and refurbishment the network. |  | 500,000 | O& M | NWSDB Raddol ugama WRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Encourage to wear personal protective |  | 10,000 | O& M | NWSDB Raddol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| equipments during the maintainance work. |  |  | ugama WRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P4: Clening work in Aeration,stabilization, settling tank etc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improve the safety fence | 200,000 | O& | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| where can be exposed to |  | M | Manag |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| the risks. |  |  | er |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | office |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | (Ja |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ela/Eka |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | la) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wearing personal | 10,000 | O& | NWSDB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| protective equipments |  | M | Raddol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| during the operation. |  |  | ugama |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | WRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Implementation of the Improvement Plan

SSP implementation was challenging, and building upon the initial commitments and motivation of all stakeholders required continuous follow-up. The SSP team leader organized meetings with the SSP task forces on a regular basis to review progress and discuss challenges.

## MONITORING OF CONTROL MEASURES AND VERIFICATION OF PERFORMANCE

### Definition and Implementation of Operational Monitoring

Monitoring and verification plans were developed using the Tool 5.1. Tool 5.1 helped to decide which control measures should be monitored to ensure each operation.

Table 5.1 Operational monitoring overview

| Sanitation step | Control measures for a detailed operational monitoring plan |
| --- | --- |
| T2: Conveyance of FS by vacuum trucks | - Wearing Personal Protective Equipment |
| T1: Conveyance of sewer through the | - Installation of a penstock valve in the main manhole just before the collecting well. |

| network and pumping stations |  |
| --- | --- |
| P4: Clening work in Aeration,stabilization , settling tank etc. | - Constructing safety fences covering the areas with a possible risk of falling into the tanks - Wearing personal protective equipments during the operation. |

|  | Who monitors it? | Engineering Assistant who approve discharge | When is it taken? | Immediately |
| --- | --- | --- | --- | --- |
|  | When is it monitored? | Every time they discharge to the WRC | Who needs to be informed of the action? | Regional Manager (Ja Ela/ Ekala) |

| - Operational monitoring plan for: Wearing personal protective equipments during the operation. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
| Vacuum truck operators who discharge fecal sludge to Raddolugama water reclamation center | What is monitored? | Whether operators are wearing PPE | What action is to be taken? | Imposing penalty charges |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | At the fecal sludge unloading point of Raddolugama WRC | Who takes the action? | Engineering Assistant who approve for discharge |

Table 5.2 Operational monitoring plan for T2: Conveyance of FS by vacuum truc

Table 5.3 Operational monitoring plan for T1: Conveyance of Sewer through the network and pumping stations

| - Operational monitoring plan for: Instalation of penstock valve in main manhole just before to the collecting well. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
| Collecting well inflow should be stopped during the cleaning works of final well | What is monitored? | Stopping of inflow to the final well | What action is to be taken? | - Close the valve correctly - Taking out the workers from the final well |
|  | How is it monitored? | Visual observation |  |  |
|  | Where is it monitored? | Final well of Raddolugama WRC | Who takes the action? | OIC – Raddolugama/ EA (Civil) |
|  | Who monitors it? | EA (Civil) | When is it taken? | immidiately |
|  | When is it monitored? | During the cleaning works of final well | Who needs to be informed of the action? | EA (Mechanical) & Regional Manager (Ja Ela) |

Table 5.4 Operational monitoring plan for T1: Conveyance of Sewer through the network and pumping stations

| - Operational monitoring plan for: Wearing personal protective equipments during the operation and maintainance work. |  |  |
| --- | --- | --- |
| Operational limits | Operational monitoring of the control measure | Corrective action when the operational limit is exceeded |

| All the workers who engage in Operation and maintainance work should wear the PPE. | What is monitored? | Whether workers are wearing PPE | What action is to be taken? | Apply penalties to workers who do not wear the PPE. |
| --- | --- | --- | --- | --- |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | Sewer network at the Raddolugama water reclamation center | Who takes the action? | Engineering Assistant who supervised the operation and maintainance work. |
|  | Who monitors it? | Engineering Assistant who supervised the operation and maintainance work. | When is it taken? | After warning 3 times |
|  | When is it monitored? | Every time they are engaged with the O&M works. | Who needs to be informed of the action? | Regional Manager (Ja Ela/ Ekala) |

| - Operational monitoring plan for: Improve the safety fence where can be exposed to the risks. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
| All the employees and visitors should avoid from falling in to the tanks | What is monitored? | Covering all the unsafe areas with safety fence | What action is to be taken? | Fixing safety fence |
|  | How is it monitored? | By visual observation |  |  |
|  | Where is it monitored? | At the Raddolugama water reclamation center | Who takes the action? | OIC - Raddolugama |

Table 5.5 Operational monitoring plan for P4: Clening work in Aeration,stabilization, settling tank etc.

|  | Who monitors it? | Engineering Assistant | When is it taken? | As soon as possible. (according to the fund availability) |
| --- | --- | --- | --- | --- |
|  | When is it monitored? | Daily | Who needs to be informed of the action? | Regional Manager (Ja Ela/ Ekala) |

Table 5.6 Operational monitoring plan for P4: Clening work in Aeration,stabilization, settling tank etc

| - Operational monitoring plan for: Wearing personal protective equipments during the Cleaning and operation work. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operational limits | Operational monitoring of the control measure |  | Corrective action when the operational limit is exceeded |  |
| All the workers who engage in Operation and maintenance work should avoid accidents | What is monitored? | Operators are wearing PPE | What action is to be taken? | Apply penalties to workers how do not wear the PPE. |
|  | How is it monitored? | By visul observation |  |  |
|  | Where is it monitored? | At the Raddolugama water reclamation center and sewer network | Who takes the action? | OIC - Raddolugama |
|  | Who monitors it? | Engineering Assistant who supervises the operation and maintenance work. | When is it taken? | After warning 3 times |
|  | When is it monitored? | Every time the workers are engaged with the operation and maintenance work. | Who needs to be informed of the action? | Regional Manager (Ja Ela/ Ekala) |

  - Verification of System Performance

| Sanitation Step | Verification |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | What | Limit | When | Who | Method |
| T1: Conveyance of Sewer through the network and pumping stations. | the time taken to attend a leak | < 48 hours | Monthly | Regional Manager (Ekala/Ja Ela) | Monthly progress report & monthly progress meeting. |
| T1: Conveyance of Sewer through the network and pumping stations. | % of No. of complaints attended within 48 hours. | > 98% | Monthly | Regional Manager (Ekala/Ja Ela) | Monthly report & monthly progress meeting |
| T2: Conveyance of FS by vacuum trucks | pH Value, Colour, Odor and Oil | Ph – 6-8 Colour Oder - acceptable level according to the visual observation Oil – Not Allow | Before discharge | Pump Operator | Field testing & Visual observations |
| P4: Treatment Processe | BOD, COD,TSS, Colour Level in the influent, and the effluent of the treatment plant. | COD <250mg/l BOD < 30mg/l TSS < 50 mg/l | Once a week | Lab Assistant | By sample testing |
| P4: Treatment Processe | BOD, COD,TSS, Colour Level in the influent, and the effluent of the treatment plant. | COD <250mg/l BOD < 30mg/l TSS < 50 mg/l | Once a week | Lab Assistant | By sample testing |

  - Auditing the System

It was decided to review and audit the sanition saftety planning of Raddolugama WRC internally once in an year and externally once in an year.

## DEVELOPMENT OF SUPPORTING PROGRAMMES AND REVIEW PLANS

### Identification and Implementation of Supporting Programmes

### Programme to enhance the protection during cleaning of Main collecting well. The ssp was target to improve the safety of workers during the cleaning of main well at Raddolugama wastewater treatment plant. There are is no perfect closing system for stop the entering of waste water to the main collecting well during the preventive maintenance work. Therefore, it was decided to install a gate at manhole prior to the main collecting well. Currently, the preparation of suppliers list is being progressed.

### Programme to enhance the safety on the treatment Plant

### At present, there is no perfect security fence on the treatment plant. Hence, it was decided to install a perfect fence on the plant in order to secure the safety of worker and visitors during walking on the treatment plant floor. Currently, procurement of material for the above is being processed.

- Programme to enhance the coverage with vacuum pump service providers.

The SSP was to target the Raddolugama wastewater treatment plant and the vacuum track service providers who discharge the fecal sludge to the Raddolugama wastewater treatment plant. By conducting the programmes connected with the Environment police, it will be able to increase the number of the private vacuum truck service providers who get the service from Raddolugama wastewater treatment plant. It will ensure the safe sewage disposal in the areas around the Raddolugama by providing a safe end use option.

### Programme to reduce the storm water collecting to the sewer conveyance system.

During the rainy period, storm water is diverted to the sewer collecting network in the Raddolugama National Housing Scheme. Therefore wastewater is diluted and the biological treatment process is disturbed. The storm water network is maintained by the Katana Pradeshiya sabha. Therefore supportive programme to be conducted with Katana Pradeshiya Sabha to implement a suitable solution such as refurbishment the storm water network.

### Programme to ensure the quality of the treatment process and the effluent by improving the onsite testing facility in the Raddolugama net work area.

The quality of the treatment process is verified by testing the influent and effluent at once a week by the Ekala reclamation center laboratory. By increasing the onsite testing facility in the Raddolugama wastewater treatment plant itself, it will be helpful to ensure the quality of the treatment process. It will ensure the safe sewage disposal in the areas around the Raddolugama by providing a safe end use option.

### Map the location of the leak point/overflow, on the network map.

Raddolugama wastewater treatment plant and the collecting network is older than 40 years. Therefore most of the pipes have decayed. By mapping the leak points/overflow points in a network map by using the GPS coordinates, it will be helpful to identify the most vulnerable areas in the collecting network and it will be helpful to identify priority areas, which require the Refurbishment in the network.

### Periodic Reviewing and Updating the SSP Outputs

It was decided to revise the SSP in 12 months according to the improvements of the proposed SSP and the new risk that arose.

Annexes

Planning Template	……

Introduction	3

The requirement for an emergency response plan	3

Section 1. System Information	4

Section 2. Chain of Command – Lines of Authority	5

Section 3. Events that Cause Emergencies	6

Section 4. Emergency Notification	7

Section 5. Effective Communication	71

Section 6. Response Actions for Specific Events	72

Section 7. Alternative Water Sources	14

Section 8. Plan Approval	15

# Introduction

NWSDB, Raddolugama water reclamation center has formulated an Emergency Response Unit. This Unit is planned to response all types of Emergencies in the Raddolugama WRC. The aim of this plan is to treat the waste water due to an emergency with in the 24 hours and to cater the minimum lifeline and waste water treat during emergency until supply is restored or normalcy is attained.

This plan involves mainly Disaster Management Center and other stakeholders of Raddolugama wrc such as Pradeshiya Sabha, Schools, Department of Health, Divisional Secretariat, Irrigation Depatment, CEB, Police and other local authorities.

Emergency Response Plan in Raddolugama wrc can be categories as follows,

- Response Plan

  - Major distribution main failure

  - Power outage in water supply system

  - Intake reservoir/Service tanks

  - Water quality issues

  - Protocol for managing threats to the waste water treat

  - Protocol for response to unauthorized entry to a water supply asset

  - Treat to the system infrastructure/Landslides

- Contingency Plans

  - Intermittent supply

  - Water main shutdown procedures

- Risk Management

- Customer complains

- Financial and Corporate Services

- Strategy and Communications

- People and Culture

- Infrastructure Services

# Section 01. System Information

System information

| Scheme Name and Address | Raddolugama WRC |  |  |  |
| --- | --- | --- | --- | --- |
| Basic Description and Location of System Facilities | Refer SSP |  |  |  |
| Location/Town | Raddolugama |  |  |  |
| Population Served and Service Connections from Division | 10000 people | 2237 connections |  |  |
| System Owner | National Water Supply and Drainage Board |  |  |  |
|  |  |  | 0114831468 | Office Phone |
| Name, Title, and Phone Number of Person Responsible for Maintaining and Implementing the Emergency Plan | Manager, C.J Ransingha |  | 0777633321 | Mobile |

# Section 02. Chain of Command – Lines of Authority

The first response step in any emergency is to inform the person at the top of this list, who is responsible for managing the emergency and making key decisions.

Chain of command – lines of authority

| Name and Title | Responsibilities During an Emergency | Contact Numbers |
| --- | --- | --- |
| OIC-Raddolugama (WRC) | Operation and Maintenance Plant | 0714436521 |
| EA (civil)-Raddoluga ma (WRC) | Operation and Maintenance Network system | 0714898653 |
| O&M Engineer or Area Engineer | Coordination, resource allocation, Operation and Maintenance | 0764583668 |
| Electrical EA | Coordination, resource allocation, Operation and Maintenance | 0773900557 |
| Mechanical EA | Coordination, resource allocation, Operation and Maintenance | 0716407880 |
| Chemist | Water quality analysis | 0771453791 |
| Manager | Coordination, resource allocation | 0777633321 |
| DGM | Authorization | 0772918534 |

# Section 03. Events that Cause Emergencies

| Type of Event | Probability or Risk (High-Med-Low) | Comments |
| --- | --- | --- |
| Power failure | Low to high |  |
| Chlorine leakage | Low to high |  |
| Mechanical equipment failure | Low to high |  |
| Electronic equipment failure | Low to high |  |
| Earth Slip / drought | Low to high |  |
| Terrorist attack | Low to high |  |
| Biological attack | Low to high |  |

The events listed below may cause water system emergencies. They are arranged from highest to lowest probable risk. The list shall be updated as the emergencies occurred in the same WSS and the other WSSs operated in Sri Lanka Events that cause emergencies

# Section 04. Emergency Notification

Notification call-up lists - Use these lists to notify first responders of an emergency.

| Organization or Department | Name & Position | Telephone | Email |
| --- | --- | --- | --- |
| Local Law Enforcement | OIC-Raddolugama Police station | 0112292857 |  |
| Fire Department | Fire Brigade | 0112422222 |  |
| Emergency Medical Services | Ambulance service | 1990 |  |
| Chlorine leakage | Paranthen chemicals | 0112437556 |  |
| Neighboring Water Reclamation Center (not connected) | OIC- Ekala | 0707517860 |  |
| Electricity Board | C E B Ja ela | 0114344736 |  |
| Irrigation Department | Enginner | 0714443662 |  |
| Hospitals or Clinic(s) | MOH | 0714511732 |  |

| Chemist | T.A.D A. I Jaythilaka | 0771453791 |  |
| --- | --- | --- | --- |
| Mechanical/Electrical Section | Mech EA Ele Ea | 0716407880 0773900557 |  |
| RDA / PRDA / PS | RDA- EA | 0714472580 |  |
|  | PRDA-General | 0312233621 |  |
|  | PS-TO | 0773753087 |  |
| JCB | Operator | 0716169247 |  |
| Supply Section | Supply Officer | 0714482919 |  |

| Organization or Department | Name & Position | Telephone | Email |
| --- | --- | --- | --- |
| Newspaper - Local | Media notifications done by the Secretary of the Ministry. Only inform to DGM for further actions. |  |  |
| Newspaper – Regional/State/Tribal |  |  |  |
| Radio |  |  |  |
| Radio |  |  |  |
| TV Station |  |  |  |

### Media Notification

Notification procedures

Notify water system customers of potential water shortage

| Who is Responsible: | DGM(GCS)/Manager(Ekala) |
| --- | --- |
| Procedures: | Public Announcement, Media message |

Contact service and repair team

| Who is Responsible: | OIC Raddolugama WRC |
| --- | --- |

| Procedures: | Assigning working team with resources |
| --- | --- |

Contact neighboring waste water systems, if necessary

| Who is Responsible: | O & M Engineer |
| --- | --- |
| Procedures: | Telephone call/messages |

Procedures for issuing a health advisory

| Who is Responsible: | Manager, Chemist, Sociologist, |
| --- | --- |
| Procedures: | Contact MOH office, PHI |

# Section 05. Effective Communication

Communication with customers, the news media, and the general public is a critical part of emergency response.

### Designated public spokesperson

Designate a spokesperson (and alternate) and contact your local primacy agency for delivering messages to the news media and the public.

### Designate a spokesperson and alternates

| Spokesperson | Alternate |
| --- | --- |
| General Manager | DGM-GCS |

Health advisories

During events when water quality and human health are in question, it may be necessary to issue a health advisory that gives advice or recommendations to water system customers on how to protect their health when drinking water is considered unsafe. These advisories are issued when the health risks to the consumers are sufficient, in the estimation of the water system, state or tribal, or local health officials, to warrant such advice.

# Section 06. Response Actions for Specific Events

In any event, there are a series of general steps to take:

- Analyze the type and severity of the emergency

- Take immediate actions to save lives

- Take action to reduce injuries and system damage

- Make repairs based on priority demand

- Return the system to normal operation.

The following tables identify the assessment, set forth immediate response actions, define what notifications need to be made, and describe important follow-up actions.

  - Power outage

| 7 Assessment | Finding out the details for power outage such as duration of outage, affected areas etc from the Electricity board |
| --- | --- |
| Immediate Actions | Telephone Call |
| Notifications | OIC Raddolugama (WRC), Pump operators |
| Follow-up Actions | Power supply from Generator |

  - Network line break

| 8 Assessment | Affected areas and consumers, priority customers, |
| --- | --- |
| Immediate Actions | Assigned team for rectification |
| Notifications | OIC Raddolugama (WRC), O&M Engineer(Ja Ela/Ekala) |
| Follow-up Actions | Immediate rectification / Corrective actions / study the situation / case studies |

  - Chlorine system failure

| 9 Assessment | Chlorine neutralization / Health assessment of infected workers |
| --- | --- |
| Immediate Actions | Contact health services / shut down of the plant / first – aids / rectification of the system / restore the security / commissioning of the plant |
| Notifications | Fire / police / health services / top management |
| Follow-up Actions | Case study / corrective actions |

  - Treatment equipment

| 10 Assessment | Quality of the treated water / PH, BOD,COD,TSS, |
| --- | --- |
| Immediate Actions | Rectify the problem and Recommissioning of plant after rectification |
| Notifications | Consumers / top management Media if situation lasts long |
| Follow-up Actions | Case study / corrective actions |

  - Source pump failure

| 11 Assessment | Quantity of waste water pumping from the plant |
| --- | --- |
| Immediate Actions | Inform Eng(O&M) and M&E group / operation of stand-by pumps |
| Notifications | Eng(O&M) / M&E over the phone / top management |
| Follow-up Actions | Case study / corrective measures / spares arrangement |

  - Microbial (coliform, E. coli) contamination

| 12 Assessment | Bacteriological reports of the treated water |
| --- | --- |
| Immediate Actions | Shut down of the plant / adjust disinfection system / disinfection of stored water / rectification and recommissioning |
| Notifications | OIC (Raddolugama / Eng (O&M) / Chemist |

| Follow-up Actions | Case study / corrective measures |
| --- | --- |

  - Chemical contamination

| 13 Assessment | Chemical composition report of treated water |
| --- | --- |
| Immediate Actions | Shut down the operation / identification of contaminant source / rectification and recommissioning |
| Notifications | OIC (Raddolugama / Eng(O&M)/Manager(WR-J/E)/ Chemist |
| Follow-up Actions | Case study / corrective measures / evaluating |

  - Vandalism or terrorist attack

| 14 Assessment | Chemical/ Bacteriological composition report of treated water and plant equipment report |
| --- | --- |
| Immediate Actions | Shut down the plant / contact security services |
| Notifications | Workers / public / top management / security services (police and forces) |
| Follow-up Actions | Case study / evaluating / mental condition survey |

I.	Earth Slip

n/a

15  Assessment

n/a

15  Assessment

| Immediate Actions | Shut down of the plant / Protection of M&E equipment / protection of the plant |
| --- | --- |
| Notifications | Disaster Management Centre / top management / fire |
| Follow-up Actions | Rectification and recommissionning |

- Hazardous materials spill in vicinity of sources or system lines

| 16 Assessment | Chemical / biological and other reports |
| --- | --- |
| Immediate Actions | Rectification and recommissioning |
| Notifications | Top management / consumers |
| Follow-up Actions | Analyzing the situation |

- Electronic equipment failure

| 17 Assessment | Water quality and quantity / responsiveness of the particular equipment |
| --- | --- |
| Immediate Actions | Inform to ME/EE/EA(Mech) |
| Notifications | Eng(O&M)/ R.Manager(WR-Jaela Ekala) |
| Follow-up Actions | R.Manager(WR-Jaela Ekala)Case study / corrective measures |

- Fire

| 18 Assessment | Water quality report of treated water and plant equipment report |
| --- | --- |
| Immediate Actions | Inform to relevant party(OIC/RM) , used fire extinguisher(training person) and go to “assemble point “ |
| Notifications | OIC/Eng(O&M) / R.Manager(WR-Jaela Ekala) |
| Follow-up Actions | Analyze situation |

R.Manager(WR-Jaela Ekala)

- Other

| 19 Assessment | As per the situation |
| --- | --- |
| Immediate Actions | Inform to relevant party / Eng(O&M) / R.Manager(WR-Jaela Ekala) |
| Notifications | Eng(O&M) / R.Manager(WR-Jaela Ekala) |
| Follow-up Actions | Analyze situation |

Section 7. Alternative Water Sources

Alternate source(s) of water

| Alternative Sources | Names | Phone | Availability | Is the Water Safe for Drinking? |
| --- | --- | --- | --- | --- |
| Tanker trucks in | NWSDB Jae | Eng (O&M) | Available | Safe |
| the area available | la/Ekala (WR), |  |  |  |
| to deliver bulk |  |  |  |  |
| water for potable |  |  |  |  |
| use |  |  |  |  |
| Tanker trucks in | Disaster Mgt Cen. AG Offices (PS) Irrigation Dept. | 0112136136 | Available | Safe |
| the area available |  | 031 2222836 |  |  |
| to deliver bulk |  | 0773753087 |  |  |
| water for potable |  | 0714443662 |  |  |
| use (with other |  |  |  |  |
| organizations) |  |  |  |  |
| Tanker trucks in | PRDA | 0312233621 | Available | Unsafe |
| the area available | Irrigation Dept. |  |  |  |
| to deliver bulk water for |  | 0714443662 |  |  |
| non-potable use |  |  |  |  |

Section 8. Plan Approval

This plan is officially in effect when reviewed, approved, and signed by the following people:

| Name/Title | Signature | Date |
| --- | --- | --- |
| G.D.M Nevil (DGM) |  |  |

| Eng.C.J Ranasingha (R.Manager(WR- Ekala/Jaela) |  |  |
| --- | --- | --- |
| Eng.D.M.W.JB. Dissanayaka.(O &M Engineer) |  |  |
| A.W.A.D.J.R.Abesekara(OIC Raddolugama WRC) |  |  |